3GPP TSG-SA WG1 Meeting #102 															S1-23xxxx

22 – 26 May, 2023, Berlin, Germany		                                   (revision of S1-231079)

Source:	vivo, Huawei

pCR Title:	Power consumption descriptions for use case 24 and 25

Draft Spec:	3GPP TR 22.840 V1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Yanchao Kang, kangyanchao@vivo.com，

Huan Li,  huan.li@vivo.com

Abstract: This document provides an update of Use case 5.24 and 5.25.

Discussion

The scope of TR22.840 with the following descriptions:

An ambient IoT device has low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g., NB-IoT/eMTC devices). Ambient IoT devices can be maintenance free and can have long life span (e.g., more than 10 years).

The power consumption is not added in neither the KPI table nor the definition of Ambient IoT device. Since it is an important aspect, it is proposed to add clarification in the use case description for better clarifications.

RAN#99 (RP-230808) achieved consensus of Device Power consumption for three types of Ambient IoT devices as follows:

For Device A, the power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW],

For Device B, the target during transmitting/receiving is such that:

-	Device A power consumption ≪ Device B power consumption < Device C power consumption; or

-	Device A power consumption ≤ Device B power consumption < Device C power consumption.

The device power consumption for Device C is ≤ 1 mW to ≤ 10 mW.

For use case 5.24 and 5.25, it is proposed to use power consumption of Device C as above.

****************Start of Change *************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]                      EPC Tag Data standards version 1.1 Rev.1.24

[16] 	Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

[17] 	Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

[18] 	Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

[18] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[20] 	Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

[21] 	Mishu M K, Rokonuzzaman M, Pasupuleti J, et al. An adaptive TE-PV hybrid energy harvesting system for self-powered iot sensor applications[J]. Sensors, 2021, 21(8): 2604.

[22]	Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

[23] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[24] 	H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

[25] 	Hoang T, Ferin G, Bantignies C, et al. Aging assessment of piezoelectric energy harvester using electrical loads[C]//Journal of Physics: Conference Series. IOP Publishing, 2019, 1407(1): 012078.

[26] 	https://www.usda.gov/foodwaste/faqs

[27] 	https://www.fda.gov/food/new-era-smarter-food-safety#:~:text=Welcome%20to%20the%20New%20Era,reducing%20the%20number%20of%20illnesses

[28] 	Alam AU, Rathi P, Beshai H, Sarabha GK, Deen MJ. Fruit Quality Monitoring with Smart Packaging. Sensors (Basel). 2021 Feb 22;21(4):1509. doi: 10.3390/s21041509. PMID: 33671571; PMCID: PMC7926787.

[29] 	Taoukis, Petros & Nychas, George-John. (2000). Use of time-temperature integrators and predictive modeling for shelf life control of chilled fish under dynamic storage conditions. International journal of food microbiology. 53. 21-31. 10.1016/S0168-1605(99)00142-7.

[30] 	Mishu, M.K., Rokonuzzaman, M., Pasupuleti, J., Shakeri, M., Rahman, K.S., Hamid, F.A., Tiong, S.K. and Amin, N., 2020. Prospective efficient ambient energy harvesting sources for iot-equipped sensor applications. Electronics, 9(9), p.1345.

[31] 	Bagchi, S., Abdelzaher, T.F., Govindan, R., Shenoy, P., Atrey, A., Ghosh, P. and Xu, R., 2020. New frontiers in IoT: Networking, systems, reliability, and security challenges. IEEE Internet of Things Journal, 7(12), pp.11330-11346.

[32] 	O’Neill, V. and Soh, B., 2022. Improving Fault Tolerance and Reliability of Heterogeneous Multi-Agent IoT Systems Using Intelligence Transfer. Electronics, 11(17), p.2724.

[33] 	Katanbaf, M., Jain, V. and Smith, J.R., 2020. Relacks: Reliable backscatter communication in indoor environments. Proceedings of the ACM on Interactive, Mobile, Wearable and Ubiquitous Technologies, 4(2), pp.1-24.

[34] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[35] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[36] 	https://www.mokosmart.com/use-iot-fire-detector-sensor/

[37]  	https://www.5gamericas.org/wpcontent/uploads/2019/07/5G_Americas_URLLLC_White_Paper_ Final__updateJW.pdf

[38] 	TS 22.261 https://www.3gpp.org/ftp/Specs/archive/22_series/22.261/22261-j00.zip

[39] 	 	    TR 22.866: "enhanced Relays for Energy Efficiency and Extensive Coverage".

[40] 		     		Global RFID Sensor Market - Global Industry Analysis, Size, Share, Growth, Trends

and Forecast 2021-2031, Transparency Market Research, June 2021

[41]		     		Sensors: Technologies and Global Markets (

https://www.bccresearch.com/market-

research/instrumentation-and-sensors/sensors-technologies-markets-report.html),BCC

Research

[42]		     		Berckmans, D. Precision livestock farming technologies for welfare management in

intensive livestock systems. Rev. Sci. Tech. Off. Int. Epiz. 2014, 33, 189–196.

[43]	Precision Agriculture for Crop and Livestock Farming - Brief Review, Animals 2021

[44]                	Journal of Dairy Science - Comparison of 2 systems of pasture allocation on milking intervals and total daily milk yield of dairy cows in a pasture-based automatic milking system, American Dairy Science Association, 2013

[45]	Grazing and automation, Proceedings 4th Meeting European Grassland Federation WG “Grazing”, 2015

[46] 	Grazing Methods, UK Agriculture Food and Environment, https://grazer.ca.uky.edu/content/grazing-methods-which-one-you#:~:text=Strip%20Grazing%20%E2%80%93%20This%20technique%20involves,a%20new%20allocation%20of%20forage.

[47]                	Nora O’Donovan, https://www.independent.ie/business/farming/dairy/dairy-    advice/putting-a-size-on-paddocks-30447793.html

[48]	A. van den Pol-van Dasselaar, P. W. Blokland, et al, Beweidbare oppervlakte en weidegang op melkveebedrijven in Nederland (Pasture area and grazing on dairy farms in the Netherlands), Livestock Research Wageningen, 2015

[49]	Laura Paine, A Summary of Dairy Grazing Practices in Wisconsin

[50]	R. J. Rendell, Designing Paddocks on an Irrigated Dairy Farm, Victoria, Australia

[51]	The EU pig meat sector, European Parliament

[52]	http://www.bestgenetics.com.cn/EN/PigFarm1

[53]		    			Lisette. E. van der Zande, et al, Individual detection and tracking of group housed pigs

in their home pen using computer vision, Front. Anim. Sci. 2:669312. doi:

10.3389/fanim.2021.669312, April 2021

[54]	Carol Souza de Silva, Wageningen University, the impact of Pen Size and Stocking Density on behaviour and welfare of growing pigs

[55]	Xue Li, Xia Xiong, et al, Effects of stocking density on growth performance, blood parameters and immunity of growing pigs, December, 2020

[56] 	https://www.concreteconstruction.net/projects/infrastructure/so-many-manholes-so-little-time_o#:~:text=U.S.%20EPA%20estimates%20the%20number,from%20100%20to%20500%20feet.

[57]	http://news.cjn.cn/24hour/wh24/201101/t1276749.htm

[58] 	https://www.dibblecorp.com/insights/manhole-infrastructure-assessments/

[59] 	https://www.francebleu.fr/infos/faits-divers-justice/un-homme-retrouve-mort-dans-une-bouche-d-egout-a-saint-nazaire-1666456765

[60]	https://guancha.gmw.cn/2022-05/16/content_35739227.htm

[61]	Intelligent manhole cover national standard, GB/T 41401, 2022, the Ministry of Housing and Urban-Rural Development of the People's Republic of China

[62]	MEMS gauge pressure sensor NSPGD1 Novosense, liquid level measurement, https://www.novosns.com/Public/Uploads/uploadfile/files/20220726/-897.pdf

[63]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[64]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[65] 	Sung-Pil CHANG, Jinkyo CHOO, Values of Bridge in the Formation of Cities

[66]	https://www.thepaper.cn/newsDetail_forward_4643273

[67] 	https://news.sina.cn/gn/2021-12-19/detail-ikyamrmy9954352.d.html

[68] 	https://www.nytimes.com/2020/12/22/world/europe/genoa-bridge-collapse.html

[69]	https://www.theguardian.com/world/2022/oct/31/india-bridge-collapse-death-toll-rises-rescue-efforts-continue

[70]	Role of the Local Government in Monitoring and Maintenance Bridges, IAPA Proceedings Conference, 2019

[71]	https://elastisense.com/structural-health-monitoring-bridges

[72]	GB/T 39559.2-2020 National Specification for operational monitoring of urban rail transit facilities – Part 2: Bridge

[73]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[74]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[75]	Big data analysis for bridges health monitoring, Tongji University, 2017

[76]	Beijing Dongsha bridge safe operation monitoring, http://www.ccea.zju.edu.cn/_upload/article/files/64/e6/a824c7d04932af200750fbd9e690/fb0a5e9c-a1e7-47b8-bdb4-82b584d1c52f.pdf

[77]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[78]	https://www.rfidjournal.com/esls-are-becoming-more-affordable-for-retailers

[79]	https://www.newswire.ca/news-releases/sobeys-pilots-smart-cart-the-first-intelligent-grocery-shopping-cart-803013190.html

[80]                     https://en.wikipedia.org/wiki/Electronic_paper

[81]	F. Palacio, et al. (2006). "Active RFID tag with sensing capabilities and low power consumption".

[82]	T. O. John, H. C. Ukwuoma, S. Danjuma and M. Ibrahim, "Energy consumption in wireless sensor network", Energy, vol. 7, no. 8, 2016.

[83]	Tuna, G., and V. C. Gungor. "Energy harvesting and battery technologies for powering wireless sensor networks." Industrial Wireless Sensor Networks. Woodhead Publishing, 2016. 25-38.

[84]	M. Zeinali and J. Thompson, "Impact of Compression and Small Cell Deployment on NB-IoT Devices Coverage and Energy Consumption with a Realistic Simulation Model", Sensors. no. 19, 2021.

[xx]	K. Tang et al., "A 75.3 pJ/b Ultra-Low Power MEMS-Based FSK Transmitter in ISM-915 MHz Band for Pico-IoT Applications," 2021 IEEE International Symposium on Circuits and Systems (ISCAS), 2021, pp. 1-4, doi: 10.1109/ISCAS51556.2021.9401715.

[yy]	J. Bae and H. Yoo, "A low energy injection-locked FSK transceiver with frequency-to-amplitude conversion for body sensor applications," 2010 Symposium on VLSI Circuits, 2010, pp. 133-134, doi: 10.1109/VLSIC.2010.5560325.

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

*******************Next Change*****************

5.24.1	Description

Manholes date back to the mid-19th century. Sanitary sewer manholes are primarily used for joining or/and changing the direction of the sewer. As part of the underground infrastructure, manholes provide access for maintenance and for potentially installing additional sewer lines. Worldwide, there have been increasingly a large number of manholes in villages and cities (e.g. parks, sidewalks, parking lots, streets, etc.) as modern civilization grows. For example, U.S. EPA estimates the number of sewer manholes nationwide to be 12 million. Most coincide with the typical lengths of city and suburban blocks, about 100 to 500 feet apart [56]. The total count of utility manholes (adding onto sewer manholes) in the United States is approximately 20 million. A typical large Chinese city has around 1 million manholes. For example, more than 123 million manholes are in use in Wuhan alone [57], which the local government has plans to digitally identify and register each of them. Usually local authorities such as municipalities monitor and inspect assets of city infrastructure. At times manned manhole inspection is carried out with cameras underground to verify degrees of deterioration and provide rehabilitation recommendations. In that process, surface features such as manhole cover and pavement can be reported if restoration is needed [58].

However, manhole covers alone are rarely monitored frequently enough. Often gone unnoticed are they doing their job to keep traffic (e.g. motorist, cyclists) and pedestrians safe, until one falls. In and outside busy cities, falling into manholes causes potential danger of severe injuries or death is lurking, not only where poorly lit.

A recent incident in October 2022 downtown Saint-Nazaire (west coast France) concerns a 39-year-old man found dead early Saturday morning, drowned head down leaving lower legs hanging upright outside the manhole [59]. There are also many stories where children fall into manholes, parents desperately trying to pull them outside but often deadly tragedy prevails [60]. As a matter of fact, accidental fall due to damaged or missing manhole covers has become a silent killer around us. But this is preventable thanks to the use of Ambient IoT.

In Q1 2022, China published intelligent manhole cover national standard GB/T 41401 [61] for municipalities to digitally manage this important asset. The standard requires the manholes to be identifiable by the asset management application, displacement of manhole cover (due to e.g. accidental damage or theft) can be detected by tilt sensors. Additionally, underground water level sensors, vibration sensors (e.g. to detect shock events), and temperature sensors could be deployed.

Figure 5.24.1-1: Manhole where the fatal accident took place [59], photo © Maxppp- Sébastien Jarry

For the manhole cover use case, a large number of sensors (a type of Ambient IoT devices) need to be efficiently connected, particularly because they have very limited power source. The communication power consumption of such Ambient IoT devices are expected to be less than 1 mW [xx] [yy]. As manhole covers are stationary and deployed in outdoor public areas. And because this use case concerns road safety, the communication service availability with sufficient 5G network coverage is important.  The data acquisition process of these sensor data is not latency critical. The sensor data (tilt, underground water level, shock) is needed once every 15 minutes. The acquisition of detected abnormality is required within 30 seconds [61]. All these sensors should be durable and maintenance free, as sending technicians to solely replace sensors at each manhole location would be an extra process adding to municipalities OPEX and environmental impact.

In this use case, the municipality M responds to a recent tragic incident similar to [59] by implementing the smart manhole cover management programme “La Bouche d'égout sans Souci”. M has service level agreement with La-Tel-Verte to provide 5G network coverage and enable communication of Ambient IoT devices with the 5G network. La-Tel-Verte per service level agreement provides energy-efficient communication and management services to the municipality M:

- interfacing with municipality M’s manhole cover management platform where application “La Bouche d'égout sans Souci” runs;

- providing energy efficient device management for the Ambient IoT devices based on the instructions from the manhole cover management platform;

- providing energy efficient operation (e.g. inventory, read) the Ambient IoT devices based on the instructions from the manhole cover management platform;

- providing energy efficient security mechanisms for the communication between Ambient IoT devices and the network.

*******************Next Change*****************

5.25.1	Description

Bridges as public infrastructure characterize city formation. To facilitate transportation, more bridges are built as cities form and evolve. Apart from obvious roles bridges play in transortation of people and goods, its importance also lies in its being the social infrastructure and cutural assets [65]. As the number of bridges grow, longer and bigger bridges are put in use. Incidents of bridge collapsing usually lead to catastrophy. In the course of city expansion and population growth, numerous deaths due to bridge collapse have been recorded, such as recent events reported in [66], [67] and [68]. The most heart-breaking tragic to date is the Indian pedestrian bridge collapse in Guikarat in the midst of Diwali religious celebration, where death toll rises above 130 [69]. As awareness continues to rise, various actors including local governements are starting to take on a more active role in monitoring and maintenance of those vital infrastructure [70].

Ambient IoT can be used for smart bridge health monitoring to contribute to prevention of disasters. As bridges are parts of outdoor public infrastructure and the use case is about safety, the communication service availability with sufficient 5G network coverage are important. Additionally, as more health data of a bridge is collected, the corresponding actors (e.g. local governmement) have deeper knowledge in its health state to better control the maintenance work and eventually control the cost [71]. In 2022, China published national specification GB/T 39339.2 that hightlights safety monitoring of bridges as part of transit facilities in cities [72].

In this use case, local government of city Philario responds to a recent tragic incident by implementing the smart bridge health monitoring programme “SAFE”. Philario has service level agreement with O-Tel to provide 5G network coverage and enable communication of Ambient IoT devices with the 5G network O-Tel per service level agreement provides energy-efficient communication and management services to the local government of Philario:

- interfacing with local government Philario’s smart bridge health monitoring platform;

- providing energy efficient device management for the Ambient IoT devices based on the instructions from Philario’s smart bridge health monitoring platform;

- providing energy efficient operation (e.g. inventory, read) the Ambient IoT devices based on the instructions from Philario’s smart bridge health monitoring platform;

- providing energy efficient security mechanisms for the communication between Ambient IoT devices and the network.

The communication power consumption of such Ambient IoT devices are expected to be less than 1 mW [xx] [yy].

*******************End of Change*****************