3GPP TSG SA WG 1 Meeting #102 	S1-231417

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231296)

Source:	Huawei, Vodafone?

pCR Title:	Pseudo-CR on FS_Ambient IoT, update KPI Device Density to clause 5.26 Elderly Health Care

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Shuang Zhang (zhang.shuang2@huawei.com)

Abstract: This document provides a TP to TR 22.840 v.1.1.0 to update KPI Device Density to clause 5.26 Elderly Health Care. The revision includes the additional information to clarify the proposed modification.

1. Introduction

The clause 5.26 “Elderly Health Care” has been agreed and captured in TR 22.840, which introduces the use case for sending command to light a small LED (embedded in an Ambient IoT device) on a medicine case as a reminder for an elderly person to take pills. It is an interesting use case and has positive social impact. However, the Device density values as captured can be modified to be more representative.

2. Reason for Change

In KPI Table 5.26.6-1, Device density values for indoor and outdoor scenarios are captured as “<20 devices/100m2” and “<200 devices/100m2” respectively.

Indoors, according to available national regulation related to min standard living space for EU countries [y1], the living space of a studio apartment in Italy is 28 m2 for a single and 38 for two occupants (36m2 in portugal for single occupancy accommodation and 20m2 in Spain). Assuming in EU care home has highest density of elderly dwellers and the studio apartment complexes are used, then data from [y1] can be used to derive indoor device density. Assuming every dweller has one medicine box in a studio complex, there are probably 5 devices per 100m2 surface area. However, in care homes the elderly residents meet in shared space for activities, the original indoor device density is considered reasonable.

Outdoors, however, as captured in the Service Flow, the use case is about a number of elderly friends meeting up to play tennis. Though nature or city outskirts might be chosen (where population is more scarce), tennis courts are most likely the place where the highest concentration of elderly sporters happens to be. A tennis court is 23.77m x 8.23m and 23.77m x 10.97m for singles matches and doubles matches respectively. For 10 people playing in a doubles matches court, the device density is < 5/100m2. But normally, the density should be much lower as only 4 players can play at the same time in one tennis court. Nonetheless, it is generally sufficient if the outdoor device density is not higher than indoor device density. Therefore, the outdoor device density as captures should be updated.

3. Conclusions

Change the outdoor Device density from “<200 devices/100m2” to “<20 devices/100m2”.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]                      EPC Tag Data standards version 1.1 Rev.1.24

[16] 	Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

[17] 	Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

[18] 	Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

[18] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[20] 	Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

[21] 	Mishu M K, Rokonuzzaman M, Pasupuleti J, et al. An adaptive TE-PV hybrid energy harvesting system for self-powered iot sensor applications[J]. Sensors, 2021, 21(8): 2604.

[22]	Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

[23] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[24] 	H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

[25] 	Hoang T, Ferin G, Bantignies C, et al. Aging assessment of piezoelectric energy harvester using electrical loads[C]//Journal of Physics: Conference Series. IOP Publishing, 2019, 1407(1): 012078.

[26] 	https://www.usda.gov/foodwaste/faqs

[27] 	https://www.fda.gov/food/new-era-smarter-food-safety#:~:text=Welcome%20to%20the%20New%20Era,reducing%20the%20number%20of%20illnesses

[28] 	Alam AU, Rathi P, Beshai H, Sarabha GK, Deen MJ. Fruit Quality Monitoring with Smart Packaging. Sensors (Basel). 2021 Feb 22;21(4):1509. doi: 10.3390/s21041509. PMID: 33671571; PMCID: PMC7926787.

[29] 	Taoukis, Petros & Nychas, George-John. (2000). Use of time-temperature integrators and predictive modeling for shelf life control of chilled fish under dynamic storage conditions. International journal of food microbiology. 53. 21-31. 10.1016/S0168-1605(99)00142-7.

[30] 	Mishu, M.K., Rokonuzzaman, M., Pasupuleti, J., Shakeri, M., Rahman, K.S., Hamid, F.A., Tiong, S.K. and Amin, N., 2020. Prospective efficient ambient energy harvesting sources for iot-equipped sensor applications. Electronics, 9(9), p.1345.

[31] 	Bagchi, S., Abdelzaher, T.F., Govindan, R., Shenoy, P., Atrey, A., Ghosh, P. and Xu, R., 2020. New frontiers in IoT: Networking, systems, reliability, and security challenges. IEEE Internet of Things Journal, 7(12), pp.11330-11346.

[32] 	O’Neill, V. and Soh, B., 2022. Improving Fault Tolerance and Reliability of Heterogeneous Multi-Agent IoT Systems Using Intelligence Transfer. Electronics, 11(17), p.2724.

[33] 	Katanbaf, M., Jain, V. and Smith, J.R., 2020. Relacks: Reliable backscatter communication in indoor environments. Proceedings of the ACM on Interactive, Mobile, Wearable and Ubiquitous Technologies, 4(2), pp.1-24.

[34] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[35] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[36] 	https://www.mokosmart.com/use-iot-fire-detector-sensor/

[37]  	https://www.5gamericas.org/wpcontent/uploads/2019/07/5G_Americas_URLLLC_White_Paper_ Final__updateJW.pdf

[38] 	TS 22.261 https://www.3gpp.org/ftp/Specs/archive/22_series/22.261/22261-j00.zip

[39] 	 	    TR 22.866: "enhanced Relays for Energy Efficiency and Extensive Coverage".

[40] 		     		Global RFID Sensor Market - Global Industry Analysis, Size, Share, Growth, Trends

and Forecast 2021-2031, Transparency Market Research, June 2021

[41]		     		Sensors: Technologies and Global Markets (https://www.bccresearch.com/market-

research/instrumentation-and-sensors/sensors-technologies-markets-report.html),BCC

Research

[42]		     		Berckmans, D. Precision livestock farming technologies for welfare management in

intensive livestock systems. Rev. Sci. Tech. Off. Int. Epiz. 2014, 33, 189–196.

[43]	Precision Agriculture for Crop and Livestock Farming - Brief Review, Animals 2021

[44]                	Journal of Dairy Science - Comparison of 2 systems of pasture allocation on milking intervals and total daily milk yield of dairy cows in a pasture-based automatic milking system, American Dairy Science Association, 2013

[45]	Grazing and automation, Proceedings 4th Meeting European Grassland Federation WG “Grazing”, 2015

[46] 	Grazing Methods, UK Agriculture Food and Environment, https://grazer.ca.uky.edu/content/grazing-methods-which-one-you#:~:text=Strip%20Grazing%20%E2%80%93%20This%20technique%20involves,a%20new%20allocation%20of%20forage.

[47]                	Nora O’Donovan, https://www.independent.ie/business/farming/dairy/dairy-    advice/putting-a-size-on-paddocks-30447793.html

[48]	A. van den Pol-van Dasselaar, P. W. Blokland, et al, Beweidbare oppervlakte en weidegang op melkveebedrijven in Nederland (Pasture area and grazing on dairy farms in the Netherlands), Livestock Research Wageningen, 2015

[49]	Laura Paine, A Summary of Dairy Grazing Practices in Wisconsin

[50]	R. J. Rendell, Designing Paddocks on an Irrigated Dairy Farm, Victoria, Australia

[51]	The EU pig meat sector, European Parliament

[52]	http://www.bestgenetics.com.cn/EN/PigFarm1

[53]		    			Lisette. E. van der Zande, et al, Individual detection and tracking of group housed pigs

in their home pen using computer vision, Front. Anim. Sci. 2:669312. doi:

10.3389/fanim.2021.669312, April 2021

[54]	Carol Souza de Silva, Wageningen University, the impact of Pen Size and Stocking Density on behaviour and welfare of growing pigs

[55]	Xue Li, Xia Xiong, et al, Effects of stocking density on growth performance, blood parameters and immunity of growing pigs, December, 2020

[56] 	https://www.concreteconstruction.net/projects/infrastructure/so-many-manholes-so-little-time_o#:~:text=U.S.%20EPA%20estimates%20the%20number,from%20100%20to%20500%20feet.

[57]	http://news.cjn.cn/24hour/wh24/201101/t1276749.htm

[58] 	https://www.dibblecorp.com/insights/manhole-infrastructure-assessments/

[59] 	https://www.francebleu.fr/infos/faits-divers-justice/un-homme-retrouve-mort-dans-une-bouche-d-egout-a-saint-nazaire-1666456765

[60]	https://guancha.gmw.cn/2022-05/16/content_35739227.htm

[61]	Intelligent manhole cover national standard, GB/T 41401, 2022, the Ministry of Housing and Urban-Rural Development of the People's Republic of China

[62]	MEMS gauge pressure sensor NSPGD1 Novosense, liquid level measurement, https://www.novosns.com/Public/Uploads/uploadfile/files/20220726/-897.pdf

[63]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[64]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[65] 	Sung-Pil CHANG, Jinkyo CHOO, Values of Bridge in the Formation of Cities

[66]	https://www.thepaper.cn/newsDetail_forward_4643273

[67] 	https://news.sina.cn/gn/2021-12-19/detail-ikyamrmy9954352.d.html

[68] 	https://www.nytimes.com/2020/12/22/world/europe/genoa-bridge-collapse.html

[69]	https://www.theguardian.com/world/2022/oct/31/india-bridge-collapse-death-toll-rises-rescue-efforts-continue

[70]	Role of the Local Government in Monitoring and Maintenance Bridges, IAPA Proceedings Conference, 2019

[71]	https://elastisense.com/structural-health-monitoring-bridges

[72]	GB/T 39559.2-2020 National Specification for operational monitoring of urban rail transit facilities – Part 2: Bridge

[73]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[74]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[75]	Big data analysis for bridges health monitoring, Tongji University, 2017

[76]	Beijing Dongsha bridge safe operation monitoring, http://www.ccea.zju.edu.cn/_upload/article/files/64/e6/a824c7d04932af200750fbd9e690/fb0a5e9c-a1e7-47b8-bdb4-82b584d1c52f.pdf

[77]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[78]	https://www.rfidjournal.com/esls-are-becoming-more-affordable-for-retailers

[79]	https://www.newswire.ca/news-releases/sobeys-pilots-smart-cart-the-first-intelligent-grocery-shopping-cart-803013190.html

[80]                     https://en.wikipedia.org/wiki/Electronic_paper

[81]	F. Palacio, et al. (2006). "Active RFID tag with sensing capabilities and low power consumption".

[82]	T. O. John, H. C. Ukwuoma, S. Danjuma and M. Ibrahim, "Energy consumption in wireless sensor network", Energy, vol. 7, no. 8, 2016.

[83]	Tuna, G., and V. C. Gungor. "Energy harvesting and battery technologies for powering wireless sensor networks." Industrial Wireless Sensor Networks. Woodhead Publishing, 2016. 25-38.

[84]	M. Zeinali and J. Thompson, "Impact of Compression and Small Cell Deployment on NB-IoT Devices Coverage and Energy Consumption with a Realistic Simulation Model", Sensors. no. 19, 2021.

[y1]	Letizia Appolloni and Daniela D’Alessandro, “Housing Spaces in Nine European Countries: A Comparison of Dimensional Requirements”, International Journal of Environmental Research and Public Health, April 2021

* * * Second Change * * * *

### 5.26.6	 Potential New Requirements needed to support the use case

[PR.5.26.6-001] The 5G system shall be able to support mechanisms to communicate efficiently with Ambient IoT devices.

[PR.5.26.6-002] When setting up communication to an Ambient IoT device the 5G system shall be able to handle the unavailability of Ambient IoT devices either due to lack of power or due to power saving mechanisms of the Ambient IoT device.

[PR.5.26.6-003] The 5G system shall be able to provide an Ambient IoT service with following KPIs:

Table 5.26.6-1: Ambient IoT service KPI for elderly health care

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoors elderly health care scenario | 1 s | NA | NA | <1 kbit/s | <100 bits | <20 per 100 m2 | 20 m | <250 m2 (note 1) | Static | NA | NA | NA | NA |
| Outdoors elderly health care scenario | 1 s | NA | NA | <1 kbit/s | <100 bits | <20 per 100 m2 (note 2) | 200 m | City wide including rural areas | Static | NA | NA | NA | NA |
| NOTE 1: Average size of a big house. NOTE 2: Based on outdoor tennis court sizes (four players in a 23.77m x 10.97m doubles matches court), assuming one medicine box per elderly consumer, the device density is not higher than 5/100m2. |  |  |  |  |  |  |  |  |  |  |  |  |  |