3GPP TSG SA WG 1 Meeting #102 	S1-231437

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231225)

Source:	DENSO CORPORATION

pCR Title:	Pseudo-CR on Update KPI of Use case 5.28

Draft Spec:	3GPP TR 23.837 v.1.0.0

Agenda item:	7.1

Document for:	Approval

Contact:					Misato Mori (misato.mori.j3u@jp.denso.com)

Abstract: This document proposes to update KPI of use case 5.28.

1. Introduction

It is proposed to update KPI table and clean up the text.

2. Reason for Change

- KPI table

| KPI | Reason for Change |
| --- | --- |
| Sensing Service Area | In vehicles sensing for ADAS, sensing can be used for outdoor and indoor, e.g. indoor parking space. |
| Accuracy of positioning estimate by sensing, Vertical | To reduce the possibility of false detection in situations where there is an object such as a wall or sign in front of a vehicle, a vertical KPI should be set. For safe driving, the sensing should be able to distinguish whether the object in front of a vehicle is a vehicle on the same road or a wall several meters above the ground. Figure 1: Example of sensing an object in front of a vehicle. Figure 2: Example of sensing an object located several meters above the ground. |
| Accuracy of positioning estimate by sensing, Horizontal | Simplified KPIs by removing mimimum values and range of values. The range of values makes it difficlt to be consistent with other KPI values and increases complexity. |
| Accuracy of velocity estimate by sensing, Horizontal |  |
| Refreshing rate |  |
| Missed detection |  |

- Clean up the text

In order to align the text, “±” and unit symbols (m/s and %) can be removed from the KPI table.

3. Conclusions

Update KPI of use case 5.28.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v1.0.0.

* * * First Change * * * *

### 5.28.6	Potential New Requirements needed to support the use case

[PR 5.28.6-1] The 5G system shall be able to configure and authorize UEs supporting V2X applications to perform sensing.

[PR 5.28.6-2] The 5G system shall be able to collect charging information for UEs supporting V2X applications when performing sensing.

[PR 5.28.6-3] The 5G system shall be able to support the following KPIs:

Table 5.28.6-1 KPIs for Vehicles Sensing for ADAS

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| ADAS [Long range Radar] | Outdoor | [95] | [≤1.3] NOTE 2 | ≤0.5 | [≤ 0.12] NOTE 4 | N/A | [0.4] NOTE 5 | [≤ 0.6] NOTE 4 | [50] | [≤ 0.2] | [≤ 10] | [<1] |
| ADAS [Short range Radar] | Indoor (Parking space)/Outdoor | [95] | [≤2.6] NOTE 3 | ≤0.5 | ≤ 0.12 NOTE 4 | N/A | [0.4] NOTE 5 | [≤ 0.6] NOTE 4 | [20] | [≤ 0.05] | [≤ 10] | [<1] |
| NOTE 1: The terms in Table 5.28.6-1 are found in Section 3.1. NOTE 2: Assuming typical max range of 250m, range accuracy of 10cm, azimuth accuracy of ±0.3deg. Positioning accuracy as (Min-Max) within field of view. NOTE 3: Assuming typical max range of 30m, range accuracy of 2cm, azimuth accuracy of ±5deg.Positioning accuracy as (Min-Max) within field of view. NOTE 4: Velocity accuracy and resolution is typically reported for the radial velocity (not absolute H/V velocity). NOTE 5: The range resolution typically reported as 3D ranging distance accuracy. It determined assuming using 400 MHz bandwidth which is supported in 5G mmWave networks using formular C/2*B where C is the speed of light, B is the bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *