3GPP TSG- SA1 Meeting # 102 	S1-231440

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231128, 0448)

Title:	Consolidation requirements on FS_EnergyServ

Agenda Item:	7.10

Source:	China Mobile, Samsung

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This pCR provides consolidation requirements on FS_EnergyServ.

Energy Utilization as Service Criteria: 5.1, 5.5

Different energy states of network elements and network functions: 5.2

Monitoring and measurement related to energy efficiency: 5.1, 5.3, 5.4, 5.9, 5.10, (5.6, 5.8, 5.9 with FFS)

Information exposure related to Energy Usage: 5.3, 5.4, 5.5, 5.9, 5.10, (5.6, 5.7, 5.8, 5.9 with FFS)

6	Consolidated Potential Requirements

Editor's Note: The following consolidated potential requirements will be revised and updated as discussion going on to cover updates in existing use cases and potential requirements in new use cases.

6.1	Energy Utilization as Service Criteria

This subclause contains the requirements related to Energy Utilization as a Performance Criteria and supporting energy credit limit for specific service.

Table 6.1-1 – Energy Utilization as Service Criteria Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1-1 | Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy utilization rate for services without QoS criteria (also termed "best effort" services.) NOTE x: The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. This requirement's applicability is limited to UEs that only support services without QoS criteria. | PR 5.1.6-1 Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy utilization rate for services without QoS criteria (also termed "best effort" services.) NOTE 1: The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. This requirement's applicability is limited to UEs that only support services without QoS criteria. |  |
| CPR 6.1-2 | Subject to operator’s policy, the 5G system shall support enforcement of subscription policies that define a maximum energy utilization rate for services without associated QoS criteria (also termed "best effort" services.) | PR 5.1.6-2 Subject to operator’s policy, the 5G system shall support enforcement of subscription policies that define a maximum energy utilization rate for services without associated QoS criteria (also termed "best effort" services.) |  |
| CPR 6.1-3 | The 5G network shall support a means to define maximum energy utilization rate with specific granularities: a) subscriber granularity (considering all services of the 5G network for the subscriber); b) network slice granularity. NOTE x: The energy utilization of the UE is out of scope of this requirement. | [PR 5.1.6-3] The 5G network shall support a means to define maximum energy utilization rate with specific granularities: a) subscriber granularity (considering all services of the 5G network for the subscriber); b) network slice granularity. NOTE 2: The energy utilization of the UE is out of scope of this requirement. |  |
| CPR 6.1-4 | Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services. | P.R 5.5.6-1 Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services. |  |
| CPR 6.1-5 | Subject to operator’s policy, the 5G system shall support subscription policies that support a means to associate energy utilization units with charging records. | P.R 5.5.6-2 Subject to operator’s policy, the 5G system shall support subscription policies that support a means to associate energy utilization units with charging records. |  |
| CPR 6.1-6 | Subject to operator’s policy, the 5G system shall support a mechanism to perform energy utilization credit limit control for services. NOTE x: The result of the credit control is not specified by this requirement. Examples include gating, increased charging rates, etc. NOTE x: Credit control [18] compares against a credit control limit. In this use case, charging events are assigned a corresponding energy utilization and this is compared against a policy of energy credit limit. The use case assumes it is possible that there is a new policy to limit energy utilization allowed. | P.R 5.5.6-4 Subject to operator’s policy, the 5G system shall support a mechanism to perform energy utilization credit limit control for services. NOTE 1: The result of the credit control is not specified by this requirement. Examples include gating, increased charging rates, etc. NOTE 2: Credit control [18] compares against a credit control limit. In this use case, charging events are assigned a corresponding energy utilization and this is compared against a policy of energy credit limit. The use case assumes it is possible that there is a new policy to limit energy utilization allowed. |  |

6.2	Different energy states of network elements and network functions

This subclause contains the requirements related to different energy states of network elements and network functions and dynamic changes.

Table 6.2-1 – Different energy efficiency modes of network functions Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | The 5G system shall support different energy states of network elements and network functions. | PR 5.2.6-1 The 5G system shall support different energy states of network elements and network functions. |  |
| CPR 6.2-2 | 5G system shall support dynamic changes of energy states of network elements and network functions based on pre-configured policy with authorised 3rd party. NOTE x: Pre-configured policy may include: the time of changing energy states, which energy state map to which level of load, etc. | PR 5.2.6-2 The 5G system shall support dynamic changes of energy states of network elements and network functions, based on pre-configured policy with authorised 3rd party. NOTE 1: Pre-configured policy may include: the time of changing energy states, which energy state map to which level of load, etc. |  |
| CPR 6.2-3 | The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions. | PR 5.2.6-3 The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions. |  |

6.3	Monitoring and measurement related to energy efficiency

This subclause contains the requirements related to different energy efficiency modes of network functions and dynamic changes.

Table 6.3-1 – Monitor related to energy efficiency Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to operator's policy, the 5G network shall support energy utilization monitoring at per network slice and per subscriber granularity. NOTE x: Energy utilization monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. | [PR 5.1.6-4] Subject to operator's policy, the 5G network shall support energy utilization monitoring at per network slice and per subscriber granularity. NOTE 3: Energy utilization monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. |  |
| CPR 6.4-1 | Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios. | PR.5.3.6-1 Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties. |  |
| CPR 6.4-2 | Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy usage information of the NPN, including the shared network function(s) which is (are) serving the NPN. | [PR.5.4.6-1] Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy efficiency information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties. |  |
| CPR 6.4-8 | Subject to operator’s policy, the 5G system shall be able to acquire the ratio of renewable energy used to provide dedicated communication service to the 3rd party on periodic basis. NOTE x: The reporting period could be set, e.g., on monthly or yearly basis. | [PR.5.9.6-2] Subject to operator’s policy, the 5G system shall be able to provide to a 3rd party the reporting of the ratio of renewable energy used to provide dedicated communication service to the 3rd party on periodic basis. NOTE 2: The reporting period could be set, e.g., on monthly or yearly basis. |  |
| CPR 6.4-9 | Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to acquire the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s energy consumption, and the carbon intensity of operator’s network. NOTE x: The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations. NOTE x: The granularity of reporting (e.g., per month) is not discussed in this study. | [PR 5.10.6-1] Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide a mechanism to expose to the authorized third parties the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s energy consumption, and the carbon intensity of operator’s network. NOTE 1: The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations. NOTE 2: The granularity of reporting (e.g., per month) is not discussed in this study. |  |

Editor's Note: The potential requirements PR 5.6.6-1, PR 5.8.6-1, PR 5.9.6-1 will be considered to consolidation after the ENs are resolved.

6.4	Information exposure related to Energy Usage

This subclause contains the requirements related to information exposure related to energy usage.

Table 6.4-1 – Information exposure related to Energy Usage Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties. | PR.5.3.6-1 Subject to operator’s policy and consent by the vertical customer, the 5G system shall be able to acquire energy usage information of the network functions serving the customer, independently from NG-RAN deployment scenarios, and expose this information to the customer and authorized third parties. |  |
| CPR 6.4-2 | Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy usage information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties. | [PR.5.4.6-1] Subject to operator’s policy and consent by the customer of NPN, the 5G system shall be able to acquire energy efficiency information of the NPN, including the shared network function(s) which is (are) serving the NPN, and expose this information to the NPN customer and authorized third parties. |  |
| CPR 6.4-3 | Subject to operator’s policy, the 5G system shall support a means to expose energy utilization to authorized third parties for services, including energy utilization information clearly identifies the 'approaching' enforcement of an energy credit limit. | P.R 5.5.6-3 Subject to operator’s policy, the 5G system shall support a means to expose energy utilization to authorized third parties for services, such that the energy utilization information clearly identifies the 'approaching' enforcement of an energy credit limit. |  |
| CPR 6.4-8 | Subject to operator’s policy, the 5G system shall be able to provide to a 3rd party the reporting of the ratio of renewable energy used to provide dedicated communication service to the 3rd party on periodic basis. NOTE x: The reporting period could be set, e.g., on monthly or yearly basis. | [PR.5.9.6-2] Subject to operator’s policy, the 5G system shall be able to provide to a 3rd party the reporting of the ratio of renewable energy used to provide dedicated communication service to the 3rd party on periodic basis. NOTE 2: The reporting period could be set, e.g., on monthly or yearly basis. |  |
| CPR 6.4-9 | Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide a mechanism to expose to the authorized third parties the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s energy consumption, and the carbon intensity of operator’s network. NOTE x: The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations. NOTE x: The granularity of reporting (e.g., per month) is not discussed in this study. | [PR 5.10.6-1] Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide a mechanism to expose to the authorized third parties the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s energy consumption, and the carbon intensity of operator’s network. NOTE 1: The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations. NOTE 2: The granularity of reporting (e.g., per month) is not discussed in this study. |  |

Editor's Note: The potential requirements PR 5.6.6-1, PR 5.7.6-1, PR 5.7.6-2, PR 5.8.6-2, PR 5.9.6-1 will be considered to consolidation after the ENs are resolved.