3GPP TSG-SA1 Meeting #102	S1-23456

Berlin, Germany, 22 – 26 May, 2023	  (Revision of S1-231444)

Title:	Ambient IoT functional CPR

Agenda Item:	7.2

Source:	OPPO (Rapporteur)

Contact:	Yang Xu 	xuyang@oppo.com

Weijie Xu xuweijie@oppo.com

Abstract: This document is for AIoT functional CPR proposal with consideration of latest discussion.

****************Start of Change***************

7	Consolidated potential requirements and KPIs

7.1	Consolidated potential requirements

Mapping table is used per each subclause for consolidated potential requirements.

7.1.1	Communication aspects of Ambient IoT devices

Table 7.1.1 – [Communication aspects of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.1-1 | The 5G system shall be able to support mechanisms to communicate with an ambient IoT device. Note 1: for example, the communication between 5G network/UE and Ambient IoT device can include periodic sensor reporting or network-initiated inventory. | PR 5.2.6-001 PR.5.6.6-001 PR 5.11.6-001 PR.5.12.6-002 PR.5.13.6-001 PR.5.1.6-001 PR.5.4.6-001 PR.5.2.6-002 PR.5.12.6-002 PR.5.8.6-003 PR.5.27-004 PR.5.13.6-004 PR.5.3.6-001 PR 5.22.6-1, PR5.23.6-1, PR.5.24.6-1 PR.5.26.6-001 |  |
| CPR 7.1.1-2 | The 5G system shall be able to support UE or 5G network to communicate with a group of Ambient IoT devices simultaneously. |  |  |
| CPR 7.1.1-3 | The 5G system shall support energy efficient communication for an Ambient IoT device while meeting the communication performance requirements. | PR 5.3.6-001, PR 5.13.6-004, PR 5.22.6-001, |  |
| CPR 7.1.1-4 | The 5G network shall support a mechanism to authorize a UE to communicate with an Ambient IoT device. | PR 5.8.6-002 PR.5.12.6-001 PR.5.14.6-001 PR.5.21.6-003 PR.5.21.6-001 |  |

7.1.2	Positioning/location of Ambient IoT devices

Table 7.1.2 – [Positioning/location of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G system shall support positioning services for Ambient IoT devices. | PR 5.9.6-002 PR 5.12.6-004 PR 5.2.6-004 |  |

7.1.3	Management of Ambient IoT devices

Table 7.1.3  [Management of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.3-2 | The 5G system shall support a mechanism to: - disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals - enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently not able to transmit RF signals | P.R.5.1.6-005 PR 5.16.6-003 PR 5.17.6-001 |  |

7.1.4	Collected information and network capability exposure

Table 7.1.4  [Collected information and network capability exposure] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.4-1 | Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to get information from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party. | PR.5.1.6-002 PR.5.12.6-003 PR 5.13.6-005 PR.5.14.6-003 PR 5.16.6-004 PR 5.18.6-1 PR.5.21.6-004 PR 5.2.6-003 |  |
| CPR 7.1.4-2 | Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices to the trusted 3rd party. Note: A third party can request information on e.g. the Ambient IoT device identity and position. |  |  |
| CPR 7.1.4-5 | The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value). | PR 5.3.6-003 PR 5.19.6-001 PR.5.27-003 |  |

7.1.5	Charging

Table 7.1.5  [Charging] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

7.1.6	Security and privacy

Table 7.1.6  [Security and privacy] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.6-2 | The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device. | P.R.5.1.6-003 PR 5.3.6-002 PR.5.6-002 P.R.5.13.6-002 PR.5.21.6-005 PR.5.8.6-004 P.R.5.1.6-004 PR 5.8.6-005 PR. 5.12.6-006 P.R.5.13.6-003 PR. 5.14.6-005 PR 5.16.6-001 PR.5.20.6-002 |  |
| CPR 7.1.6-4 | The 5G network shall provide a suitable means for the operator to authorize communication between a UE and an Ambient IoT device. | PR. 5.12.6-005 PR. 5.14.6-004 |  |

*******************End of Change*****************