3GPP TSG SA WG 1 Meeting #102 	S1-231506

Berlin, Germany, 22-26 May 2023	(revision of S1-231344)

(revision of S1-231019)

Source:	ZTE, China Unicom

pCR Title:	Requirements Consolidation- Further Aspects

Draft Spec:	3GPP TR 22.851 V1.1.0

Agenda item:	7.4

Document for:	Approval

Contact:	Yvette Koza

Yvette.koza@zte.com.cn

Abstract: Requirement Consolidation-Other aspects on Network Sharing.

1. Introduction

Consolidation of requirements according to the defined Use cases.

2. Reason for Change

Consolidations of requirements on some other aspects have been handled in separate contributions. In this contribution consolidated requirements with regard to Mobility Aspects, Roaming as well as Regulatory Services are defined.

3. Conclusions

None.

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.851 V1.1.0.

* * *For Information * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Indirect Network Sharing: describes the communication between the shared access NG-RAN and the Participating NG-RAN Operator’s core network being routed through the Hosting NG-RAN Operator’s core network.

Hosted Service: a service containing the operator's own application(s) and/or trusted third-party application(s) in the Service Hosting Environment, which can be accessed by the user.

Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

Hosting NG-RAN Operator: the operator that has operational control of a shared NG-RAN.

NOTE 1: Hosting NG-RAN Operator can also be a Hosting RAN Operator. See 3GPP TS 22.101 [2].

Participating NG-RAN Operator: authorized operator that is sharing NG-RAN resources provided by a Hosting NG-RAN Operator.

NOTE 2: Participating NG-RAN Operator can also be participating operator. See 3GPP TS 22.101 [2].

Shared NG-RAN: NG-RAN that is shared among a number of operators.

* * * First Change * * * *

# 7	Consolidated requirements

## 7.1	General

Potential requirements on Indirect Network Sharing have been identified in Section 5 out of the use cases under the following aspects

- General

- Mobility

- Network access control

- Regulatory services

- Charging

These requirements have been especially introduced to support the newly defined network sharing scenario, where a NG-RAN is shared among multiple operators without necessarily assuming a direct connection between the shared NG-RAN and the Participating NG-RAN Operator’s core network.

NOTE: These enhanced requirements may go beyond existing RAN sharing requirements in Rel-13.

* * * Next Change * * * *

### 7.2.X	Mobility

Table 7.2.X-1 Consolidated Requirements to Mobility

| CPR | Consolidated Potential Requirement | Original PR | Comment |
| --- | --- | --- | --- |
| CPR 7.2.X-001 | In case of Indirect Network Sharing scenario and subject to hosting and participating operators’ policies, the 5G system shall support service continuity and minimize the impact on the user experience and service interruptions between different Shared NG-RANs and/or between a Shared NG-RAN and a non-Shared NG-RAN networks. | [PR 5.2.6-002] [PR 5.3.6-001] [PR 5.2.6-003] | PRs merged |
| CPR 7.2.X-002 | In case of indirect network sharing, the 5G system shall enable the Shared NG-RAN of a hosting operator to provide services for inbound roaming users. NOTE: Inbound roaming users may not have a roaming agreement with the hosting operator but only with a participating operator. | [PR 5.4.6-001] | PR modified |

### 7.2.Y	Regulatory Services

Table 7.2.Y-1 Consolidated Requirements to the Regulatory Services

| CPR | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2.Y-001 | In case of Indirect Network Sharing and subject to regulatory requirements and mutual agreement between the participating operators and the hosting operator, the 5G system shall support regulatory services (e.g., PWS, emergency calls). | [PR 5.6.6-001] [PR 5.8.6-001] | PRs merged |
| CPR 7.2.Y-002 | In Indirect Network Sharing, the 5G system shall be able to provide a UE accessing a Shared NG-RAN network with positioning service in compliance with regulatory requirements. | [PR 5.6.6-002] | PR modified |
| CPR 7.2.Y-003 | Subject to regulatory requirements and mutual agreement between the participating operators and the hosting operator the Shared NG-RAN operator shall be able to broadcast PWS messages originated from the core network of the hosting operator. | [PR 5.8.6-002] | PR modified |

* * * End of Changes * * * *