3GPP TSG-SA WG1 Meeting #102 	S1-231584

Berlin, Germany, 22-26 May 2023	(revision of S1-231092)

Title:	22.856 pCR UPdate 5.8 Use Case Supporting multi-service coordination in one 
	mobile metaverse service

Agenda Item:	7.3

Document for:	Approval

Source:	Samsung, China Mobile

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This pCR proposes revision of an existing requirement that is currently 'for further study' for the use case in 5.8.

Introduction

Revises the sole requirement for the use case to add clarity and remove the 'For further study' editor's note.

A clear problem with supporting more than one AR media service at the same time is that the characteristics of the media may vary. For example, one media may be lower resolution or have higher delay than the other. In the example used in the use case, the user is painting on a virtual easel while watching a concert. If the live concert media 'refresh rate' were too slow, inconsistencies would emerge between the foreground (easel) and background (concert). Such gaps would rapidly become apparent to the user (perhaps as gaps or holes in the concert presentation).

The requirement has been restated below as:

[PR 5.8.6-1] The 5G system shall provide the capability of reduce the differences between different mobile metaverse services communication performance for a given UE to prevent inconsistency of experience due to XR media with divergent characteristics, e.g. resolution, latency or packet loss.

This concretely offers a means to coordinate multiple applications: to provide a capability to reduce the divergence in communication performance between different mobile metaverse services. This is both achievable and not yet supported by the current system.

The coordination discussed in this pCR is for services that are provided to a single UE, where that UE can have more than one 'terminal equipment' that it serves.

TR 21.905 includes this definition:

Terminal Equipment (TE): Equipment that provides the functions necessary for the operation of the access protocols by the user. A functional group on the user side of a user-network interface (source: ITU-T I.112).

The UE's 'upper layers' may be in more than one device, e.g. VR glasses, gloves, etc.

Proposal

It is proposed to agree to the changes below to 22.856, 1.0.0.

Start of changes

## 5.8	Supporting multi-service coordination in one mobile metaverse service

### 5.8.1	Description

There’s a major difference between a metaverse service and traditional multi-media service. A mobile metaverse service provides a platform which supports different applications to complete a task such as gaming, online-working, online-education, etc. Users will have no limitations on the terminals they use. In existing XR applications, specific brand of VR glasses or gloves are required to be used in a game, different brands of VR glassed and gloves will be very hard to map and coordinate in a same game. But in mobile metaverse services, the nature of the standard will support the coordination between different equipment belonging to different applications or brands.

metaverse

### 5.8.2	Pre-conditions

John has a pair of VR glasses and a pair of tactile gloves. Usually, he uses VR glasses for VR games and tactile gloves for vertical painting where he can feel the brushstrokes. These two activities were running on two different network slices. As the VR glasses was bought to play VR games, the VR game application has a network slice A which is better support the game service. Tactile gloves belong to Brand B which has another network slice B.

In the mobile metaverse, there are many different types of services such as games, concerts, education, etc. And the mobile metaverse services has subscribed different network slice for these different types of service, and different QoS for different flows accordingly for better user experience.

### 5.8.3	Service Flows

1. John opens a mobile metaverse service, in which both VR glasses and tactile gloves are needed, and he would like to draw a picture with tactile gloves and see a live music show at the same time.

2. In the subscription between mobile metaverse service (which can be hosted by operators or other companies) and network, the video flow, audio flow in live music condition were subscribes to QoS 1 and QoS2 in slice A, the video flow and tactile flow in painting condition were subscribes to QoS 3 and QoS4 in slice B in art painting.

3. In John’s VR glasses he can see the singer and other listeners and at the same time. At the same time, he can see his painting on a virtual easel and use a virtual brush to paint, while he can feel the brushstrokes with the tactile feedback.

4. In this case, the mobile metaverse service will have a policy to use a same QoS level for the video flows in live music condition and painting condition and inform network on this decision.

5. As John is painting and enjoying the live show at the same time, the coordination between video flow, audio flow in live music mobile metaverse service and the video flow and tactile flow in painting mobile metaverse service need to be coordinated. This coordination information need to be share with the network for policy modification.

6. Network will do this dynamic policy modification for John.

### 5.8.4	Post-conditions

John used both the VR glasses and the tactile gloves in distinct mobile metaverse services with very good user combined experience.

### 5.8.5	Existing features partly or fully covering the use case functionality

The 5G system can support different communication performance policies for services and provides some support for resolving conflicts between the policies of different services.

There is however no way to for the 5G system to coordinate the communication performance delivered so that divergence in communication performance is reduced for distinct services (i.e. from different service providers).

3GPP TS 23.503 [xx] clause 4.3.1 includes the following general requirement "The PCC framework shall allow the resolution of conflicts which would otherwise cause a subscriber's Subscribed Guaranteed Bandwidth QoS to be exceeded.".

3GPP TS 23.503 [xx] clause 6.1.3.7 explains that "Service pre-emption priority enables the PCF to resolve conflicts where the activation of all requested active PCC rules for services would result in a cumulative authorized QoS which exceeds the Subscribed Guaranteed bandwidth QoS.".

A note in 3GPP TS 23.503 [xx] clause 6.1.3.7 includes the following sentence: "Normative PCF requirements for conflict handling are not defined.".

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-1] The 5G system shall provide the capability of reducing the differences between different mobile metaverse services communication performance for a given UE to prevent inconsistency of experience due to XR media with divergent or conflicting characteristics, e.g., resolution, latency or packet loss.

NOTE:	The UE can provide communication services for more than one terminal equipment.

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., "Large-scale Localization Datasets in Crowded Indoor Spaces, " NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837: " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928: "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA "C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	 ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[40] 	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[41] 	3GPP TS 22.826: "Study on communication services for critical medical applications".

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[43]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[44]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[45]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[46]	3GPP TS 22.105: "Services and service capabilities".

[47]	Virtual humans: https://en.wikipedia.org/wiki/Virtual_humans

[48]	Kwang Soon Kim, et al., "Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[49]	I.-J. Hirsh and C. E. J. Sherrick, "Perceived order in different sense modalities, " Journal of Experimental Psychology, vol. 62, no. 5, pp. 423–432, 1961.

[50]                      VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

[51]	3GPP TR 23.700-80: "Study on 5G System Support for AI/ML-based Services".

[52]	EU data protection rules | European Commission (europa.eu) (https://ec.europa.eu/info/law/law-topic/data-protection/eu-data-protection-rules_en)

[53] 	California Consumer Privacy Act (CCPA) | State of California - Department of Justice - Office of the Attorney General (https://oag.ca.gov/privacy/ccpa)

[54]	Y. Sun, Z. Chen, M. Tao, and H. Liu, "Communications, caching, and computing for mobile virtual reality: Modelingand trade off, " IEEE Trans. Commun., vol. 67, no. 11, pp. 7573–7586, Nov. 2019.

[55]	Y. Cai, J. Llorca, A. M. Tulino and A. F. Molisch, "Compute- and Data-Intensive Networks: The Key to the Metaverse," 2022 1st International Conference on 6G Networking (6GNet), 2022.

[56]	3GPP TS 22.226: "Global Text Telephony"

[57]	ITU-T SG16 "ITU-T SG 16 Work on Accessibility - How ITU is Pioneering Telecom Accessibility for All", https://www.itu.int/en/ITU-T/studygroups/com16/accessibility/Pages/telecom.aspx, accessed 30.1.23.

[58]	Ankit Ojha, Ayush Pandey, Shubham Maurya, Abhishek Thakur, Dr. Dayananda P, "Sign Language to Text and Speech Translation in Real Time Using Convolutional Neural Network," INTERNATIONAL JOURNAL OF ENGINEERING RESEARCH & TECHNOLOGY (IJERT) NCAIT – 2020 (Volume 8 – Issue 15).

[59] 	R. Kazantsev and D. Vatolin, "Power Consumption of Video-Decoders on Various Android Devices," 2021 Picture Coding Symposium (PCS), Bristol, United Kingdom, 2021, pp. 1-5, doi: 10.1109/PCS50896.2021.9477481.

[60]	glTF 2.0 specification, https://registry.khronos.org/glTF/specs/2.0/glTF-2.0.html, accessed 02/02/2023..

[61]	B. Egger, W. A. P. Smith, A. Tewari, S. Wuhrer, M. Zollhoefer, T. Beeler, F. Bernard, T. Bolkart, A. Kortylewski, S. Romdhani, C. Theobalt, V. Blanz, and T. Vetter. "3D Morphable Face Models—Past, Present, and Future". ACM Trans. Graph. 39, 5, Article 157 (Jun 2020)

[62]	M. Zollhöfer, J. Thies, P. Garrido, D. Bradley, T. Beeler, P. Pérez, M. Stamminger, M. Nießner, and C. Theobalt. "State of the Art on Monocular 3D Face Reconstruction, Tracking, and Applications". Computer Graphics Forum (2018).

[x]				3GPP TS 23.503, "	Policy and charging control framework for the 5G System (5GS); Stage ".

End of changes