3GPP TSG-SA WG1 Meeting #102 	S1-231623

Berlin, Germany, 22-26 May 2023	(revision of S1-231094, 1582)

Title:	22.856 pCR: Removal of all Editor's Notes

Agenda Item:	7.3

Document for:	Approval

Source:	Samsung, Huawei

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: To prepare the TR for approval, this pCR removes all Editor's Notes and material that remains FFS. To retain this material, other pCRs that are agreed at SA1 102 can replace the changes in this pCR.

changes in S1-231623

- removed changes due to agreements in 1690 (5.7), 1727 (5.8), 1598 (5.17)

changes in S1-231582

- removed changes to 5.15, 5.27, 5.28, Annex C

- updated the terminology for 'immersive' (adapting the Cambridge dictionary definition)

- removed 'digital asset container' (the text can be used as a starting point to introduce digital asset functionality in the new TS, perhaps)

Introduction

As per the abstract.

Proposal

It is proposed to agree to the changes below to 22.856, 1.0.0.

Start of changes

Next change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

autonomous virtual alter ego: an AI-based digital representation behaving autonomously on behalf of a user herself/himself in the mobile metaverse services.

avatar: a digital representation specific to media that encodes facial (possibly body) position, motions and expressions of a person or some software generated entity.

Conference: An IP multimedia session with two or more participants. Each conference has a "conference focus". A conference can be uniquely identified by a user. Examples for a conference could be a Telepresence or a multimedia game, in which the conference focus is located in a game server.

NOTE 1: This definition was taken from TS 22.228 [2].

Conference Focus: The conference focus is an entity which has abilities to host conferences including their creation, maintenance, and manipulation of the media. A conference focus implements the conference policy (e.g. rules for talk burst control, assign priorities and participant’s rights).

NOTE 2: This definition was taken from TS 22.228 [2].

digital asset: digitally stored information that is uniquely identifiable and can be used to realize value according to their licensing conditions and applicable regulations. Examples of digital assets include digital image (avatar), software licenses, gift certificates and files (e.g. music files) that have been purchased under a license that allows resale.

digital representation: the mobile metaverse media associated with the presentation of a particular virtual or physical object. The digital representation could present the current state of the object. One example of a digital representation is an avatar, see Annex A.

digital twin: A real-time representation of physical assets in a digital world.

Note: This definition was taken from [29].

gesture: a change in the pose that is considered significant, i.e. as a discriminated interaction with a mobile metaverse service.

immersive: a characteristic of a service experience or AR/MR/VR media, seeming to surround the user, so that they feel completely involved.

localization: A known location in 3 dimensional space, including an orientation, e.g. defined as pitch, yaw and roll.

location related service experience: user interaction and information provided by a service to a user that is relevant to the physical location in which the user accesses the service.

location agnostic service experience: user interaction and information provided by a service to a user that has little or no relation to the physical location in which the user accesses the service. Rather the service provides interaction and information concerning either a distant or a non-existent physical location.

mobile metaverse media: media communicated or enabled using the 5G system including audio, video, XR (including haptic) media, and data from which media can be constructed (e.g. a 'point cloud' that could be used to generate XR media.)

mobile metaverse: the user experience enabled by the 5G system of interactive and/or immersive XR media, including haptic media.

mobile metaverse server:	an application server that supports one or more mobile metaverse services to a user access by means of the 5G system.

mobile metaverse service: the service that provides a mobile metaverse experience to a user by means of the 5G system.

pose: the relative location, orientation and direction of the parts of a whole. The pose can refer the user, specifically used in terms of identifying the position of a user's body. The pose can also also refer to an entity or object (whose parts can adopt different locations, orientations, etc.) that the user interacts with by means of mobile metaverse services.

service information: this information is out of scope of standardization but could contain, e.g. a URL, media data, media access information, etc. This information is used by an application to access a service.

spatial anchor: an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content.

spatial map: A collection of information that corresponds to space, including information gathered from sensors concerning characteristics of the forms in that space, especially appearance information.

spatial mapping service: A service offered by a mobile network operator that gathers sensor data in order to create and maintain a Spatial Map that can be used to offer customers Spatial Localization Service.

spatial localization service: A service offered by a mobile network operator that can provide customers with Localization.

User Identifier: a piece of information used to identify one specific User Identity in one or more systems.

NOTE 6: This definition was taken from TS 22.101 [4].

User Identity: information representing a user in a specific context. A user can have several user identities, e.g. a User Identity in the context of his profession, or a private User Identity for some aspects of private life.

NOTE 7: This definition was taken from TS 22.101 [4].

User Identity Profile: A collection of information associated with the User Identities of a user.

NOTE 8: This definition was taken from TS 22.101 [4].

Next change

# 4	Overview

Mobile metaverse services are discussed in this technical report both in the abstract and concrete. Specific services mentioned in the TR include:

-	Situational awareness for drivers, pedestrians, cyclists, to increase safety and efficiency of transport (see 5.2).

-	XR enabled collaborative and concurrent engineering, to enable local and remote collaboration (see 5.3).

-	Participatory in and passive observation of virtual reality events, e.g. basketball (see 5.6).

-	Presentation of AR content, e.g. a feature length movie, on a virtual screen (see 5.7).

-	Remote critical health care, including surgery and treatment (see 5.10).

The study also considers a number of use cases that feature new service enablers, including:

-	Providing users with information and services that are of local relevance (see 5.1).

-	Enhancements to IMS to support multiple users and multi-modal XR communication (see 5.3).

-	Support for spatial anchors to link service information to specific locations (see 5.4).

-	Support for spatial localization and mapping services, and enablers for them in the 5GS (see 5.5).

-	Support for multi-service coordination for different input and output devices and diverse services (see 5.8).

-	Support for synchronization of different data streams and predicted network conditions (especially latency) to enable immersive remote collaboration despite significant distance and therefore communication delay between participants (see 5.9).

Next change

### 5.10.6	Potential New Requirements needed to support the use case

[PR 5.10.6-1] The 5G system shall provide a means to synchronize multiple service data flows (e.g., heart rate, video, audio) of multiple UEs associated with Critical HealthCare services.

Next change

### 5.14.1	Description

The concept "mobile metaverse" and "metaverse" became popular during the coronavirus pandemic as lockdown measures and work-from-home policies pushed more people online for both business and pleasure, increasing demand for ways to make online interaction more lifelike. The term covers a wide variety of location agnostic service experiences, from workplace tools to games and community platforms. It generally refers to shared and immersive digital service experiences (i.e. mobile metaverse services) that people can experience by means of using XR devices. By 2026, 25% of people are estimated to spend at least one hour a day using services that provide immersive XR media for work, shopping, education, social media and/or entertainment, according to the latest study by Gartner, Inc. (a U.S.-based technology research and consulting company).

Mobile metaverse service technologies are still in the early stages of being adopted. Currently there are already many digital environments (i.e. mobile metaverse services that offer location agnostic and location related service experiences), which typically run in silos and are not interconnected. From end users’ view point, there are several basic requirements to be addressed:

- Depending on the immersive XR media service the user wants to connect to, he/she can choose his/her digital representation and the related information when needed: avatar (one or more), e-money (e.g. financial services as payment, his/her means of payment), ID, purchased items…

- A user is able to transition between immersive mobile metaverse services using the similar digital representation seamlessly and taking into account the constraints of the mobile metaverse services accessed. The transfer of information via the operator's network ensures the semantic compatibility – possibly through abstraction - between the origin and the destination. This also ensures, if necessary, the confidentiality of the origin and the destination.

In the following use case, Alice uses an immersive mobile metaverse service of a travel company, and a trip interests her. She would like to verify if she has enough money to buy the trip. She needs to link the immersive mobile metaverse service of her bank with the current immersive mobile metaverse service of the travel company so that her profile is automatically shared between these two services (she has previously given authorisation).

In this use case 'digital representations' are expected to interwork with specific services. The use case does not propose to define a new 'standard' for digital representations (avatars, electronic money and financial service, IDs, purchased items.) Rather personal information is stored and retrieved to improve service delivery and to assure privacy and security. If some formats, etc. are shared between different service providers, this would present an opportunity for consistency and continuity for the user of those two services, enabled by this use case.

Next change

### 5.21.5	Existing features partly or fully covering the use case functionality

No existing features are identified.

### 5.21.6	Potential New Requirements needed to support the use case

[PR 5.21.6-1] Subject to the operator’s policy, 5G system shall be able to provide the means for the authorized third party to provide the input data about specific virtual event(s) e.g. failure of the NW equipment, burst traffic on the virtual 5G system(s) for the mobile Metaverse service.

Next change

### 5.23.6	Potential New Requirements needed to support the use case

Next change

### 5.24.6	Potential New Requirements needed to support the use case

[PR 5.24.6-1] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall support mechanisms to identify an avatar and associate the avatar with a subscriber (i.e. the owner of the avatar).

[PR 5.24.6-2] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall be able to authorize the avatar to be used in mobile metaverse services.

[PR 5.24.6-3] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall provide time-bound authorization services for an avatar to be used in mobile metaverse services.

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., "Large-scale Localization Datasets in Crowded Indoor Spaces, " NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837: " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928: "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA "C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	 ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[40] 	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[41] 	3GPP TS 22.826: "Study on communication services for critical medical applications".

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[43]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[44]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[45]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[46]	3GPP TS 22.105: "Services and service capabilities".

[47]	Virtual humans: https://en.wikipedia.org/wiki/Virtual_humans

[48]	Kwang Soon Kim, et al., "Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[49]	I.-J. Hirsh and C. E. J. Sherrick, "Perceived order in different sense modalities, " Journal of Experimental Psychology, vol. 62, no. 5, pp. 423–432, 1961.

[50]                      VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

[51]	3GPP TR 23.700-80: "Study on 5G System Support for AI/ML-based Services".

[52]	EU data protection rules | European Commission (europa.eu) (https://ec.europa.eu/info/law/law-topic/data-protection/eu-data-protection-rules_en)

[53] 	California Consumer Privacy Act (CCPA) | State of California - Department of Justice - Office of the Attorney General (https://oag.ca.gov/privacy/ccpa)

[54]	Y. Sun, Z. Chen, M. Tao, and H. Liu, "Communications, caching, and computing for mobile virtual reality: Modelingand trade off, " IEEE Trans. Commun., vol. 67, no. 11, pp. 7573–7586, Nov. 2019.

[55]	Y. Cai, J. Llorca, A. M. Tulino and A. F. Molisch, "Compute- and Data-Intensive Networks: The Key to the Metaverse," 2022 1st International Conference on 6G Networking (6GNet), 2022.

[56]	3GPP TS 22.226: "Global Text Telephony"

[57]	ITU-T SG16 "ITU-T SG 16 Work on Accessibility - How ITU is Pioneering Telecom Accessibility for All", https://www.itu.int/en/ITU-T/studygroups/com16/accessibility/Pages/telecom.aspx, accessed 30.1.23.

[58]	Ankit Ojha, Ayush Pandey, Shubham Maurya, Abhishek Thakur, Dr. Dayananda P, "Sign Language to Text and Speech Translation in Real Time Using Convolutional Neural Network," INTERNATIONAL JOURNAL OF ENGINEERING RESEARCH & TECHNOLOGY (IJERT) NCAIT – 2020 (Volume 8 – Issue 15).

[59] 	R. Kazantsev and D. Vatolin, "Power Consumption of Video-Decoders on Various Android Devices," 2021 Picture Coding Symposium (PCS), Bristol, United Kingdom, 2021, pp. 1-5, doi: 10.1109/PCS50896.2021.9477481.

[60]	glTF 2.0 specification, https://registry.khronos.org/glTF/specs/2.0/glTF-2.0.html, accessed 02/02/2023..

[61]	B. Egger, W. A. P. Smith, A. Tewari, S. Wuhrer, M. Zollhoefer, T. Beeler, F. Bernard, T. Bolkart, A. Kortylewski, S. Romdhani, C. Theobalt, V. Blanz, and T. Vetter. "3D Morphable Face Models—Past, Present, and Future". ACM Trans. Graph. 39, 5, Article 157 (Jun 2020)

[62]	M. Zollhöfer, J. Thies, P. Garrido, D. Bradley, T. Beeler, P. Pérez, M. Stamminger, M. Nießner, and C. Theobalt. "State of the Art on Monocular 3D Face Reconstruction, Tracking, and Applications". Computer Graphics Forum (2018).

[x]	3GPP TS 28.310: " Management and orchestration; Energy efficiency of 5G ".

Next change

End of changes