3GPP TSG SA WG 1 Meeting #102 	S1-231742

Berlin, Germany, 22 – 26 May 2023	(revision of S1-231689, S1-231423, S1-231242)

Source:	Apple

pCR Title:	Pseudo-CR on Sensing Signals Terminology Update

Draft Spec:	3GPP TR 22.837 v0.4.0

Agenda item:	7.1

Document for:	Approval

Contact:	Mona Mustapha

Abstract: Proposes to define the term 'sensing signals'

1. Introduction

In discussion paper S1-230297 submitted at SA1#101 in Athens, Apple raised a question about the term 'sensing signals' and that this has not been defined in TR 22.837 but that it is used extensively in the use cases and in the agreed terms 'sensing receiver' and 'sensing transmitter'.

2. Reason for Change

To ensure clarity in the terminology, it is proposed to define the term 'sensing signals'.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.4.0.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3GPP sensing data: Data derived from 3GPP radio signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes, and optionally processed within the 5G system.

5G Wireless sensing: 5GS feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g. shape, size, orientation, speed, location, distances or relative motion between objects, etc) using NR RF signals and, in some cases, previously defined information available in EPC and/or E-UTRA.

Motion rate accuracy describes the closeness of the measured magnitude of the target object’s vibration frequency caused by part(s) of the target object to the true magnitude of the target object’s vibration frequency.

Editor’s Note: Terminology of motion rate accuracy needs to be refined.

Non-3GPP sensing data: Data provided by non-3GPP sensors (e.g. video, LiDAR, sonar) about an object or environment of interest for sensing purposes.

Sensing group: a set of sensing transmitters and sensing receivers whose location is known and whose sensing measurement data can be collected synchronously.

Sensing measurement process: process of collecting sensing measurement data.

Sensing receiver: A Sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is an NR RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

Sensing result: processed 3GPP sensing data requested by a service consumer.

Sensing service area location:  an area location whether with or without obstacle, the 5G system can provide sensing service with certain quality.

Sensing signals: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

NOTE:	The definition refers to NR RF signals and, in some cases, previously defined information available in EPC and/or E-UTRA can be used, without leading to impacts on EPC and E-UTRA.

Sensing target area: an area that needs to be sensed by deriving the dynamic characteristics of the area from any moving obstacles (e.g. cars, human, animals) from the impacted (e.g. reflected, refracted, diffracted) wireless signals. There are 2 kinds of target area:

- Static sensing target area: a pre-defined area that does not move from the sensing transmitter’s perspective.

- Moving sensing target area: a trusted zone with a target that moves from the sensing transmitter’s perspective.

Sensing transmitter: A Sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is an NR RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5G system, e.g. the data is communicated using a standard protocol to an interface defined by the 5G system.

The following KPIs apply to the definition of the use cases on sensing qualitative requirements:

-	Sensing service area: a service area where sensing services would solely rely on infrastructures and sensing technologies that can be assumed to be present anywhere where 5G is present. This includes both indoor and outdoor environments.

The following KPIs apply to the definition of the use cases on sensing quantitative requirements:

-	Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e. position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e. velocity) of the target object’s velocity to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g. range, velocity) to be allowed to detect objects in different magnitude.

-	Missed detection probability denotes the ratio of missing event to acquire a sensing result over all events during any predetermined period when the 5G system attempts to acquire a sensing result. It applies only to binary sensing results.

-	False alarm probability denotes the ratio of detecting an event that does not represent the characteristics of a target object or environment over all events during any predetermined period when the 5G system attempts to acquire a sensing result. It applies only to binary sensing results.

-	Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

-	Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results reporting to the application server.