3GPP TSG-SA1 Meeting #104	S1-233174

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.011 | CR | 0356 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Connected mode PLMN search as solution to many Dual Steer use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY (Dual Steer) |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 22.841, SA1 has studied may use cases relating to a UE using different PLMNs and/or Access Technologies. Many of these use cases can be solved using existing standards for intra or inter-PLMN handover, or, by network reselection. However, for operational reasons inter-PLMN handover is widely not implemented. PLMN reselection frequently takes a considerable time, in part, because the existing specification for PLMN search prohibits such searches during RRC-Connected mode. This ban is a relic of GSM phase 1 where a connected mode UE needed all of its time to transmit, recover from high power transmission, change frequency, measure a neighbour cell, change frequency and receive. However, modern UEs (e.g. that support Carrier Aggregation or Dual Connectivity) frequently have multiple receivers that can – optionally – be used to search for (and then maintain knowledge of) other PLMNs during RRC Connected state. Then, if the UE looses coverage of the selected PLMN, the UE can switch to one of these other PLMNs, camp on a cell, and establish an RRC Connection, Attach and activate a PDN connection within a few hundred milli seconds. Thus by removing the ban on connected mode PLMN search, many of TR 22.841’s use cases can be delivered with no RAN or CN impact. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ban on connected mode PLMN search is removed. Optional connected mode search is allowed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessarily long outages at PLMN change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 3.2.2.5	Periodic network selection attempts

If the Operator controlled signal threshold per access technology is not set on the USIM, a UE in Automatic Mode shall make periodic attempts to look for a higher priority PLMN including associated Access Technology of the same country as the currently received PLMN including associated Access Technology or for a higher priority PLMN including associated Access Technology that uses a Shared MCC (e.g. MCC=901). If the currently received PLMN including associated Access Technology uses a Shared MCC, also a higher priority PLMN including associated Access Technology using any non-Shared MCC shall be considered. For the ranking of PLMNs the UE shall use the order used in clause 3.2.2.2. In the case that there is no associated Access Technology identifier, the mobile shall assume that all Access Technologies provided by a PLMN are of equal priority. Moreover, periodic network selection shall not lead to change of access technology within the registered PLMN.

When a Disaster Condition is applicable, a UE in Automatic Mode that is registered to a PLMN in the "Forbidden PLMN" data field in the SIM/USIM may make periodic attempts to look for an allowable PLMN of the same country as the currently received PLMN.

In the case that the UE has stored a list of equivalent PLMNs, the UE shall only select a PLMN if it has a higher priority than all the PLMNs, in the list of equivalent PLMNs, which are of the same country as the currently registered PLMN.

NOTE 1: 	In the context of this 3GPP TS, the term country is to be interpreted not as a political entity but as a single Mobile Country Code (MCC). For example the USA and India have multiple MCC. Such cases are in fact treated as exceptions in the 3GPP specifications. For all other countries, multiple MCCs may be used, however the specifications have not taken this into account and there could be adverse effects such as the UE being unable to detect that multiple MCCs are within the same country.

NOTE 2:	There are some situations where one MCC represents multiple political entities or where an MCC is independent of political entities. Examples include Shared MCCs, where international satellite operations typically use the Shared MCC with value 901.

NOTE 3:	For the purposes of PLMN selection, the associated Access Technology using a Shared MCC is restricted to the satellite NG-RAN.

In the case that there are multiple EHPLMNs available, the UE shall not attempt to select a higher priority EHPLMN when an EHPLMN has already been selected. The priorities of EHPLMNs are only applicable when the UE is on a VPLMN and multiple EHPLMNs are available.

The UE shall make reselection attempts while in idle mode. In case of GPRS terminals, the UE shall make reselection attempts while in Idle or Stand-by mode. In case of terminals operating E-UTRA and/or NR, the UE shall make reselection attempts while in Idle or RRC-Inactive mode.

In order to minimise the time to reselect a PLMN after loosing coverage of the selected PLMN, the UE may at any time search for other PLMNs and maintain knowledge of the channels appropriate to attempt access to them.

NOTE 3a:	This may be appropriate for a UE with multiple receivers that is in E-UTRA/NR RRC Connected state. The UE implementation is not intended to be specified, but ought to be aware that many operators use network sharing or base station site sharing (especially in rural areas near the edge of coverage) and hence coverage might be lost on multiple operators at nearly the same time.

In the case of a UE receiving an eMBMS transport service when in idle mode, the UE may postpone the higher priority PLMN search and any reselection attempt until the eMBMS transport service has finished or been stopped.

The interval between attempts shall be stored in the SIM/USIM.

Only the service provider shall be able to select for which of the previous situations, periodic network selection shall be attempted and to set the interval value.

For UEs supporting only Access Technologies other than the following: NB-IoT, GERAN EC-GSM-IoT and Category M1 [17] of E-UTRAN enhanced-MTC the UE shall interpret the interval value to be between 6 minutes and 8 hours, with a step size of 6 minutes.

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

For UEs supporting a combination of IoT and non-IoT Access Technologies the UE shall interpret the interval value to determine the timer value as above based on the Access Technology currently in use by the UE.

One interval value shall be designated to indicate that no periodic attempts shall be made.

In the absence of a permitted value in the SIM/USIM, or the SIM/USIM is phase 1 and therefore does not contain the datafield, then a default value of 60 minutes, shall be used by the UE except for those UEs only supporting any of the following, or a combination of: NB-IoT, GERAN EC-GSM-IoT [18], and Category M1 [17] of E-UTRAN enhanced-MTC. For those UEs a default value of 72 hours shall be used.

NOTE 4:	Use of values less than 60 minutes may result in excessive UE battery drain.

Periodic network selection based on Operator controlled signal threshold criteria

If the Operator controlled signal threshold per access technology is set on the USIM, the UE shall perform the periodic network selection procedure considering all PLMNs, of higher or lower priority than the current PLMN, in the same order as in clause 3.2.2.2, with the following conditions:

-	the UE shall select a higher priority PLMN/access technology combination if the target PLMN’s access technology signal quality is equal to or higher than the Operator controlled signal threshold per access technology;

-	the UE shall select a lower priority PLMN/access technology combination only if the RPLMN’s access technology signal quality is lower than the Operator controlled signal threshold per access technology, and the target PLMN’s access technology signal quality is equal or higher than the Operator controlled signal threshold per access technology.

NOTE 5:	The UE does not perform the periodic attempts if the RPLMN is a HPLMN or EHPLMN, and the RPLMN’s access technology signal quality is equal or higher than the Operator controlled signal threshold per access technology.

If no PLMN, including the RPLMN, fulfils the Operator controlled signal threshold criteria, the UE shall perform the normal periodic network selection for higher priority PLMNs (as described above), without applying the Operator controlled signal threshold per access technology.

*************** end of changes ***********************************************************