3GPP TSG- SA1 Meeting # 105	S1-240258

Athens, Greece, 26 Feb - 1 March 2024	(revision of S1-240235, S1-240210, 1-240052)

# Source:	Novamint, Thales, Airbus, Eutelsat Group, Fraunhofer IIS, TNO, ESA, SES, ETRI, vivo, SKY Perfect JSAT, Sateliot, Lockheed Martin, Hughes Network systems, CATT, Nokia, Nokia Shanghai Bell, OQ Technology, China Telecom, Firstnet, Honor, NEC, Google Inc., IRT Saint Exupery, AT&T, ROBERT BOSCH GmbH

# Title:	New SID: Study on satellite access - Phase 4

# Document for:	Approval

Agenda Item:	7

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on satellite access - Phase 4

Acronym: FS_5GSAT_Ph4

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770002 | Study on using Satellite Access in 5G | Previous Rel-16 work covering satellite access in 5GS related use cases and requirements (ref. TR22.822: FS_5GSAT) |
| 790001 | New Services and Markets Technology Enablers– Phase 2 | Stage 1 work item of 5G system. |
| 720005 | New Services and Markets Technology Enablers | Stage 1 work item of 5G system |
| 800010 | Integration of Satellite Access in 5G | Rel-18 Stage-1 Normative Work on the support of satellite |
| 860005 | Integration of satellite systems in the 5G architecture (5GSAT_ARCH) | Rel-18 Stage-2 Normative Work on the support of satellite |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS (IoT_SAT_ARCH_EPS) | The work will take into account stage-2 Normative Work on the support of NB-IoT NTN in EPS |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks (LTE_NBIOT_eMTC_NTN) | The work will take into account stage-2 Normative Work on the support of NB-IoT NTN in EPS |
| 960016 | Study on satellite access - Phase 3 | Rel-19 stage-1 study on the support of satellite access phase 3 (ref. TR22.865 FS_5GSAT_Ph3) |
| 1000024 | Satellite access - Phase 3 | Rel-19 stage-1 Normative Work on the support of satellite |
| 1010033 | Study on Integration of satellite components in the 5G architecture Phase 3 | Rel-19 stage-2 study on the support of satellite access phase 3 (FS_5GSAT_Ph3_ARCH) |

# 3	Justification

New capabilities for satellite access such as Store and Forward Satellite operation and UE-Satellite-UE communication have been addressed in SA1 5GSat phase 3 for Release 19. First commercial deployments of 5G based satellite are happening and many new use cases for satellite access emerging are to be addressed still in the context of 5G advanced (i.e. in Release 20).

These use cases and capabilities to be addressed in Rel-20 are the following:

- Enhancements of existing use cases:

1/ Enhanced support of Multicast and Broadcast Services (MBS) over Satellite

Broadcast services constitute more than 50% of the revenue of satellite operators today based on classical DVB systems. A migration to 3GPP based technology shall enable MBS services to be available to allow a smooth evolution of the satellite operator’s broadcast services. MBS has the potential to complement the terrestrial and satellite networks with an efficient means to multicast and broadcast content to the end users. which can provide substantial improvements, especially regarding efficient resource usage to distribute content, such as software updates, media content for example. It can complement as well unidirectional link services. MBS is already supported in the 5G system However, several use cases when the satellite acts as the access network (e.g., Broadcast Live TV Distribution, Reliable multicast, Emergency messaging broadcast) and the impacts on the service requirements have not been addressed yet.

2/ Enhanced support for emergency calls/messaging and mission critical services via satellite

With the first commercial deployments of 5G based satellite, there is a strong demand for supporting 5G emergency communication and mission critical services via satellite (e.g., emergency call for automotive, emergency messaging for handsets or IoT devices, PWS). Given that the regulation is evolving at least with higher location accuracy requirements; there is therefore a need for enhanced support of emergency calls/messaging services and mission critical service by satellite access.

3/ Resilient UE-Satellite-UE communication with intermittent ground connectivity

There is a need to support more robust system resilience to enhance UE-Satellite- UE communication with intermittent ground connectivity for example in case of ground disaster or ground connection failure (ISL outage or feeder link failure).

4) Support of IMS-based GEO satellite call service

The newly launched mobile phones with satellite connections are becoming increasingly popular in the market. Considering enabling it as a basic mobile operator service , GEO (Geostationary Earth Orbit) satellites offer a unique advantage in terms of global coverage. IMS service is firstly proposed in 3GPP as an access-agnostic service and in the 5G system it was by default supported no matter what the access technology is. However, considering the limitations of GEO’s transmission rate using narrow band satellite access technology, and large propagation delay to support the IMS service, it is necessary to address the gaps between IMS service and GEO as satellite access. In addition, the regulatory aspect of utilizing IMS-based voice calls over GEO satellite for emergency service.

- New use cases:

5/ Support for multi-orbits satellite access networks

Several satellite operators are expected to deploy multi-orbits satellite networks combining LEO, MEO and GEO with 5G systems in the coming years. To take advantage of different orbit satellite systems, it is necessary to investigate new use cases for different services to enhance the user’s service experience considering a same satellite operator manages those different orbit networks (LEO, MEO, GEO).

6/ Robust notification alert to increase reachability via satellite access

There are several use cases where terrestrial networks are not available and users are relying solely on satellite to provide communication and are expecting to be reachable via satellite. However, when the UE is placed in pockets, backpacks, in vehicles, boats… it results in poor reception conditions which is leading to miss calls and messages. A robust notification alert will allow to address this problem of unreachability to UE in those conditions (e.g., low SNR conditions, NLOS channel conditions).

In addition to the service requirements, KPIs for a 5G system with satellite access have been introduced in TS 22.261 (section 7.4) in the context of Rel-16. Since the KPIs for satellite access have not been updated and do not reflect KPIs for 5G Advanced for the satellite access. This is especially valid for service to vehicular/drone mounted devices for verticals as well as for IoT devices. It is important to review the KPIs of 5G satellite networks and potentially update them. Furthermore, there is a need to clarify KPIs associated to service continuity.

# 4	Objective

The objectives of this study are the following:

-	to identify the gaps and potential new service requirements as well as related potential regulatory requirements if applicable for 5G system over satellite, for the following use cases:

- Broadcast Live TV Distribution, reliable multicast, Emergency messaging broadcast use cases over satellite

- Emergency communications, PWS and mission critical services via satellite

- Resilient UE-Satellite-UE communication with intermittent ground connectivity in case of ground disaster or ground connection failure (ISL outage or feeder link failure).

-	to study new use cases and identify new potential requirements for 5G system over satellite, including:

- to support multi-orbits satellite networks (LEO, MEO, GEO) for multiple services;

- to be able to notify the user that an incoming call had failed when the UE was unreachable in satellite access

-	to study IMS voice call service under GEO satellite access

- to identify the gaps to support IMS voice call service under GEO satellite access network conditions and to define corresponding potential service requirements to support IMS-based voice calls over GEO satellite

- to identify the regulatory aspect of utilizing IMS-based voice calls over GEO satellite for emergency service

-	to identify the gap and potential updates of the related KPIs of 5G Advanced satellite networks

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.xxx | Study on satellite access - Phase 4 | TSG#106 | TSG#107 | Thierry Bérisot, Novamint, (tberisot@novamint.com) Co-rapporteur Xu Xia, China Telecom (xiaxu@chinatelecom.cn) |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Rapporteur: Thierry Bérisot, Novamint, tberisot@novamint.com
Co-rapporteur: Xu Xia, China Telecom, xiaxu@chinatelecom.cn

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Novamint

Thales

Airbus

Eutelsat Group

Fraunhofer IIS

TNO

SES

ESA

ETRI

Vivo

SKY Perfect JSAT Corporation

Sateliot

Lockheed Martin

Hughes Network systems

CATT

Nokia

Nokia Shanghai Bell

OQ Technology

China Telecom

Firstnet

Honor

NEC

Google Inc.

IRT Saint Exupery

AT&T

ROBERT BOSCH GmbH

Erillisverkot