3GPP TSG SA WG 1 Meeting #109 	S1-250033

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	Qualcomm

pCR Title:	Pseudo-CR on Support of legacy services and requirements

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Francesco Pica
fpica <at> qti.qualcomm.com

Abstract: This document provides a text proposal for capturing migration requirements related to support of legacy services and other requirements.

1. Introduction

TS 22.261 includes the following 5GS requirements related to Legacy service support:

===============================

### 5.1.1 	Description

The 5G system supports most of the existing EPS services, in addition to many new services. The existing EPS services can be accessed using the new 5G access technologies even where the EPS specifications might indicate E-UTRA(N) only. Only new or changed service requirements for new or changed services are specified in this TS. The few EPS capabilities that are not supported by the 5G system are identified in clause 5.1.2.2 below.

### 5.1.2	Requirements

#### 5.1.2.2	Legacy service support

In principle, the 5G system shall support all EPS capabilities (e.g. from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173, 22.468). However,

-	voice service continuity from NG-RAN to GERAN shall not be supported,

-	voice service continuity from NG-RAN to UTRAN CS should be supported (see Note),

-	voice service continuity from GERAN to NG-RAN shall not be supported,

-	voice service continuity from UTRAN to NG-RAN shall not be supported,

-	CS fallback from NG-RAN to GERAN shall not be supported,

-	CS fallback from NG-RAN to UTRAN shall not be supported,

-	seamless handover between NG-RAN and GERAN shall not be supported,

-	seamless handover between NG-RAN and UTRAN shall not be supported,

-	access to a 5G core network via GERAN or UTRAN shall not be supported,

-	video service continuity between 5GS and UMTS shall not be supported,

-	IP address preservation for PS service when UE moves between 5GS and GSM/UMTS shall not be supported, and

-	Service continuity between 5GS and CDMA2000 shall not be supported.

NOTE:	Architectural or protocol changes needed to support voice service continuity from NG-RAN to UTRAN CS are expected to have minimum impact on architecture, specifications, or the development of the 5G New Core and New Radio.

===============================

A similar approach can be used for 6G, i.e. most requirements defined for 5G can be automatically “inherited” by 6G, with some exception.

In addition to the above general approach, explicit requirements can be added for supporting key telecommunication services and/or services subject to regulatory requirements, based on MNO interest, for example MMTEL, Emergency services, SMS, PWS, MPS.

Specific consideration should be given to 5GS requirements including other 3GPP systems/RATs (beyond the explicit exclusion list above), e.g. whether requirements involving 5G and 4G should apply automatically to 6G and 5G, etc. For those inter-RAT requirements, it would be preferred to explicitly indicate 6G support, or lack of it (where applicable).

Based on the above, it is proposed to describe three different type of legacy support requirements, related to a) general services, b) other requirements and c) interworking with legacy systems.

This contribution covers a) and b).

Interworking with legacy systems and other inter-RAT requirements are covered in separate contributions.

2. Reason for Change

It is necessary to define 6G requirements related to support of legacy services and functionalities.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

# 5	System and Operational Aspects

## 5.1	Support of legacy service requirements

### 5.1.1 	Introduction

The 6G system is assumed to support most of the existing 5GS service requirements (e.g. from TS 22.261, …), plus new requirements.

The existing service requirements defined in 5G specifications and referring to 5G only. i.e. not including other 3GPP systems/RATs, can be applied to 6G by implicitly replacing the terms 5G, 5GS, 5GC, NR or NG-RAN, with 6G, 6GS, 6G CN, 6G radio or 6G RAN, respectively.

Note: the above 6G terms should not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN. Detailed definition is FFS.

5G requirements involving multiple 3GPP RATs (e.g. 5G and 4G) shall be explicitly defined for 6G (either supported or not supported). Existing 5GS requirements related to non-3GPP access, as well as Satellite access, are assumed to be implicitly supported by 6G, unless stated otherwise.

The following sections cover two parts: basic services (supported or not supported) and other 5GS requirements (implicit support of legacy requirements unless explicitly excluded).

### 5.1.2	Support of services

#### The 6G system shall be able to support the following services:

#### Editor’s note: add list of services (with ref. SA1 TS) that should be supported, e.g., MMTEL, Emergency services, SMS, PWS, MPS, etc.

#### The following services do not need to be supported by the 6G system:

#### Editor’s note: add list of services (and ref SA1 TS) not to be supported.

### 5.1.3	Support of other legacy requirements

The 6G system shall be able to support all 5G system requirements and capabilities, except the following:

-	voice service continuity from 6G RAN to UTRAN CS

Editor’s note: further exceptions are FFS, e.g. about mobility and interoperability between 6G and 4G, or other 5GS requirements (including those inherited from 4G).

Note: the above requirement refers to existing requirements defined for 5G only, i.e. not including other legacy 3GPP systems or RATs, unless explicitly indicated (in existing SA1 specifications and/or in this report).

Note: other legacy capabilities excluded for 5G (i.e. listed in TS 22.261, sec. 5.1.2.2) are assumed not to be supported for 6G as well.