3GPP TSG SA WG 1 Meeting #109 	S1-250108

Athens, Greece, 17-21 February 2025

Source:	SKY Perfect JSAT Corporation

pCR Title:	Pseudo-CR on Continuing Support for 5G NTN Features in 6G Networks.

Draft Spec:	3GPP TR 22.870 V0.1.1

Agenda item:	8.1.1 FS_6G-REQ – System and Operation Aspects

Document for:	Approval

Contact:	Atsumu Mishima <mishima-atsumu@sptvjsat.com>

Abstract: This document provides a new use case on continuing Support for 5G NTN Features in 6G Networks.

1. Introduction

This contribution proposes a new use case on Continuing Support for 5G NTN Features in 6G Networks.

2. Reason for Change

TR 22.870 v0.1.1 does not yet contain use case that referring to continued support for NTN functions specified in 5G.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.1.1.

* * * First Change * * * *

5.x	Use case on Continuing Support for 5G NTN Features in 6G Networks

5.x.1	Description

The satellite communication market is rapidly expanding and is expected to become a critical infrastructure by 2030 and beyond, given its anticipated applications in diverse fields such as IoT, remote area connectivity, and military and defence purposes.

Supporting Non-Terrestrial Networks (NTN) will be indispensable for 6G. Satellites, in particular, which provide wide-area coverage without relying on terrestrial infrastructure, play a crucial role in complementing ground networks by ensuring communication in unconnected regions or during disasters. They also contribute to building resilient industrial infrastructure and bridging the global digital divide.

In general, satellite operations are designed for long-term use. Specifically:

- LEO satellites are expected to have a lifespan of over 5 years, with advancements in technology enabling operations exceeding 10 years.

- GEO satellites typically have a lifespan of over 15 years, with many cases of operation extending beyond their designed lifespan.

The long lifecycle of satellites contributes to the long-term stability of communication networks. Considering this extended operational potential, it is essential for the 6G system to continue supporting the functions of Non-Terrestrial Networks (NTN), which were standardized in the 5G. This long-term characteristic serves as a foundation for the continued utilization of NTN technologies in 6G.

Given the longevity of satellites and the effectiveness of NTN technology established under 5G, it is both rational and essential to support these functions in 6G as well. This ensures the development of a sustainable communication infrastructure.

Additionally, commercial deployment of 5G NTN is still in its early stages, with many operators expected to launch services in the latter half of the 2020s. Considering this timeline, the period allocated for standardizing and developing 5G NTN has been short, making it challenging for NTN specifications and service models to fully mature within 5G. This lack of maturity could lead to missed business opportunities. However, replacing recently adopted 5G systems with 6G systems would be impractical for many satellite operators. Therefore, while aiming to enhance NTN capabilities and add essential functionalities under 6G, these should not be limited exclusively to new specifications for 6G RAN. Instead, it is necessary to build upon and integrate the existing foundation of 5G NTN systems.

5.x.2	Pre-conditions

- Satellite operator A (Sat A) implements 5G system with satellite access from 2027 to 2030.

- The 5G system utilized with communication satellite and ground equipment is to be deployed more than 10 years from the implementation.

Sat A may have the following challenges;

- Sat A tries to provide services including new features with as less investment as possible.

- Sat A tries to provide customers with as much features as possible without replacing their UEs.

5.x.3	Service Flows

1. 	Sat A will deploy 6G system in the early 2030s.

2.	Sat A will provide customers with not only services featured by 6G system but also exiting services featured by 5G system.

5.x.4	Post-conditions

Sat A can provide customers with not only services featured by 6G system but also exiting services featured by 5G system with streamlined systems.

5.x.5	Existing features partly or fully covering the use case functionality

TS 22.261 specify requirements for a 5G system with satellite access.

5.x.6	Potential New Requirements needed to support the use case

[PR 5.x-1] The 6G system with satellite access shall support all the features provided by 5G system with satellite access (e.g. VSAT UE capabilities, multi-orbit satellite access, and emergency communications) from day one.

[PR 5.x-2] The 6G system with satellite access shall strive to leverage the 5G system with satellite radio access, considered its commercial deployment.