3GPP TSG-SA WG1 Meeting #109 	S1-250124

Athens, Greece, 17-21 Feb. 2025

Title:	Proposal for supporting legacy service aspects

Agenda Item:	8.1.1 System and Operation Aspects

Source:	ZTE, China Telecom, China Mobile

Contact:	xu.ling@zte.com.cn, xiaxu@chinatelecom.cn,

Abstract: This contribution proposes supported and not supported list of legacy services and capabilities for 6G.

- Discussion:

During the SA1#108, 6G supporting of legacy services and capabilities was discussed and S1-244804 is the baseline to continue the study on this aspect.  In the S1-244804, supported list and not supported list are expected to be included in the clause.

To progress this study, following points are suggested to be considered and achieve consensus:

- The meaning of “supported list”: When we talk about the supporting legacy service, it doesn’t mean that the related 6G technologies supporting these legacy items will be restricted by the legacy architecture and technologies. And considering the 5G deployment experience, some of the legacy items could be improvement and enhancement by 6G further, e.g. simplify network slicing. So, a NOTE is suggested to be added in the description clause to state that “The items in supported list could be improved and enhanced by 6G, and the references for the items are not intended to restrict the implementation of 6G system”.

- The meaning of “not supported list”: That means some of the items don’t need to be supported, some of them could be supported by legacy system for a long time, e.g. the NBIoT could be supported by legacy system for a long time.

- The items not in the lists: Considering the development of the marketing and customer’s demands, some of the legacy services are not very sure to support or not in the first 6G release.  So, it is suggested to note that “The item, which is neither in the supported list, nor in the not supported list, means that it is suggested not to be considered in the first 6G release.”

- Item granularity: At least the basic IMT-2020(5G) usage scenarios, i.e. eMBB, URLLC, mMTC need to be included. Based on these, the additional items, e.g.  Message, voice, Roaming, Location, NTN etc. with the granularity of existed feature level. The detail could be found in the text below.

- Considering some of the legacy IoT device will be supported by legacy system for a very long time, and the legacy system and 6G will be coexisted for a long time, it is suggested to “support RedCap like or AmbientIoT, and not support NBIoT in the first release of 6G.”

- Connection with legacy network: Considering the experience of 5G SA/NSA, it is suggested to simplify the 6G interworking with legacy network, i.e. not support access 6GC via legacy RANs, nor access 5GC via 6G RAN.

- Proposal:

---------- Proposed changes ----------

5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

5.x	Support of legacy services and capabilities

### 5.x.1	Description

In addition to new services envisioned for the 6G system, there are existing services specified in 3GPP for previous 3GPP generations. It is envisioned that some of these existing services could be deemed as required for support in the 6G system, while others are not or at least not in the first Release of 6G.

NOTE 1:	The items in the supported list could be improved and enhanced by 6G, and the references for the items are not intended to restrict the implementation of 6G system

NOTE 2:	The item, which is neither in the supported list, nor in the not supported list, means that it is suggested not to be considered in the first release of 6G.

### 5.x.2	Potential Requirements

Editor's note: granularity and definition of all items listed in this clause is to be considered.

Editor's note: each item needs to have a reference e.g. to a specific TS and clause.

Editor’s note: following two options are presented in different forms in supported list and the same in not supported list. It is dependent on WG’s discussion to choose one of the forms.

Op1:

The 6G system shall support all 5GS services from e.g. TS22.261, TS22.104, TS22.011, TS22.173, TS22.071, TS22.262, TS22.278, TS22.185, TS22.186, TS22.125, TS22.263, TS22.115, TS22.101, TS22.153, TS22.268, TS 22.280; TS 22.179, TS22.281, TS22.282, TS22.289, TS22.468, TS22.368.

The following are not supported in the 6G system:

-	voice service continuity from 6G-RAN to GERAN,

-	voice service continuity from 6G-RAN to UTRAN CS,

-	voice service continuity from 6G-RAN to E-UTRAN

-	voice service continuity from GERAN to 6G-RAN,

-	voice service continuity from UTRAN to 6G-RAN,

-	voice service continuity from E-UTRAN to 6G-RAN,

-	CS fallback from 6G-RAN to GERAN,

-	CS fallback from 6G-RAN to UTRAN,

-	CS fallback from 6G-RAN to E-UTRAN,

-	seamless handover between 6G-RAN and GERAN,

-	seamless handover between 6G-RAN and UTRAN,

-	seamless handover between 6G-RAN and E-UTRAN,

-	access to a 6G core network via GERAN or UTRAN or E-UTRAN or NR-RAN,

-	access to a 5G core network via 6G-RAN,

-	video service continuity between 6GS and UMTS,

-	video service continuity between 6GS and EPS,

-	IP address preservation for PS service when UE moves between 6GS and GSM/UMTS/EPS.

Op2:

The 6G system shall support the following legacy services and capabilities:

-	eMBB and extended to XR, Tactile and multi-modality, Metaverse, refers TS22.261,TS22.156,

-	URLLC with 5GLAN, 5G Timing, NPN, Ethernet transport, supporting cyber-physical control in vertical, Critical Medical Applications, Smart Energy and Infrastructure, etc. refers TS22.261 and TS22.104,

-	mMTC, supporting Redcap-like and Ambient IoT services, refers TS22.261, TS22.104, TS22.368, TS22.369,

-	Message service refers TS22.262,

-	IMS related services refer TS22.261, TS22.173,

-	Roaming related services refer TS22.011, TS22.101, TS22.261,

-	Supporting Video, Imaging and Audio for Professional Applications refers TS22.263 and TS22.261,

-	Supporting UAV, refers TS22.125,

-	Supporting AI/ML applications, refers TS22.261,

-	Satellite access, refers TS22.261

-	Location, positioning, refer TS22.261 and TS22.071,

-	Sensing service refers TS22.137,

-	Emergency calls, refers TS22.101

-	PWS, refers TS22.268,

-	MPS, refers TS22.153.

The following are not supported in the 6G system:

-	voice service continuity from 6G-RAN to GERAN,

-	voice service continuity from 6G-RAN to UTRAN CS,

-	voice service continuity from 6G-RAN to E-UTRAN

-	voice service continuity from GERAN to 6G-RAN,

-	voice service continuity from UTRAN to 6G-RAN,

-	voice service continuity from E-UTRAN to 6G-RAN,

-	CS fallback from 6G-RAN to GERAN,

-	CS fallback from 6G-RAN to UTRAN,

-	CS fallback from 6G-RAN to E-UTRAN,

-	seamless handover between 6G-RAN and GERAN,

-	seamless handover between 6G-RAN and UTRAN,

-	seamless handover between 6G-RAN and E-UTRAN,

-	access to a 6G core network via GERAN or UTRAN or E-UTRAN or NR-RAN,

-	access to a 5G core network via 6G-RAN

-	video service continuity between 6GS and UMTS,

-	video service continuity between 6GS and EPS,

-	IP address preservation for PS service when UE moves between 6GS and GSM/UMTS/EPS.

Editor's note: further exceptions are FFS.