3GPP TSG-SA WG1 Meeting #109 	S1-250126

Athens, Greece, 17 - 21 February 2025

Title:	Use case on ubiquitous emergency rescue via UAVs

Agenda Item:	8.1.4

Source:	CSCN, ZTE Corporation

Contact:	    liqt@chinasatnet.com.cn, xu.ling@zte.com.cn; li.junli@zte.com.cn

Abstract: This contribution describes a new use case about the emergency rescue via multiple types of UAV through NTNs.

* * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5.1X]	QIAN LI et all, “6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies” in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[6.5x]	GSMA, "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[6.5y] 	3GPP TR 32.972: " Study on system and functional aspects of energy efficiency in 5G networks".

[6.5z]	3GPP TR 23.700-66

[7x]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7.2x]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[7.2y]	5GAA: Technical report “Evolution of vehicular communication systems beyond 5G”

[7.3x]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication",

[7.3y]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1"

[7.3z]	SAE J3016 202104 Levels of Driving Automation.

[7.4x]	TR 22.837, Feasibility Study on Integrated Sensing and Communication.

[7.4y]	G. Liu, L. Ma, Y. Xue, et al, “SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC,” IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[7.4z]	Yasin, Jawad N., et al. "Unmanned aerial vehicles (uavs): Collision avoidance systems and approaches." IEEE access 8 (2020): 105139-105155.

[7.5r1]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, “The Rise of Radar for Autonomous Vehicles”, IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[7.5r2] 	https://www.geospatialworld.net/prime/vodafone-country-scale-digital-twins-engineer-networks.

[7.5r3] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[7,6x]	3GPP TS 22.261: "Service requirements for the 5G system".

[9.1X] 	 Wang et.al, “Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality”, IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057

[9.2x1] 	https://en.wikipedia.org/wiki/Adreno

[9.2x2] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/

[9.2x3] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects)

[9.2x4] 	https://phlearn.com/tutorial/how-many-exposures-hdr/

[9.2x5]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw

[9.2x6]	https://www.dotphoton.com/products/jetraw-core

[9.2x7] 	https://www.lambdatest.com/learning-hub/web-performance-testing

[9.2x8] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing

[9.2x9] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[8.4X]	EUSPA, “Report on Aviation and Drones – User Needs and Requirements”, 2023.

[8.6x]	3GPP TR 36.763: “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

[9.0x]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[9.0y]	3GPP TS 22.261: " Service requirements for the 5G system; Stage 1".

[9.0z]	3GPP TS 22.156: "Mobile Metaverse Services".

[9.3X] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[9.3Y] 	3GPP TS 23.502: "Procedures for the 5G System".

[9.4x]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[11.1a]	https://www.easa.europa.eu/en/what-is-uam.

[11.1b]	https://www.autoflight.com/en/air/.

[11,1c]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[11.1d]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

11.1e]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/

[11.1f]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[11.1g]	https://www.flyeye.io/drone-acronym-daa/.

[W.2x]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf)

[8.xx] 	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION

* * * New use case * * * *

8.x	Use case on ubiquitous emergency rescue via UAVs

8.x.1	Description

When emergency situations (such as natural disasters or accidents) occurs, timely and effective rescue is crucial for saving lives and minimizing damage. Traditional rescue operations are affected by various factors, including the severity of a disaster, the availability of transport resources, and the damage to communication infrastructure. By utilizing unmanned aerial vehicles (UAVs) and other low-altitude aircraft supported by NTN, the ubiquitous emergency rescue can overcome limitations posed by adverse factors to minimize casualties and property losses.

The UAVs equipped with intelligent rescue planning system, high-resolution cameras, thermal imaging, and environmental sensors, could gather affected areas environment data in real-time, then deliver to the Emergency Response Agency (ERA). The ERA will design a rescue plan based on these information and dispatch UAVs embedded with 6G base station (e.g., UAV-based BS) to provide temporary communication for rescue team on site. Introducing low-altitude aircraft e.g. UAV, into emergency rescue can significantly enhance situational awareness, rescue resource management, ultimately leading to more effective and timely responses in crisis situations.

Refer the latest CHINA airspace classification [8.xx], the low altitude area is below 3000m, where the low-altitude aircraft could fly. The communication coverage for altitude less than 3000m is needed accordingly. However, in some area, e.g. forests, mountains, etc. the terrestrial network is not available which great impacts the rescue operation. The Non-Terrestrial Networks (NTNs), supported by Low Earth Orbit (LEO) satellite networks, High-Altitude Platform Stations (HAPS), multi-orbit satellites and other space-based infrastructure, could compensate the coverage blind spots and insufficient coverage of terrestrial network to improve communication service reliability.

6G base stations on board UAVs can work in collaboration with different types of High-Altitude Platform Stations (HAPS), multi-orbit satellites (LEO, MEO, GEO) and other space-based infrastructure can operate in collaboration to provide a robust coverage.

So, supported by 6G NTN and NTN collaboration with terrestrial network, UAVs for rescue operation could expanded to more location area and rescue the people who have lost the connection, in a timely and accurate manner.

Beside NTN based communication service, 6G NTN system will provide AI, sensing and computing capabilities in the operator managed Service Hosting Environment, which will help intelligent rescue planing system to process data more intelligent and faster, thus will improve rescue accuracy and efficiency.

8.x.2	Pre-conditions

ERA has already applied flight permission for its UAVs from local aviation authority. In order to carry out rescue operations effectively, the rescue teams are equipped with two kinds of UAVs, i.e.Type-1 UAVs (one kind of rescue UAV, equipped with UE and intelligent rescue planning system, high-resolution cameras, advanced sensors) and Type-2 UAVs( another kind of rescue UAV, embedded with 6G base station with communication and sensing capabilities and service host environment).

Operator B provide 6G TN and NTN service.  The NTN service is comprised of communication and sensing services.

The ERA has a contract with Operator B to achieve 6G TN and NTN services.

The rescue team is equipped with Augmented Reality (AR) devices.

8.x.3	Service Flows

Figure 8.x.3-1 Illustration of emergency rescue via UAVs supported by the NTN.

- When an earthquake occurs, the terrestrial network breaks down. ERA dispatches Type-1 UAV A11, A12 and Type-2 UAV X21 to the disaster area.

- With the help of 6G NTN network for C2 communication, UAV A11, A12 and X21 successful arrive the disaster area.

- The UAV X21 working as the temporary BS, provides 6G coverage for the ground rescue team and low altitude UAV A11, A12, with the assistance of 6G NTN network for the disaster area.

- Along with the ground rescue team moving and UAV A11, A12 flying, the UAV X21 could adjust its location to continue providing communication service.

- When the 6G NTN backhaul link quality for UAV X21 degrades, the 6G system utilizes a dynamic switching mechanism to seamlessly transfer UAV X21's communication connection to the nearest HAPS or LEO satellite, ensuring uninterrupted rescue task operation. The UAV A11 and A12 collect and deliver disaster area environment data (e.g. thermal imaging to locate survivors and environmental monitoring for hazardous conditions, etc.) to ERA through 6G NTN and 6G terrestrial network as the orange line showed in the figure. The data delivery need real time and higher data rate in uplink (e.g., 50Mbit/s) even in complicated geographical environments and long transmission distance.

- The UAV A11 and A12 can also deliver disaster area environment data (e.g. environmental monitoring for hazardous conditions, etc.) to UAV X21. The BS on boarding UAV X21 sense the environment conditions. The SHE on boarding the UAV X21 processes all the data and provides the sensing results, to the ERA.

- The 6G NTN also provide the sensing result of the disaster area environment to ERA.

- ERA receives the sensing result, analysis and identify survivor’s location, then direct the rescue team, UAV A11 and A12 to go to the location.

- The rescue team on-site with augmented reality (AR) glasses receives real-time data from UAV A11 and A12 through the communication link provided by UAV X21. These AR glasses can use image recognition technology to identify survivors' faces, heat sources, and dangerous areas, and overlay information in the field of view, providing visual decision support and helping them better understand the situation on site.

- Suddenly UAV A11 identifies the existence of survivor and forwards the message to rescue team via the BS on boarding UAV X21, as the red line showed in figure, which require high reliability and low latency transmission in this link.

- According to the information of survivor from UAV A11, the rescue team will carry out the rescue as soon as possible.

- Finally, all rescue tasks are successfully completed

8.x.4	Post-conditions

All the UAVs can be efficiently work for rescue through 6G NTN network. Survivor is rescued and treated.

8.x.5	Existing features partly or fully covering the use case functionality

In TS22.261, following NTN and sensing requirements have been specified.

The 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

The 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

The 5G system shall be able to support service continuity for mobile base station relays using at least one 3GPP satellite NG-RAN.

NOTE 7:	This requirement applies to scenarios where there is a transition between two 3GPP NG-RAN, operated by the same MNO, involving at least one 3GPP satellite NG-RAN.

The 6G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

8.x.6	Potential New Requirements needed to support the use case

[PR.8.x.6-1]  The 6G system with satellite access shall be able to support horizontal and vertical coverage for aircraft with variant altitudes (i.e.  from 0 to 3km).

[PR.8.x.6-2]  The 6G system with satellite access shall be able to support 6G base station on board UAV providing radio access for UEs.

[PR.8.x.6-3]  Subject to operator policy, agreement with the 3rd party and user’s consent, the 6G system shall  provide Service Hosting Environment on board UAV to process the data collected from 3GPP UEs.

[PR.8.x.6-4]  The 6G system with satellite access shall be able to support the collaboration of multi-orbit satellites (LEO, MEO, GEO), HAPS and 6G moving base station on board UAV including dynamic handover to enhance seamless network coverage.

[PR.8.x.6-5]  The 6G system with satellite access shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

[PR.8.x.6-6]  The 6G system shall be able to support mobile base station on board aircraft to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

[PR.8.x.6-7] The 6G system with satellite access shall support communication performance requirements in table 8.x.6-1

Table 8.x.6-1 Performance requirements for ubiquitous emergency rescue via UAVs

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Communication Latency | Reliability | Overall UE density | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ubiquitous emergency rescue type-1 UAV | TBD | [50]M bit/s | TBD | [500]G bit/s/km2 | [10]ms | [99.999]% | TBD | Pedestrians |  |

Editor’s note: the KPI for type-2 UAV is FFS.

* * * End of the First Change * * * *