3GPP TSG-SA WG1 Meeting #109 	   S1-250127

Athens, Greece, 17 - 21 February 2025

Title:	Use case on low-altitude logistics supported by the NTN

Agenda Item:	8.1.4

Source:	CSCN, ZTE Corporation

Contact:	    liqt@chinasatnet.com.cn, xu.ling@zte.com.cn,  zheng.shuang@zte.com.cn

Abstract: This contribution describes a new use case about the low-altitude logistics delivery via the UAVs supported by the NTN communication, sensing and data processing.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5.1X]	QIAN LI et all, “6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies” in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[6.5x]	GSMA, "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[6.5y] 	3GPP TR 32.972: " Study on system and functional aspects of energy efficiency in 5G networks".

[6.5z]	3GPP TR 23.700-66

[7x]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7.2x]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[7.2y]	5GAA: Technical report “Evolution of vehicular communication systems beyond 5G”

[7.3x]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication",

[7.3y]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1"

[7.3z]	SAE J3016 202104 Levels of Driving Automation.

[7.4x]	TR 22.837, Feasibility Study on Integrated Sensing and Communication.

[7.4y]	G. Liu, L. Ma, Y. Xue, et al, “SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC,” IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[7.4z]	Yasin, Jawad N., et al. "Unmanned aerial vehicles (uavs): Collision avoidance systems and approaches." IEEE access 8 (2020): 105139-105155.

[7.5r1]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, “The Rise of Radar for Autonomous Vehicles”, IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[7.5r2] 	https://www.geospatialworld.net/prime/vodafone-country-scale-digital-twins-engineer-networks.

[7.5r3] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[7,6x]	3GPP TS 22.261: "Service requirements for the 5G system".

[9.1X] 	 Wang et.al, “Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality”, IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057

[9.2x1] 	https://en.wikipedia.org/wiki/Adreno

[9.2x2] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/

[9.2x3] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects)

[9.2x4] 	https://phlearn.com/tutorial/how-many-exposures-hdr/

[9.2x5]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw

[9.2x6]	https://www.dotphoton.com/products/jetraw-core

[9.2x7] 	https://www.lambdatest.com/learning-hub/web-performance-testing

[9.2x8] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing

[9.2x9] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[8.4X]	EUSPA, “Report on Aviation and Drones – User Needs and Requirements”, 2023.

[8.6x]	3GPP TR 36.763: “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

[9.0x]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[9.0y]	3GPP TS 22.261: " Service requirements for the 5G system; Stage 1".

[9.0z]	3GPP TS 22.156: "Mobile Metaverse Services".

[9.3X] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[9.3Y] 	3GPP TS 23.502: "Procedures for the 5G System".

[9.4x]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[11.1a]	https://www.easa.europa.eu/en/what-is-uam.

[11.1b]	https://www.autoflight.com/en/air/.

[11,1c]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[11.1d]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

11.1e]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/

[11.1f]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[11.1g]	https://www.flyeye.io/drone-acronym-daa/.

[W.2x]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf)

[8.xx] 	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION

* * * New use case * * * *

8.x	Use case on low-altitude logistics supported by NTN

8.x.1	Description

For remote regions with inconvenient transportation, complex terrain and rapidly changing weather, traditional logistics delivery methods struggle to meet the needs of local residents. By utilizing unmanned aerial vehicles (UAVs) and other low-altitude aircraft, the low-altitude logistics can overcome limitations posed by terrain and other factors to swiftly deliver the medicine, food, and emergency supplies to the residents live in such remote region, which exhibits its tremendous potential and advantages.

The development and operation of the low-altitude economy heavily rely on robust communication networks to ensure the safety and reliability of UAV systems. However, depending solely on traditional terrestrial networks (such as 4G/5G cellular networks) presents several critical challenges, for example:

- The density of ground base station deployments is limited, particularly in remote areas or challenging terrains such as mountains or forests. This can lead to insufficient or unstable signal coverage, limiting the expansion of the low-altitude economy into these regions.

- UAVs often operate in environments with high mobility and dynamic changes. Ground networks, which are primarily designed to serve relatively stationary or slowly moving objects, will struggle to maintain stable connections and experience higher data transmission delays.

- The coverage height of ground base stations is limited. This can lead to the flight range of medium and large-size UAV being restricted, especially in long-distance, long-haul and heavy-load flight mission. Refer the latest CHINA airspace classification [8.xx], the low altitude area is below 3000m, where the low-altitude aircraft could fly

To address these challenges, the low-altitude economy need reliance on Non-Terrestrial Networks (NTNs), such as Low Earth Orbit (LEO) satellite networks, High-Altitude Platform Stations (HAPS), and other space-based or non-conventional communication infrastructures. NTN can offer broader and more reliable coverage, accommodate high-dynamics flight scenarios, and provide enhanced security and regulatory capabilities.

Additionally, in some challenging environment, such as steep mountains, deep valleys, and dense forests, it brings significant difficulties for UAV flight. The rapidly changing weather conditions in remote mountainous regions, such as sudden strong winds, rain, or fog, further complicate UAVs’ operations. And the UAVs that flights over different areas in a remote region are affected by different weather conditions as following Figure 1, e.g., some UAVs flight over the area 1 are affected by the heavy rain, while the UAVs flights over the area 2 are affected by the dense fog. Under such scenarios, the NTN can provide satellite-based communication, for efficient data exchange between UAVs and leveraging advanced sensing and processing capabilities to assist optimizing flight paths and ensure task operation.

Figure 8.x.1-1. Illustration of logistics delivery in remote mountain areas.

Following is an example of service flow to describe the low-altitude logistics delivery via the UAVs supported by the NTN networks.

8.x.2	Pre-conditions

To improve the delivery efficiency, the logistics company A bought a large number of UAVs for the delivery, and it has a dispatch center that is responsible for scheduling and optimizing the flying route of UAVs.

The UAVs have been configured with the camera and sensor, which can be used to sense and collect the environmental data.

The UAVs of logistics company A have been configured with 6G UE that support communication via the 6G NTN and have already requires the flight permission from local aviation authority.

The logistics company A has received many delivery orders in a remote region A  from different customers, and one order is from the customer Alice to deliver the medicine to the Jack’ home, who also lives in this remote region.

The operator B has deployed 6G NTN network to provide the reliable 6G NTN service in this remote region A.

Besides the communication service, the 6G NTN is equipped with the service hosting environment, sensing and AI capabilities, which can sense and process the high-resolution image data, as well as performing the intelligent analysis of image data directly on the satellite.

The logistics company A has subscribed the 6G NTN service from operator B for its UAVs.

8.x.3	Service Flows

1. The logistics company A inputs all delivery tasks of this remote region into the dispatch center, different delivery tasks have different targeted locations in this region. Then the dispatch center selects some UAVs and distributes a suitable initial route and delivery task for each UAV taking into account environmental conditions. And Alice’s order will be delivered by the UAV ‘1-1’.

- The UAVs take their corresponding expresses and begin the delivery according to the planned initial route.

- The 6G NTN network provides coverage through satellites or high-altitude platform systems, ensuring network connectivity in this remote region. The communication between UAVs and between the UAVs and the dispatch center are supported by the NTN.

- During the delivery, UAVs that configured with camera and sensor will continuously sense and collect the condition of the airspace traffic, and the environmental condition.  Multiple UAVs can share flight status, environmental sensing data, and mission progress information through 6G NTN.

- During the flight, the collected data can also be transmitted from UAVs to the 6G NTN. And combined with the collected data that sensed by 6G NTN itself, the 6G NTN processes the collected data and detects that there exists a steep cliff ahead of UAV’1-1’, which will pose threat to UAV ‘1-1’ flight. Thus the NTN sends alert to the UAV ‘1-1’, the UAV ‘1-1’ autonomously adapts its flight action to avoid collision, ensuring safe navigation.

- Additionally, the 6G NTN can also monitor the real-time weather condition. Through on board sensing and processing, the NTN gathers the weather data in area A and detects that there will have a strong winds ahead in area A. Then NTN  immediately sends the weather status to the dispatch center. The dispatch center can dynamically assess whether to update the UAVs’ 1-1 route.

- Finally, all the UAVs safely arrives at the targeted location as scheduled, and Jack gets the delivered medicine from UAV ‘1-1.

8.x.4	Post-conditions

The UAVs can dynamically update their flight routes and successfully finish their own delivery task in such harsh mountainous region through the support of NTN network.

The logistics company A can finish all delivery orders from all customers securely and efficiently.

8.x.5	Existing features partly or fully covering the use case functionality

SA1 has identified a series of performance requirement related to the UAVs in the section 7 of 3GPP TS 22.125 V19.2.0.

However, the existing performance requirement cannot support the communication performance requirement for the UAVs in low altitude area (below 3000m) and supported by the satellite access.

8.x.6	Potential New Requirements needed to support the use case

[PR.8.x.6-1] The 6G network shall be able to support detect and tracing sensing object (e.g. UAVs and its flying environment) while 6G sensing transmitter and/or 6G sensing receiver is/are on board satellite.

[PR.8.x.6-2] The 6G network shall be able to support sensing data processing on board satellite.

Note:	The sensing data can be 3GPP sensing data or non-3GPP sensing data.

[PR.8.x.6-3] Subject to operator’s policy and user’s consent, the 6G network shall be able to expose the sensing results from the on board satellite to a trusted 3rd party.

[PR. 8.x.6-4] Subject to operator’s policy and user’s content, the 6G system shall support the communication between UAVs via satellite without going through the ground network.

[PR.8.x.6-5] The 6G system with satellite access shall support communication performance requirements in table 8.x.6-1.

Table 8.x.6-1 Performance requirements for low-altitude logistics

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Latency | reliability | Altitude AGL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low-altitude logistics in remote mountain area | [50]Mbps | [100]Mbps | TBD | TBD | TBD | 99.999% | Up to [3000]m note 1 |
| Note 1: refer [8.xx] |  |  |  |  |  |  |  |

[PR.8.X.6-6] The 6G System with satellite access shall support the transmission of sensing result information to the trusted third party with the following KPI requirements.

|  | Altitude AGL | Typical message Size | Latency | Reliability |
| --- | --- | --- | --- | --- |
| Sensing result to a 3rd party | up to [3000] m | [TBD] byte | [TBD] ms | [99.999]% |

* * * End of the First Change * * * *