3GPP TSG SA WG 1 Meeting #109 	S1-250128

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	NTT DOCOMO

pCR Title:	Pseudo-CR on Dual Registration

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	azem@docomolab-euro.com

Abstract: <provide a short description of the content>

1. Introduction

pCR on dual registration for 6G system.

2. Reason for Change

This pCR introduce requirements on dual registration for 6G system to support the separation of 6GC and 5GC networks. Such complete separation of generations, would help to enhance system efficiency, ensure a more simplified approach of network migration, and minimizes the impact when a failure occurs.

3. Conclusions

Define the requirements for 6G system to support dual registration.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.1.1.

* * * First Change * * * *

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.X	Use Case X: Dual Registration for 6G network

### 5.x.1	Description

6G system aims to meet the growing demand in the consumer market by enhancing the system performance and user experiences in more efficient way. By introducing an infrastructure built specifically for 6G services, the full 6G feature set will become available with the promised 6G high performance (which might not be possible in case of a hybrid network) allowing the operators to offer/provide all the intended benefits of 6G to their users. Further, the separate 6G core network avoids dependency between the networks at migration.

Since the 6G network is separate and independent to the legacy 5G network, the UE will have to register individually and independently via simultaneous connections to the 6GC and 5GC networks, which imply the need of supporting Dual Registration.  In order to enable an MNO to control a UE being served by both networks; Dual Registration should use same subscriber identities when registering to the 5GC and 6GC.

Dual registration is a key feature that provides a pathway for the migration of existing 5G networks and 6G enabling the UE to access both networks simultaneously (via both 5G and 6G RATs) and maintaining service continuity with minimum service interruption for the subscribers in two 3GPP networks, as the UE can use both 6G and 5G access networks simultaneously for communication.

### 5.x.2	Existing features partly or fully covering the use cases functionality

None.

### 5.x.3	Potential New Requirements needed to support the use case

[PR 5.x.3-001] 	A 3GPP system shall be able to support UE dual registration in 6GC and 5GC network using same subscriber identifier.

[PR 5.x.3-002] 	A 3GPP system shall be able to support shared subscription information between 5G and 6G networks.

[PR 5.x.3-003] 	The 3GPP system shall be able to maintain each registration independently.

[PR 5.x.3-004] 	The UE shall support dual registration in 6GC and 5GC network using same subscriber identifier and be able to maintain each registration independently.

[PR 5.x.3-005]	For using a certain service/application, the network operator shall be able to provide a mechanism enables the UE to decide either to use 6G or/and 5G network for that service/application if both networks are available.

* * * End of Change * * * *