3GPP TSG SA WG 1 Meeting #109 	S1-250139

Athens, Greece, 17-21 February 2025

Source:	Nokia

pCR Title:	Pseudo-CR on updates of use case 5.2

Draft Spec:	3GPP TR 22.870-010

Agenda item:	8.1.1

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.2 in FS_6G-REQ TR.

1. Introduction

This contribution proposes to add some potential requirements in addition to clarifying service flows and existing features.

2. Reason for Change

In service flows, as inserting malicious messages cannot be avoided and NF is more related to Firewall, NF hardening etc., changes proposed on step 2.

For the references to existing features, TR 33.841 is an SA3 internal TR and 256-bit Confidentiality and Integrity Algorithms references need to be updated.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870-010.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 35.240: "Specification of the Snow 5G based 256-bits algorithm set: specification of the 256-NEA4 encryption, the 256-NIA4 integrity, and the 256-NCA4 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[y]	3GPP TS 35.243: "Specification of the AES based 256-bits algorithm set: Specification of the 256-NEA5 encryption, the 256-NIA5 integrity, and the 256-NCA5 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[z]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

* * * Second Change * * * *

## 5.2	Use case on quantum-resistant security

### 5.2.1	Description

According to some public predications, CRQC (cryptographically relevant quantum computer) may appear in 2030s.  However, quantum computing technology poses significant threats to some classical cryptography, especially to widely-used cryptographic algorithms (including both symmetric and asymmetric cryptography) that secure much of today's data, rendering electronic secrets vulnerable to discovery. Hence, the migration to quantum-resistant algorithms shall start as soon as possible.

The following figure lists the general application scenarios of cryptographic algorithms in 5G system.

Figure 5.2.1-1 Application scenario of cryptographic algorithms for 5G system

### 5.2.2	Pre-conditions

Alice bought a new cell phone with quantum-resistant capability and started to use the cell phone to connect to mobile networks. The network also turns on the quantum-resistant capability in order to protect users from quantum attacks. The quantum attacks as shown in Figure 5.2.2-1 will be blocked by the quantum-resistant approaches.

Figure 5.2.2-1 An example of quantum attacks to the UE and the mobile network

### 5.2.3	Service Flows

Assume that an attacker has the CRQC capability, the attacker can crack the traditional cryptographic algorithms based on the CRQC function and launch attacks to the network.

1.  Alice turns on the UE. The UE attaches to a base station, registers to the mobile networks, and establishes PDU session. Alice’s UE initiates some services and starts to transfer data between her phone and the network through the mobile network.

2.  An attacker is doing some malicious behaviours like eavesdropping and deciphering the traffic using CRQC, and blocking malicious messages, etc.

3.  As the UE privacy, communication for different elements (UE\NF) and service authorization are protected by quantum-resistant approaches, the attack initiated by the attacker fails.

### 5.2.4	Post-conditions

Thanks to the quantum-resistant approaches adopted by the UE and network, the interactions between UE and network, between network elements, and between different PLMNs is protected from attackers with CRQC.

### 5.2.5	Existing features partly or fully covering the use case functionality

3GPP specified 256-bit Confidentiality and Integrity Algorithms for the Air Interface in TS 35.240 [x], TS 35.243 [y], TS 35.246 [z] in Release 18. These 256-bit algorithms can be used to protect the RRC/NAS/UP traffic of the air interface from quantum attack. But how to use these algorithms (e.g., algorithm negotiation, key derivation) in 3GPP has not specified yet. And quantum-resistance is not fully considered in other scenarios like the identity protection, the NF-to-NF authentication and communication protection, the NF service authorization, inter-PLMN protection, etc.

### 5.2.6	Potential New Requirements needed to support the use case

[PR-5.2.6-1]	The 6G system shall provide security protection for communication including subscription identifier, authentication methods and protocols against the potential attacks posed by quantum computing.