3GPP TSG SA WG 1 Meeting #109 	S1-250145

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	Qualcomm

pCR Title:	Pseudo-CR Use case on Enhanced XR User Navigation

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This use case, an enhanced XR user navigation and environmental monitoring in a factory environment, introduces some UE-related sensing functional requirements and KPIs.

1. Introduction

Although some functional requirements were covered in Release 19 ISAC Work Item, some key functional requirement required for to support UE sensing capabilities.

2. Reason for Change

In this contributions, we bring a use case to demonstrate the need for functional requirements for UE sensing request and assistance info. from the network.

3. Conclusions

-

4. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870v0.1.1.

* * * First Change * * * *

## 7.x	Use case on Enhanced XR User Navigation

### 7.x.1	Description

While immersed in an XR application, a user cannot see the real environment or can only see the area covered by the camera or eyes' field of view. The XR application needs to ensure the user's safety by alerting them to avoid collisions with static and dynamic objects in the surroundings.

Current XR applications use visual detection, based on the device cameras, to estimate a map of the environment (static objects) and dynamically detect approaching objects, people, or animals, alerting the user in case of a possible collision.

The visual detection is limited to the cameras' or eyes' field of view (typically limited to the front area); hence, it would miss dynamic items that, for example, approach the user from the back. Moreover, the cameras' visual detection may have reduced performance in dark or overexposed lighting conditions; reflecting or transparent surfaces may also pose challenges to the visual detection, thus hindering user safety.

Wireless sensing can be used to provide additional detection, complementing the visual detection, and hence providing a more robust operation that is independent of the camera's field of view and lighting conditions.

### 7.x.2	Pre-conditions

A user is using an XR application, a VR training application in a factory workshop, with the user wearing an immersive Head Mounted Display (HMD) connected to a Non-Public Network which provides data connectivity and the Sensing Service.

The user’s surroundings are not physically bounded, hence e.g. Automated Guided Vehicles (AGV) in a factory. For this sensing service, it is expected that the sensing area around the user, and the detection should be continuous, highly reliable and low latency. Since the user/UE is the intended consumer of the Sensing Results, processing of the sensing data at the UE would ensure low latency for the sensing results.

### 7.x.3	Service Flows

- First the user enables the XR application, maps the area by using the HMD cameras and starts using the XR application. Then, the application invokes the sensing service to enable RF sensing for detecting moving objects in the user surroundings.

- Then, the operator confirms that this user is authorized for the sensing service, configures the sensing service and provides information (e.g. configurations and sensing assistance info. e.g. locations of the transmitter and receiver) to the UE.

- The UE then performs sensing operations, acting as both the sensing transmitter and receiver (i.e. monostatic sensing) or the UE may act as the sensing receiver by receiving sensing signals from surrounding sensing transmitters (i.e. bi-static sensing). In addition, the UE may also receive sensing data from other sensing receivers (i.e. UEs) in its surroundings.

- The UE processes the RF sensing data alone or jointly with the non-3GPP sensing data obtained from sensors in the HMD (e.g. visual detection data from cameras, accelerometer data, etc). During processing, the UE may leverage the sensing assistance info. provided by the network in deriving the sensing results.

- Also, the processing of the RF sensing data (and/or joint processing with non-3GPP sensing data) may leverage AI/ML algorithms for the detection. For example, the AI/ML model training may be performed offline in the network and the appropriate compressed model downloaded to the UE ahead of time. With the sensing data and AI/ML techniques, the sensing results are derived which in this case would be that the objects in the environment around the user are accurately detected even when out of view of the camera.

- An AGV approaching the user from the back is detected by  the sensing service and notifies the XR application, which alerts the user to take the appropriate safety actions.

### 7.x.4	Post-conditions

Thanks to the Sensing Service, the user was alerted of a potential hazard not detected by the visual system, resulting in improved safety for the user.

### 7.x.5	Existing features partly or fully covering the use case functionality

Sensing: Release 19 ISAC requirements relating clauses 5.2.1 to 5.2.5 of TS 22.137 can be leveraged for this service. Examples of existing requirements that apply include:

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Positioning: The processing of the sensing data, especially in a bi-static setup, may require knowledge of the sensing transmitter and receiver positions. The positioning information can be derived from the 3GPP positioning service with requirements in clause 6.27.2 and high precision positioning in clause 7.3.2 of TS 22.261.

### 7.x.6  Potential New Requirements needed to support the use case

[PR 7.x.6-1] Subject to operator’s policy, the 6G network shall provide secure means for an authorized UE to request sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

[PR 7.x.6-2] Subject to operator policy, regulatory requirements, user consent, and agreement between operator and user, the 6G system shall provide a means to enable an authorized UE to process sensing data to derive a sensing result.

Note: This requirement may include the provision of sensing assistance information (such as location information of sensing transmitters and receivers and AI/ML models) for sensing data processing.

[PR 7.x.6-3] The 6G system shall support the following KPIs:

Table 7.x.6-1	Performance requirements of sensing results for Enhanced XR Navigation

| Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing Resolution |  | Max sensing service latency [ms] | Refreshing Rate [s] | Missed Detection [%] | False Alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| 10 m2 | 99 | ≤0.5 (Note 1) | 0.5 (Note 1) | 0.5 | N/A | 0.5 (Note 1) | 0.5 (Note 1) | ≤100 | 0.1 | 5% | 5% |
| Note 1: This positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the user |  |  |  |  |  |  |  |  |  |  |  |

* * * End Change * * * *