3GPP TSG SA WG 1 Meeting #109 	S1-250147

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	Qualcomm

pCR Title:	Pseudo-CR Update to Use case on Fixed Wireless Access (FWA)

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This pCR proposes FWA Key Performance Indicators for the FWA Use case in Clause 5.4.

1. Introduction

Existing FWA use case in Clause 5.4.2 introduces functional requirenments for 6G. In this contribution, we introduce KPIs for 6G FWA.

2. Reason for Change

As the demand for existing services supported by FWA expands and new services emerges, the capabilities supported by FWA (such as the data rates and capacityetc) are expected to also expand as well. High FWA data rates and capacity can be leveraged to support demanding applications such as the immersive gaming use case found in Clause 9.1. It is expected that 6G FWA will support these scenarios with higher data rates than 5G FWA supporting data rates less or equal to 10 Gbps [GSMA, GSMA_c]. These high data rates which are expected to be less than 60 Gbps on the DL and less than or equal to 5Gbps on the UL (details on are shown in Table 5.4 below), would be made possible by leveraging bandwidths across one or more spectrum bands (such as FR1, FR2 and FR3), and increasing spectral efficiency through advanced coding, modulation, MIMO and beamforming techniques.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870v0.1.1.

* * * First Change * * * *

### 5.4.2	Potential New Requirements needed to support the use case

[PR-5.4-1] The 6G system shall provide optimized network capabilities for FWA (e.g., support stationary devices).

[PR-5.4-2] The 6G system shall support FWA in relevant bands taking into consideration the regulatory requirements for each specific band.

[PR-5.4-3] The 6G system shall enable the means to provide awareness of user service characteristics (e.g., data rate, latency) to support the RAN and CN in making real-time resource allocation for FWA.

[PR-5.4-4] The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services.

[PR-5.4-5] The 6G system shall support efficient FWA connectivity.

Editor’s Note: this requirement is FFS.

[PR-5.4-6] The 6G systems shall support FWA DL capacity of less than to 60 Gbps and UL capacity less than or equal to 5 Gbps.

[PR-5.4-7] The 6G systems shall support the following KPIs:

Table 5.4-1 Performance requirements for high data rate and traffic density scenario.

|  | Scenario | User Experienced data rate (DL) | User Experienced data rate (UL) | DL FWA Data Rate | UL FWA Data Rate |
| --- | --- | --- | --- | --- | --- |
| 1 | Immersive gaming | Player/Cheerleader: - 640 Mbit/s Spectator: - 320 Mbit/s (Note 1) | Player/Cheerleader - 100 Mbit/s Spectator: - 10 Mbit/s (Note 1) | 57.6 Gbps (Note 2) | 5 Gbps (Note 3) |
| NOTE 1: We assume the following in calculating the FWA DL and UL Data rate: - FWA CPE would be used to provide the needed throughput to the Gymnasium for the immersive gaming use case in clause 9.1. - DL user experienced data rate is 640 Mbps each for 40 players, coaches, referees and cheerleaders and 320 Mbps each for 100 spectators (see clause 9.1 for details). - UL user experienced data rate is 100 Mbps for 40 players, coaches, referees and cheerleaders and 10 Mbps for 100 spectators (see Clase 9.1 for details). NOTE 2: The data rate between the RAN and FWA CPE on the DL, DL FWA Data Rate = [640 Mbps *40 + 320 Mbps*100] = 57.6 Gbps NOTE 3: The data rate between the RAN and FWA CPE on the UL, UL FWA Data Rate = [100 Mbps *40 + 10 Mbps*100 4.0] = 5 Gbps NOTE 4: Only one FWA CPE is assumed in deriving the needed DL/UL FWA Data Rate for this immersive gaming use case. Further analysis may be required on whether to one or multiple FWA CPE units to support this use case. These analyses are considered out of SA1 scope. NOTE 5: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |

* * * End Change * * * *