3GPP TSG-SA1 Meeting #109	S1-250149

17-21 Feb 2025, Athens, Greece	(revision of xx-yyxxxx)

Source:	China Telecom, Huawei, ZTE, China Mobile

pCR Title:	Use case on fast network provisioning to improve resilience

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Yinglin Chen, chenyl37@chinatelecom.cn

Abstract: This use case proposes to consider a 6G capability of fast network provisioning to improve network resilience.

1. Introduction

Please read the use case below.

2. Reason for Change

This is a use case.

3. Conclusions

None.

4. Proposal

It is proposed to study the use case in System and Operation Aspects.

* * *Start of Change (All new text)* * * *

5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.x	Fast network provisioning to improve resilience

### 5.x.1	Description

Network resilience is a critical concern for operators. They need to ensure that requested network services are always available. This involves two key scenarios: first, ensuring that requested services can be made available upon request in zero downtime; and second, maintaining the uninterrupted availability of essential network services, such as data connectivity service and voice service.

In the event of a network failure caused by unexpected incidents such as natural disasters or cyber-attacks, restoring network services in the affected area becomes a top priority. During the initial disaster response, ensuring the availability of essential network services, such as data connectivity service and voice service, is crucial.

### 5.x.2	Pre-conditions

A severe fire accident happens in an area, destroying numerous utilities, including Operator A's engine room. As a result, Operator A's PLMN is partially compromised and unable to provide network services in the disaster area.

### 5.x.3	Service Flows

- The local government requests that essential network services (e.g. data connectivity service and voice service) need to be recovered immediately in the disaster area.

- Operator A acknowledges the request and provisions the essential network services in the disaster area. This network provisioning can be achieved by newly deployed physical RAN and Edge Cloud resources in the close proximity of the disaster area, as well as utilizing communication equipment that has not been destroyed by the fire. The network function modules are automatically created to provide the essential network services for the disaster area using AI technologies. To expedite and simplify this network provisioning, only the necessary network function modules for providing essential network services are used based on the service level agreements with the government.

Case#1 Subscriber of Operator A’s PLMN

- Li is a subscriber of the PLMN of Operator A before the network failure. Li’s phone (UE 1) connects to the network in the disaster area. Li can use social media and make calls to inform and update friends and families.

- Li works for Company B, whose network services are provided by Operator A. Li can access the PLMN network to remotely access the enterprise server of Company B, with traffic routed from the network in the disaster area to the PLMN.

Case#2 Non-subscriber of Operator A’s PLMN

- Chen lost his phone in the fire. He receives a new SIM card and a new phone. Chen’s new phone (UE 2) connects to the network in the disaster area. Chen can use social media and make calls.

- Chen also works for Company B. However, UE 2 has no subscription to the PLMN of Operator A, and can only use the services provided by the network in the disaster area, thus he cannot remotely access the enterprise server of Company B.

### 5.x.4	Post-conditions

Users in the disaster area can continue to use the essential services during the PLMN network failure; will be switched back to the PLMN network services after the PLMN network is recovered.

### 5.x.5	Existing features partly or fully covering the use case functionality

Isolated E-UTRAN Operation for Public Safety (IOPS) is supported in EPS. However,

- IOPS is only applicable to public safety UEs

- IOPS only works for the no backhaul (to Macro EPC) scenario.

There are existing features, such as Network Slicing, Edge Computing, and Non-public Network, allowing to deploy network services in a local area to cater for specific requirements. These are good starting point to provision dedicated services to support local area service-level agreements, with which there are challenges on the following aspects:

-	agile service delivery: a flexible and agile enough way to meet the various service requirements in short time,

-	network and data autonomy: especially for sensitive services with high-level security needs.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] Subject to operator policies, the 6G system shall enable operators to rapidly provision network services as part of the operator’s PLMN network, in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain-level of autonomy and diverse functionalities in a given area during a specific time period.

NOTE:	The level of autonomy and the enabled functionalities can be based on, for example, operator policies and agreements with 3rd party.

* * *End of Change * * * *