3GPP TSG SA WG 1 Meeting #109 	S1-250151

Athens, Greece, 17-21 February 2025

Title:	Use Case on Critical infrastructure Monitoring

Agenda item:	8.1.7

Source:	NICT, ESA, ZTE

Contact:	Shiro Mazawa (shiro.mazawa@nict.go.jp)
Motoaki Hara (hara.motoaki@nict.go.jp)

Abstract: This contribution presents a use case for critical infrastructure monitoring scenario.

Background

In B5GPC white paper, given the worsening and more frequent occurrence of natural disasters, it is imperative to continue to pursue strategies for developing critical infrastructures that directly contribute to national resilience, such as disaster risk reduction and mitigation and countermeasures for aging infrastructures.  As part of countermeasures for aging critical infrastructures, critical infrastructure monitoring with time/positioning mapping of sensing/imaging sensor data and precise measurement of positional variation of infrastructure is required in outdoor location.

This contribution proposes a use case on critical infrastructure monitoring to be included in the study on 6G Use Cases and Service Requirements.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	B5GPC: "Beyond 5G White Paper ~Message to the 2030s~".

[2]	3GPP TS22.104:” Service requirements for cyber-physical control applications in vertical domains; Stage1”.

* * * Second Change * * * *

Followings are all new texts.

11 Further Use Cases on Industry and Verticals

11.x	Use case on Critical infrastructure Monitoring

11.x.1	Description

According to [1], given the worsening and more frequent occurrence of natural disasters, it is imperative to continue to pursue strategies for developing critical infrastructures that directly contribute to national resilience, such as disaster risk reduction and mitigation and countermeasures for aging infrastructures.  To achieve this, appropriate and efficient maintenance of critical infrastructures is crucial.  There are two types of maintenance: corrective maintenance and  predictive maintenance.  Corrective maintenance is performed after critical infrastructures have been damaged, deteriorated, or has lost their function or performance.  On the other hand, predictive maintenance involves planned inspections, repairs, and replacements of critical infrastructures to prevent a decline in function or performance during use.  To reduce overall cost, to improve safety, to increase reliability, and to extend lifespan of critical infrastructures, predictive maintenance become important.

In predictive maintenance, it is necessary to continuously monitor the health (current performance) of critical infrastructures and evaluate them.  Conventionally, monitoring critical infrastructures is performed manually, such as visual and sound inspection.  Recently, methods of monitoring critical infrastructure using ICT are attracting attention.

Critical infrastructure monitoring is a generic term for techniques used to objectively and continuously monitor changes in the condition of structures and buildings.  Two type of data are utlized to monitor critical infrastructure: sensing data and imaging data.  Sensing data are gathered by sensors attached to critical infrastructure (such as vibration sensors, temperature sensors, humidity sensors and strain sensors) and sent by UEs attached to critical infrastructure.  Note that UE can be not only device to send data from sensors, but also device to attach precise positioning and timestamp information to sensing data and also sensor itself of strain, distortion and inclination by measuring positional variation precisely.  Imaging data are gathered by surveillance camera, drones and robots and sent by UEs attached to surveillance camera, drones and robots.  Note that UE can be not only device to send data from surveillance camera, drones and robots, but also device to attach precise positioning and timestamp information to imaging data.  These data are mapped onto critical infrastructure in cyberspace and then diagnosed and predicted structural damage and deterioration.  Therefore, to consistently utilize huge amounts of data from many sensors, cameras, drones and robots, precise positioning and timestamp information are required.  And, to measure strain, distortion and inclination of critical infrastructure, precise positional variation information is essential.

11.x.2	Pre-conditions

Not applicable.

11.x.3	Service Flows

Not applicable.

11.x.4	Post-conditions

Not applicable.

11.x.5	Existing features partly or fully covering the use case functionality

In [2] clause 7.3.2.2, it is described that the 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1, and in Table 7.3.2.2-1, it is described that horizontal accuracy 0.3m with 99% availability is required.  Also, the 5G system supports accurate positioning capability with aid of Positioning Reference Unit.  Using Positioning Reference Unit, cm-level accuracy is achieved, but it is mainly for indoors.  However, cm-level accuracy of positioning and positional variation in outdoor is required in the use case, especially the use case of measuring strain, distortion and inclination of critical infrastructure.

11.x.6	Potential New Requirements needed to support the use case

The 6G system shall be able to support the following KPIs.

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Critical Infrastructure Monitoring | [10]cm | [1]m | [99%] | TBD | Outdoor | TBD |  |

The 6G system shall support accurate measurement capability of positional variation less than [1cm] at [99%] of occasions outdoor.