3GPP TSG SA WG 1 Meeting #109 	S1-250153

Athens, Greece, 17-21 February 2025

Title:	Use Case on communication using an ultra-narrow hot spot

Agenda item:	8.1.5

Source:	NICT

Contact:	Shiro Mazawa (shiro.mazawa@nict.go.jp)
Motoaki Hara (hara.motoaki@nict.go.jp)

Abstract: This contribution presents a use case for communication using an ultra-narrow hot spot.

Background

High-capacity communication services using the millimeter wave band (30GHz to 300GHz band) and terahertz band (300GHz to 1 THz) are expected.  Radio propagation tests and investigations using actual millimeter- and terahertz-wave devices have progressed, their strong linearity and low diffractive properties have been experimentally reaffirmed.  To communicate with a moving object such as a drone using these frequencies, it is necessary to know in advance the existence of an extremely narrow radio wave range, called “ultra-narrow hot spot”, and to start communicating when entering the ultra-narrow hot spot.

This contribution proposes a use case on communication using an ultra-narrow hot spot to be included in the study on 6G Use Cases and Service Requirements.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] Report ITU-R SM. 2450-0, June. 2019.

[2] M. J. Marcus, "WRC-19 Issues: Agenda Item 1.15 and the Use of 275–450 GHz," in IEEE Wireless Comm., vol. 23, no. 6, pp. 2-3, Dec. 2016.

[3] RR No.5.564A. Report ITU-R SM. 2450-0, June 2019

[4] https://www.nict.go.jp/press/2023/05/24-1.html

[5] https://standards.ieee.org/ieee/802.15.3/10801/

* * * Second Change * * * *

Followings are all new texts.

9 Immersive Communication

9.x	Use case on communication using an ultra-narrow hot spot

9.x.1	Description

High-capacity communication services using the millimeter-wave band (30 GHz to 300 GHz) and the terahertz band (300 GHz to 1 THz) are anticipated. In particular, the terahertz band, which has not been clearly designated for mobile communications, was identified as a frequency band exceeding 40 GHz between 250 GHz and 300 GHz at the World Radio Conference (WRC) organized by the ITU-R [1-3]. This bandwidth is nearly 100 times greater than the maximum available for 5G radios (400 MHz) and thus has the potential to achieve 1 Tbps, which is 100 times the throughput of 5G radios.  On the other hand, as radio propagation tests and investigations using actual millimeter- and terahertz-wave devices have progressed, their strong linearity and low diffractive properties have been experimentally reaffirmed.

There have been reports of successful communication between drones in flight using a 60 GHz band system [4], where the system is compliant with the IEEE 802.15.3 standard [5]. However, this type of communication is only possible when a moving object, such as a drone, is aware in advance of the existence and position of an extremely narrow radio wave area, called an “ultra-narrow hot spot”.  The drones must also accurately determine their own location and the precise moment they enter the hot spot's range.

Remote sensing and roving image inspection are services that handle large volumes of data. Conducting such sensing and inspection using multiple drones and other moving vehicles is expected to enhance observation capabilities from various angles while reducing costs, as fewer imaging devices will be required compared to maintaining multiple fixed imaging devices with limited view angles. However, data from multiple drones or vehicles cannot be effectively utilized unless the relative positions of these devices concerning the same observation target are accurately known. Additionally, when multiple observation devices are used simultaneously, they must synchronize their clocks to capture images of dynamic objects accurately.

9.x.2	Pre-conditions

Company A is entrusted with the management and maintenance of roads in City X. Multiple drones, operated by the control centre of Company A, patrol the roads and their surroundings within its jurisdiction, capturing high-resolution images. Mobile operator B has installed a base station on the roof of Company A's control centre. This base station not only serves ordinary mobile phones but is also equipped with a millimetre-wave transmitter directed overhead.

9.x.3	Service Flows

1. The drones are aware of accurate time and position, allowing their camera shutters to be released synchronously. The relative positions of the drones to neighbouring drones are also determined by measuring signal propagation time. Additionally, the drones have a function that automatically detects and alerts users to characteristic images, such as road ridges, sinkholes, and accidents, from the images captured by the drones using an internal AI image recognition processing function.

2. An alert indicating a traffic accident is reported to the control centre by drones patrolling near an intersection. The control centre instructs the drones that detected the traffic accident to deliver high-resolution image data as soon as possible to collect detailed information.

3. The drones head for the sky above the control centre.  There is a terahertz transmitter on the roof of the control centre building, and the antenna beams are pointed in a way that allows communication with drones passing overhead.  The area size of ultra-narrow hot spot depending on the altitude above is known by design and preliminary verification.

4. The control centre collects information on the current location of the drones and the size of the useful image data to be retrieved in advance. Based on this information, the control centre draws up an optimal schedule for approaching the ultra-narrow hot spot over the control centre, ensuring that each drone returns to the observation site in the shortest possible time without unnecessary waiting. The control centre also manages the terahertz communication system on the roof, configuring necessary device authentication settings and various parameters related to the communication schedule.

5. The drones head toward the airspace over the control centre upon request. They pass over the terahertz transmitter on the roof at a specified altitude and speed, transmitting high-resolution image data related to the traffic accident using terahertz communication, and soon return to the original area for regular roving inspections.

9.x.4	Post-conditions

Using the high-resolution image data observed from multiple viewpoints from multiple drones collected in this way, the control centre can analyse the event.  In addition, the shooting timing of the image data from multiple drones is synchronised in time and space with high precision, making multifaceted analysis possible.

9.x.5	Existing features partly or fully covering the use case functionality

In [2] clause 7.3.2.2, it is described that the 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1, and in Table 7.3.2.2-1, it is described that horizontal accuracy 0.3m with 99% availability is required in outdoors.  Also, the 5G system supports accurate positioning capability with aid of Positioning Reference Unit.  Using Positioning Reference Unit, cm-level accuracy is achieved, but it is mainly for indoors.  However, cm-level accuracy in outdoor locations is required in the use case.

9.x.6	Potential New Requirements needed to support the use case

The 6G system shall be able to support the following KPIs.

| Scenario | User-experienced data rate | End-to-end latency | Positioning Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical |  |  |  |  |  |
| Communication using an ultra-narrow hot spot | [10]Gbps | TBD | [10]cm | [10]cm | [99%] | TBD | Outdoor | TBD |  |

.