3GPP TSG-SA WG1 Meeting #109 	S1-250158

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	CATT, Nokia

pCR Title:	Editorial Change on 5.7 Use Case

Draft Spec:	3GPP TR 22.887 v1.0.0

Agenda item:	7.3.1

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes editorial update on Figure 5.7.3-1 to align with the description of the use cases 5.7 of TR22.887.

1. Introduction

The use case 5.7 was updated in SA1 #108 meeting to narrow down the scenario to V2N communication. Due to the unchangeable format of the associated figure, the change of the services flows was not reflected in the figure and an editor’s note has to be left in Figure 5.7.3-1 for reminder.

2. Reason for Change

1. Delete redundant sentences of the service flow 4;

2. Update Figure 5.7.3-1;

3. Remove the editor’s note of Figure 5.7.3-1.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.887 v1.0.0

* * * First Change * * * *

5.7 	Use case on assisting vehicular communications via multi-orbit satellite access

5.7.1	Description

Cellular Vehicle-to-Everything technology (C-V2X) is expected to play a critical role in the development of Intelligent Transportation Systems (ITS) and connected vehicles in the near future [10]. Its evolution to 5G is fostering the synergies between the automotive industry and other verticals to support more scenarios and enhanced driving experience. The 5G Automotive Association (5GAA) is shaping the path to 5G and has identified 7 major groups of use cases as Table.5.7.1-1 depicts.

Table.5.7.1-1 Typical C-V2X Use Cases on 5GAA Technical Reports [11]

| Use Case Category | Use Case Examples |  |
| --- | --- | --- |
| Safety | - Emergency braking - Intersection Management | - Assisted collision warning - Lane change |
| Vehicle Operations Management | - ECU software update - Vehicle Health Monitoring | - Remote support |
| Convenience | - Infotainment - Assisted and cooperative navigation | - Autonomous smart parking |
| Autonomous Driving (L4 and L5) | - Control if autonomous/self-driving is allowed or not - Tele-operation | - Handling of dynamic maps(update/download) - Data/Sensor Sharing |
| Platooning | - Platooning management - Determine position in platoon - Dissolve a platoon | - Manage distance within platoon - Leave a platoon - Control of platoon in steady state |
| Traffic Efficiency and Environmental Friendliness | - Speed Harmonization - Traffic jam information, e.g. traffic Light coordination | - Routing advisor, e.g. Bus Lane sharing |
| Society and Community | - Emergency vehicle approaching - Traffic light priority | - Patient monitoring - Accident Report |

The communication needs of each specific use case can vary in e.g., latency, capacity, data rate, reliability, etc. For example, software update will request a large amount of vehicles scattered all over the world to download Gigabyte-level data simultaneously without stringent requirements in the latency. Assisted collision warning requests timely information exchange between ITS and the vehicles but the message size is only hundreds of bytes.

However, it is almost consensus that ubiquitous connectivity shall be a fundamental pre-requisite to enabling full-scale digital experience for cars and a fully connected car. Relying only on the coverage of terrestrial access network, it is still a challenge to ensure the network availability and reliability of fully connected cars. Satellite access networks can complement terrestrial network based solutions, especially in white spots (e.g., rural or remote areas) for V2N use cases and in the event of outages and congestion. GEO satellite can provide the global coverage while LEO satellite can support low latency communication on top of this capability. Today, over 200 million connected vehicles worldwide are equipped with applications sharing hazard and traffic warnings on the road. [12] The differentiated characteristics of LEO and GEO satellites will lead to flexible and hybrid solutions based on multi-orbits satellite strategy and terrestrial access network.

This use case illustrates the usage of GEO satellite and LEO satellite in assisting the communication between ITS and the vehicles under the coordination of terrestrial network.

5.7.2	Pre-conditions

TerRAN deployed by the terrestrial network operator TerOP has full coverage in the urban areas (e.g., AreaA) and rural area, but limited or even no coverage in deserts and depopulated area (e.g., AreaB).

TerOP has service agreements with satellite operator SatOP1 for sharing SatRAN#1 (GEO satellite based NG-RAN) and SatOP2 for sharing SatRAN#2 (LEO satellite based NG-RAN) to realize the global coverage of all the countries.

The telematics devices (e.g., UE#1, UE#2, UE#3) installed on the autonomous driving trucks are the subscribers of TerOP and are allowed for multi-orbit satellite services under the agreement between TerOP and the vehicle management company, who is the owner of the trucks.

All UEs are capable of GEO and LEO satellite access.

5.7.3	Service Flows

1. All the trucks are launched out to deliver the goods. UE#1, UE#2 and UE#3 are registered to 5GC via nearby TerRAN and communicate with ITS via TerRAN as Figure 5.7.3-1 (a) shows, where

- UE#1 and UE#3 are downloading new firmware of Infotainment system with the traffic routed following blue colour line.

- UE#2 and UE#3 are requested to share some data for autonomous driving when they are driven into AreaA, and the data traffic is routed following red colour line.

2. As UE#1 moves into AreaB, UE#1 is switched to SatRAN#1 according to service QoS, and/or satellite availability, to continue the firmware download (with minimum interruption) and the signalling exchange as Fig 5.7.3-1(b) shows.

3a. When Area A has no terrestrial coverage in disaster scenario, UE#2 is switched to SatRAN#2 according to the operator’s policy, to continue the transmission (with minimum interruption) as Fig 5.7.3-1(b) shows.

3b. When Area A has no terrestrial coverage in disaster scenario, according to the operator’s policy, UE#3 is switched to SatRAN#2 to continue the data transmission for autonomous driving (with minimum interruption). When SatRAN#2 flies away(as T2 time), UE#3 is switched to SatRAN#1 to continue the firmware download as Figure 5.7.3-1 (b) shows.

4. When UE#1, UE#2 and UE#3 detect the coverage of TerRAN, they are switched back to TerRAN regarding the operator’s policy.

(a) communication via terrestrial RAN	(b) communication via multiple satellite RANs

Figure 5.7.3-1 Assist V2N via Multi-orbit Satellite Access

5.7.4	Post-conditions

Thanks to multi-orbit satellite services, UE#1 and UE#3 complete the firmware download and upgrade successfully, UE#2 and UE#3 share some data for autonomous driving with ITS.

5.7.5	Existing features partly or fully covering the use case functionality

SA1 has identified a series of requirements related to satellite access and captured them in TS 22.261.

For RAN sharing, the following requirement can be applicable for this use case.

- A 5G satellite access network shall support NG-RAN sharing.

For the service continuity and user plane aspects, the following requirements can be partly applicable for this use case but not consider multi-orbit situation.

- A 5G system with satellite access shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by different operators having an agreement.

- A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfils the agreed QoS.

- A 5G system with satellite access shall be capable of supporting simultaneous use of 5G satellite access network and 5G terrestrial access networks.

5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] Subject to regulatory requirement and operator’s policy, a 5G network with multi-orbit satellite access shall support mechanisms to allow a UE connecting to one of satellite access networks in different orbits considering e.g., QoS, satellite availability.

[PR 5.7.6-002] Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbit satellite access shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN.

[PR 5.7.6-003] A 5G network with multi-orbit satellite access shall be able to collect charging information for a UE served by different orbit types of satellite access networks.

Editor's Note: the terminology of the above three requirements still needs to be aligned with other part of the TR

* * * End of Changes * * * *