3GPP TSG-SA WG1 Meeting #109 	S1-250162

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.125 | CR | 0056 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhanced C2 communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph4 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The study on satellite access phase4(FS_5GSAT_Ph4) has got good progress and several consolidated requirements for enhancing the support to emergency communication and mission critical services are captured in TR 22.887 clause 6.4, including - Enhance MCX support with efficient content delivery; - Enhance MCX support with mobile base station relay; - Enhance UAV communication for MCX and associated KPIs; - Exposure of location information with finer granularity; Regarding the conclusion of the study, UAV related requirements are recommended to be introduced to TS 22.125. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces new requirements recommended from the study of satellite access phase4 as clause 6.4 of TR22.887. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the new requirements, the emergency communication and mission critical services involved satellite access can not meet the expectation of the users e.g. regulators, public safety users. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 22.261 CR0830 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.5	C2 communication

[R-6.5-001] The 3GPP system shall support C2 communication with required QoS for pre-defined C2 communication models (e.g. using direct ProSe Communication between UAV and the UAV controller, UTM-navigated C2 communication based on flight plan between UTM and the UAV, using UE-Satellite-UE Communication between UAV and UAV controller/UTM mounted on a mobile base station relay using satellite access).

NOTE: QoS is end-to-end QoS, with respective to KPIs defined in Table 7.2-1 that are applicable for all the C2 communication models.

[R-6.5-002] The 3GPP system shall support C2 communication with required QoS when switching between the C2 communication models.

[R-6.5-003] The 3GPP system shall support a mechanism for the UTM to request monitoring of the C2 communication with required QoS for pre-defined C2 communication models (e.g. using direct ProSe Communication between UAV and the UAV controller, UTM-navigated C2 communication between UTM and the UAV, using UE-Satellite-UE communication between UAV and UAV controller/UTM mounted on a mobile base station relay using satellite access).

* * * Next Change * * * *

7.2	KPIs for UAV command and control

UAVs may use a variety of flight command and control modes. Command and control (C2) communications refers to the two-way communication, which may include video, required to control the operation of the UAV itself. C2 messages may be communicated with the UAV controller, the UTM or both and may or may not be periodic. UAV controller and UTM communications may happen at essentially the same time with different required QoS. Any mission specific communication (e.g. HD video for area surveillance), if required, is additional. Different modes of control and their typical KPIs are listed in this clause below. The 5G system shall support UAV operation at altitudes of at least 120m / 400ft above ground level, e.g. the services should be provided and characterized up to 3000ft AGL.

Table 7.2-1 KPIs for command and control of UAV operation

| Control Mode | Function | Typical Message Interval | Max UAV ground speed | Typical message Size (note 1) | End to end Latency | Reliability (note 2) | Positive ACK (note 8) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Steer to waypoints (note 3) | UAV terminated C2 message | >=1 s | 300 km/h | 100 byte | 1 s | 99.9% | Required |
|  | UAV originated C2 message (note 4) | 1 s |  | 84-140 byte | 1 s | 99.9% | Not Required |
| Direct stick steering (note 5) | UAV terminated C2 message | 40 ms (note 6) | 60km/h | 24 byte | 40 ms | 99.9% | Required |
|  | UAV originated C2 message (note 7) | 40 ms |  | 84-140 byte | 40 ms | 99.9% | Not Required |
| Automatic flight on UTM (note 10) | UAV terminated C2 message | 1 s | 300 km/h | <10 kbyte | 5 s (note 9) | 99.9% | Required |
|  | UAV originated C2 message | 1 s (note 9) |  | 1500 byte | 5 s (note 9) | 99.9% | Required |
| Approaching Autonomous Navigation Infrastructure | UAV terminated C2 message | 500 ms | 50 km/h | 4 kbyte | 10 ms | 99% | Required |
|  | UAVoriginated C2 message | 500 ms |  | 4 kbyte | 140 ms | 99.99% | Required |
| NOTE 1: Message size is at the application layer and excludes any headers and security related load. The numbers shown are typical as message size depends on the commands sent and is implementation specific. NOTE 2: Message reliability is defined as the probability of successful transmission within the required latency at the application layer while under network coverage. NOTE 3: Video is neither required nor expected to be used for steering in this mode. NOTE 4: It may be possible to transmit this message on an event driven basis (e.g. approaching a geo fence). A status message may, but is not required to, be sent as a response to a control message. NOTE 5: A video feedback is required for this mode. The KPIs for video are defined in table 7.2-2. NOTE 6: UAVs on-board controllers typically update at either 50Hz (20ms) or 25Hz (40ms). NOTE 7: A status message may, but is not required to, be sent as a response to a control message A 1Hz slow mode also exists. NOTE 8: Positive ACK is sent to the originator of the message (i.e. UAV controller and / or the UTM). The 5G system makes no assumption whether an appropriate ACK is sent by the application layer. NOTE 9: At the application layer, the C2 communication between a UAV and UTM can be allowed to experience much longer traffic interruptions, e.g. timeouts of 30 s on the uplink and 300 s on the downlink. NOTE 10: This only represents periodic message exchange during a nominal mission in steady state. Itdoes not represent unusual or aperiodic events such as conveying dynamic restrictions or a flight plan to the UAV on the downlink. |  |  |  |  |  |  |  |

Table 7.2-2 KPIs for video used to aid UAV control.

| Scenario (note 2) | Data rate | End to end Latency | Reliability (note 1) | Direction | Positive ACK required |
| --- | --- | --- | --- | --- | --- |
| VLOS (visual line of sight) | 2 Mpbs at 480 p, 30 fps | 1 s | 99.9% | Sent by UAV | Not Required |
| Non-VLOS | 4 Mbps at 720 p, 30 fps 6 Mbps at 1080p 30 fps | 140 ms | 99.99% | Sent by UAV | Not Required |
| NOTE 1: Message reliability is defined as the probability of successful transmission within the required latency. NOTE 2: Maximum UAV speed is same as control mode of direct stick steering in 7.2-1 |  |  |  |  |  |

* * * End of Changes * * *