3GPP TSG-SA WG1 Meeting #109 	S1-250169

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	CATT

pCR Title:	Use Case on Avatar-based Holographic Conferencing

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes a use case about applying avatar-based holographic conference in the play rehearsal via 6G network, which requests QoS management of the transmission of avatar media and multi-modal data, and to coordinate trusted computing resources for the media rendering during the conference communication.

1. Introduction

The application of avatar communication in mobile metaverse service and multimedia telephony service has been discussed based on the 5G system. However, it is still challenging for a 5G system to achieve the expected service performance, such as stringent transmission latency and consistent experience of all the participants. 6G is expected to address the above challenges and achieve better immersive experience with enhanced network capabilities that can additionally drive more immersive telepresence applications like mixed reality classroom, virtual meetings with holographic avatars. This contribution introduces a use case on avatar-based holographic conferencing.

2. Reason for Change

Introduce new requirements based on 5G AR/XR services, mobile metaverse services and IMS multimedia telephony services.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.1.1

* * * First Change * * * *

3		Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

avatar: as defined in 3GPP TS 22.156 [29];

digital representation: as defined in 3GPP TS22.156 [29];

* * * Next Change * * * *

9.x	Use case on Avatar-based Holographic Conferencing

9.x.1	Description

Compared with video communication, the avatar communication, that uses the 2D/3D digital representation of a user, can provide interactive, real-time feedback and immersive experience without revealing the user private information such as the actual appearance, the surrounding environment, etc., and in some level mitigate the gap of communication experience between the participates with different device’s multimedia capabilities (e.g. text, audio, imaging). The application of avatar communication in mobile metaverse service and multimedia telephony service has been discussed based on the 5G system. However, it is still challenging for a 5G system to achieve the expected service performance, such as stringent transmission latency and consistent experience of all the participants.

- Stringent transmission latency

The creation of 3D avatar based on sensor data of pose, gesture and facial expression and digital representation models will introduce additional processing delay compared to normal video communication. That means the network must transmit the avatar media with lower latency to ensure the synchronized service experience for a user using avatar and a user using normal video, in the same conversational communication service. Moreover, the complexity of representation models and types of modalities will be proportional to the processing time in the same situation, and thus the better immersive experience would request more stringent transmission latency.

- Consistent experience of all the participants

The computing capabilities and display capabilities may differ among the UEs of the participants. For example, some UEs with sufficient computing capabilities (e.g. XR glasses) can render 3D avatar media and video locally, while some UEs (e.g. smartphone) only afford local image processing but can display 3D avatar. For the latter, the rendering of 3D avatar can be split to other computing nodes inside the network such as edge computing server or AR media function (in case of IMS based avatar communication). Thus, to support optimal immersive experience, the network should take a role in media transcoding and coordinating the computing resources (e.g. edge computing server, IMS media function or UE) for representation model adaption and split rendering. Moreover, when the participants of an interactive avatar conference are located in relatively distant locations, the avatar of one participant should be loaded to the rendering server of other participants simultaneously and rendered separately. Thus, the collaboration between the rendering and the transmission of multi-modal avatar media becomes crucial for all the participants to have a consistent and synchronized telepresence experience. In current system, the selection of the rendering server will mainly consider the proximity to the UE presenting the avatars, while weaken the influence of other factors, such as computing capabilities and efficiency, and the characteristics and status of the transmission path. This policy may achieve optimal for certain UE but impair the overall performance and consistency of user experience between different participants.

6G is expected to address the above challenges and achieve better immersive experience with enhanced network capabilities that can additionally drive more immersive telepresence applications like mixed reality classroom, virtual meetings with holographic avatars.

This use case describes an avatar-based holographic conference with 6G system. The holographic avatars require high-detailed modelling, high-texture mapping and high-precision simulation, so that the computing demands for avatar rendering have increased dramatically, and may exceed the UE’s computing capabilities. Meanwhile, the participants have individual preference on the representation models and expect to apply the personalized models regarding different scenarios. No matter the holographic conference is held on IMS or 3rd party’s application platform, the 6G system can offer an expected real-time virtual conferencing experience in form of holographic avatars.

9.x.2	Pre-conditions

Bob, Alice and Yun are actors living in the United States, Germany and China respectively. According to the schedule, they will rehearse all the scenes of the new play through a virtual holographic conference before gathering to the film location for the official shooting. For the scenes of different seasons, they will use corresponding avatars to stimulate the situation at that time.

Their holographic XR devices have different hardware and software configurations, but all support media rendering, task splitting, and media composition. They are the subscribers of 6G network operators in the respective countries and have been authorized to use holographic communication services.

Each operator is equipped with compute, storage and AI processing resources by deploying edge computing servers and/or cloud computing centres and IMS media functions.

9.x.3	Service Flows

The following service flows are applicable for each participant (taking Bob as an example):

- Bob joins in holographic conference and agrees to deploy its 3D avatars. During the media type negotiation and resource allocation, each of RTC applications (or IMS networks) will identify the request from the peer participant about model generation and rendering offloading.  It then enlists the help of 6G core network to coordinate proper computing resources for offloading.

- The 6G core network of peer participants selects proper edge computing servers (or AR Media Functions in case of IMS) and notify Bob’s device of the coordination result.

- When Bob performs and speaks lines, several traffic flows are transmitted between Bob and selected edge computing servers(or AR Media Functions in case of IMS) /peer participants on requested QoS, including the sensor data stream of Bob’s pose, gesture and motion, the sensor data stream of facial expression, the conversational voice flow, the interactive information and the signalling.

- Each of the selected edge computing servers (or AR Media Functions) applies the incoming multi-modal data on the corresponding latest representation model of remote participant (e.g. Bob, Alice), and also create a combined digital 3D representation including all the peer participants with configured background. The mixed model is provided as the input to the selected edge servers or the devices for rendering, which data will be aggregated, proceeded (e.g. compression, transcoding) and transmit to the corresponding peer participant for display.

- As the scene switches, Bob requests to deploy a new 3D avatar which will be transmitted to the edge computing server (or AR Media Functions) of each peer participant and taken into use immediately.

9.x.4	Post-conditions

The rendered 3D scenes can be displayed simultaneously to Bob, Alice and Yun in different locations, and they can obtain immersive and real-time user experience.

9.x.5	Existing features partly or fully covering the use case functionality

The service requirements for avatar-based real-time communication have been defined in TS 22.156.

The functional and performance requirements for tactile and multi-modal communication service, AR/VR services have been defined in clause 6.43, clause 7.6.1 and 7.11 of TS 22.261.

The support of AR media processing in IMS multimedia telephony service is defined in clause 6.39 of TS 22.261.

9.x.6	Potential New Requirements needed to support the use case

[PR-9.x.6-1] Subject to operator’s policy, the 6G system (including IMS) shall support QoS of the transmission of real time avatar media and multi-modal data (e.g. video, audio, image, haptic information) associated with the same user within the. multimedia conversational communications between two or more users.

NOTE: the avatar media stands for the animated digital representation, which is created real-time based on the sensor data for the user’s pose, gestures, and facial expressions, as specified in TS 22.156 [29]

[PR-9.x.6-2] Subject to operator policy, the 6G system (including IMS) shall support mechanisms to manage the trusted computing resources (i.e. IMS core network, or operator managed edge server(s)).

[PR-9.x.6-3] Subject to operator policy, the 6G network shall allow the authorized users to utilize the trusted computing resources (i.e. IMS core network, or operator managed edge server(s)).

[PR-9.x.6-4] Subject to operator policy, the 6G network (including IMS) shall enable core network to aggregate, process, store the rendering data from trusted computing resources, and transfer to a UE.

* * * End of Changes * * * *