3GPP TSG-SA WG1 Meeting #109 	S1-250182

Athens, Greece, 17-21 February 2025

Source:	SK telecom

New use case title:	New use case on Orchestration between RAN and AI workload

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	hyeonsoo.lee@sk.com, jaehyun_lee@sk.com

Abstract: <provide a short description of the content>

This contribution proposes a new use case on Orchestration between RAN and AI workload.

---------- Use Case template ----------

x.1	Use case on Orchestration between RAN and AI workload

x.1.1	Description

<Describe what the use case intends to achieve.>

Operators are experiencing stagnation in revenue as demographical changes. Given the substantial investment required for generational change, it is crucial to explore new monetization opportunities for operators beyond cost reduction, especially with the advent of the 6G era. One of the most promising approaches is leveraging existing assets, ranging from short-term strategies such as providing colocation services to long-term initiatives that repurpose base stations as platforms for delivering AI services

Base stations process large amounts of data, including physical layer signal processing and complex radio resource allocation in RAN. To enhance RAN performance, the use of xPUs (e.g., GPUs) is increasing, which can be starting point for AI-native RAN. Base stations experience traffic surges in specific cells and time periods where users are concentrated. This makes it possible to utilize idle base station resources during off-peak hours for AI services, generating additional revenue for operators

AI services require large-scale data training and real-time inference that demands low latency. Because base stations are located close to users, they are well-suited to meet these requirements. This strengthens the role of base stations as common infrastructure for both RAN and AI workloads.

Therefore, resource orchestration between RAN and AI workloads is one of the core technologies for AI-native RAN. The main challenge point of resource orchestration is to guarantee the RAN performance with or without AI workloads during resource management. Therefore, within a single base station, hardware resources such as GPUs should be allocated dynamically between RAN and AI workloads without any performance degradation on RAN, while ensuring the AI service requirement in the same time.

x.1.2	Pre-conditions

The operator's base stations are equipped with xPUs(e.g., GPU), enabling the processing of both AI workloads of application servers and RAN workloads

Each base station monitors the current HW resource usages for each workload, and an orchestrator in management layer collects this information from multiple base stations and distributes AI workloads accordingly.

Maintaining RAN performance (e.g., bit error rate, latency, etc.)  is the highest priority for the operator's base stations.

x.1.3	Service Flows

- Base stations A and B provides AI workload as well as RAN workload and monitor the hardware resource usage for both.

- Base station A and B report monitoring information and its current network status, including traffic volume to the Orchestrator

- As mobile traffic increases at base station A, more hardware resources are needed to handle the additional RAN workload.

- To allocate more resources for RAN processing at base station A, the orchestrator decides to migrate the AI workload from base station A to base station B.

- Based on the orchestrator's decision, hardware resource allocation is updated at both base station A and B accordingly.

x.1.4	Post-conditions

None

x.1.5	Existing features partly or fully covering the use case functionality

None

x.1.6	Potential New Requirements needed to support the use case

The 6G system shall provide a framework for operating AI workloads provided by 3rd party.

The 6G system shall orchestrate to allocate hardware resources of base stations adaptively.

The 6G system shall prioritize maintaining mobile network performance(e.g., bit error rate, latency, etc).