3GPP TSG SA WG 1 Meeting #109 	S1-250196

Athens, Greece, 17-21 February 2025

Title:	Use Case on satellite subnetwork to support satellite network service

Agenda item:	8.1.1

Source:	CSCN

Contact:	Qiuting Li <liqt@chinasatnet.com.cn>

Jingwang Ma <majw@chinasatnet.com.cn>

Abstract: This contribution proposes a new network deployment mode for PLMN network (subnetwork) to allow a more flexible, dynamic, and effective way to support satellite network service requirements. This use case illustrates how to offer diversity satellite subnetwork services for the users and derives the potential system requirements.

1. Introduction

“System and Operational Aspects” has been identified as one of key areas for 6G study. This contribution proposes a new use case on distributed subnetwork onboard the satellite to support satellite network services for the users.

2. Reason for Change

The subnetwork is the certain-level autonomous distributed segment of the PLMN network; it offers the network services with certain-level autonomy and diverse functionalities onboard satellites depending on satellite network service requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	IMT-2030 (6G) Promotion Group white paper on "6G Vision and Candidate Technologies"

* * * Second Change * * * *

5.x	Use case on subnetwork on the satellites to support satellite network services

5.x.1	Description

This use case describes offering the subnetwork services on board satellites.

The subnetwork can be regarded as the certain-level autonomous and distributed segment of PLMN network to provide network services to the users within one area, the specified group etc. The scenarios for subnetwork include the network serving the specified enterprise or organization, the network on board satellites providing satellite network services for one area.  For one subnetwork on board satellite, its network functionalities and services may reside on one or several satellites, and the subnetwork is also able to interwork with other subnetwork and the ground network to support service continuity and other required services. It is expected that subnetwork can realize more efficient network service with the reduced service delay, the reduced interworking requirements with the ground network and more network service onboard.

The subnetwork concepts are already investigated and proposed in other Standards Development Organization(s), like distributed autonomous networks released by IMT-2030 (6G) Promotion Group in white paper on "6G Vision and Candidate Technologies" [x1]. It is generally expected that the subnetwork will contribute to a more flexible, dynamic, distributed, and cost-effective deployment of the PLMN network, aiming to address the more divers scenarios that 6G enables.

5.x.2	Pre-conditions

PLMN NTN network of MNO X deployed subnetwork on board the set of satellites to provide the continuous network coverage for the specified areas. The subnetwork onboard satellites support the network services including Internet browsing and UE-Satellite-UE call application.

The users in these specified areas already have the service contact with MNO X or PLMN network of MNO Y which has service agreement with MNO X.

5.x.3	Service Flows

Figure 5.x.3-1: Subnetworks onboard satellites

- The user equipment of Bob powers on and register to the subnetwork onboard the Satellite A.

- Bob initiates the network service i.e. Internet browsing service.

- During servicing period, the subnetwork on board Satellite A needs change to Satellite B due to the satellite movement. The network functions onboard the Satellite A interwork with the network functions onboard Satellite B to perform the change operation while maintaining the service continuity for Alice and other users, and the impact on the UE can be minimized as possible, e.g. the required signalling between UE and subnetwork is minimized.

- While internet browsing application is running, Bob receives one UE-Satellite-UE communication call from Alice, and Alice is served by the subnetwork on board Satellite C. The call between Bob and Alice is successfully established.

- After they finish the call, internet browsing application on Bob’s UE still works.

5.x.4	Post-conditions

The network functions onboard satellite provides the diverse network services for the users, and the service continuity and quality are also maintained while satellites serve the users. The network functions onboard satellites create one subnetwork which can be autonomous and reliable even when the connection with the network on ground is not available, and the interworking signaling with the ground network can also be much reduced.

5.x.5	Existing features partly or fully covering the use case functionality

None.

5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 6G system shall be able to support the subnetwork onboard the satellites to provide the network services.

[PR 5.x.6-2] The 6G system shall support the interworking mechanism between the subnetworks onboard the satellites to maintain the service continuity i.e. the connectivity service for the UEs due to the mobility and change of the satellite serving the UEs.