3GPP TSG-SA1 Meeting #109	S1-250202

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0832 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of normative input based on FS_EnergyServ_Ph2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the conclusions and recommendations of FS_EnergyServ_Ph2 documented in TR22.883, new considerations and normative requirements & KPIs are proposed to be added to TS 22.261. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 6.15a.1, generic considerations are added to clarify the attribution & accuracy of energy-related information, and other clarifications. - In sub-clause 6.15a.2, normative requirements corresponding to the CPRs of TR22.883 cl 6.1.2/3/4 are added. - In sub-clause 6.15a.5, normative requirements corresponding to the CPRs of TR22.883 cl 6.1.1 are added - In clause 7, a new sub-clause is added to add KPIs for energy-related characteristics Editor’s Notes are for DRAFT version only and will be removed from the submitted CR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FS_EnergyServ_Ph2 study result in no normative work being provided on time by SA1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15a and 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.15a	Energy Efficiency as a Service Criteria

### 6.15a.1	Description

Climate change and the rising consumption of energy motivate increased energy efficiency. Energy efficiency is a strategic priority for telecom operators around the world.

Energy efficiency as (a) service criteria allows services to be delivered with diverse energy efficiency and energy consumption policies. Such criteria are expected to be subject to regulatory requirements, in particular to keep delivering critical services such as emergency calls, PWS, MPS and MCS to the extent possible. Energy consumption and efficiency information and network energy states can be exposed to third parties and energy consumption can be constrained (e.g. energy rationing).

Energy related information can include ratio of renewable energy and carbon emission information when available. Calculation of energy related information as described in the following requirements can be done by means of averaging or applying a statistical model to a combination of measurements, service configuration, statistics, computations and estimates. The requirements do not imply that some form of 'real time' monitoring, nor exact measurement is required (e.g. at flow level). Instead, they can be met by means of attribution as accurate as possible, e.g. based on data volume and network’s carbon and/or energy KPIs or metrics of the communication service, which could be collected either from the network or from sources external to 3GPP (e.g. energy supply mix provided by electricity grid providers). Hence the accuracy of the attributed information also depends on this input.

The level of accuracy of attribution and associated time period can impact what can be done with the energy-related information. It is expected that the 5G network does not consume more energy to provide this information than the estimated output value of the provided information itself.

In any case, the energy-related information considered in the following requirements only relates to the use of the communication service provided by a telecom operator to its own (home) subscribers, as a single part of the end-to-end service chain. The following requirements do not apply to the energy-related information of other providers outside of 3GPP scope potentially involved in the same service chain (e.g. video streaming providers).

### 6.15a.2	Energy related information as a service criteria

#### 6.15a.2.1	Description

Energy consumption can be monitored and considered through O&M as part of network operations [47], as well as a service criteria.

For best-effort traffic, that is, without QoS criteria, policies can be defined to limit energy use for services. This is not in conflict with the principle that performance policies will not be traded off for energy efficiency, since best-effort service has no performance guarantees.

Specifically, best-effort traffic can be subject to a policy that limits the maximum energy consumption over time, or further constrained by location (so that the energy consumption limit only applies when used in a specified service area.)

Additionally, policies can be defined with a maximum energy credit limit, e.g. for best-effort services to limit the total amount of energy consumption according to an energy charging rate. These policies expand the options of subscription policies to control energy consumption in the 5G system.

#### 6.15a.2.2	Requirements

Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a means to associate energy consumption information with charging information based on subscription policies for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services without QoS criteria.

NOTE 1: The result of the credit control is not specified by this requirement.

NOTE 2:	Credit control [49] compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed.

Subject to operator’s policy, the 5G system shall support a means to define subscription policies and means to enforce the policy that define a maximum energy consumption (i.e. quantity of energy for a specified period of time) for services without QoS criteria.

NOTE 3:	The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

The 5G system shall provide a mechanism to include Energy related information as part of charging information.

Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy related information associated with a set of application servers.

Subject to user consent and operator policy, 5G system shall be able to provide means to modify a communication service based on energy related information criteria based on subscription policies.

Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide means to operate part or the whole network according to energy consumption requirements, which may be based on subscription policies or requested by an authorized 3rd party.

Editor’s Note: insert CPRs related to TR 22.883 cl “6.1.2 Service adjustments based on energy-related characteristics”, “6.1.3 Charging-related aspects” and “6.1.4 Energy-related characteristics as service criteria”. This may imply to reword some existing requirements.

### 6.15a.3	Support of different energy states

#### 6.15a.3.1	Description

Supporting different energy states is beneficial for verticals and operators to save energy according to different working status of telecommunication equipment and manufacturing.

#### 6.15a.3.2	Requirements

The 5G system shall support different energy states of network elements and network functions.

5G system shall support dynamic changes of energy states of network elements and network functions.

NOTE: This requirement also includes the condition when providing network elements or functions to an authorised 3rd party, the dynamic changes can be based on pre-configured policy (the time of changing energy states, which energy state map to which level of load, etc.)

The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions.

### 6.15a.4	Monitoring and measurement

#### 6.15a.4.1	Description

Different levels of monitoring and measurement related to energy consumption and efficiency bring more support in energy efficiency and energy saving. In this section, monitoring and measurement related to energy consumption and efficiency include network functions in NPN and also all kinds of NG-RAN deployment scenarios.

#### 6.15a.4.2	Requirements

Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity.

NOTE 1:	Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party.

NOTE 2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc.

NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, related to same time interval e.g. hourly or daily.

NOTE 4: The network performance statistic information could be the data rate, packet delay and packet loss, etc.

### 6.15a.5	Information exposure

#### 6.15a.5.1	Description

Information related to energy consumption and efficiency is not only necessary for network internal optimization, but also will benefit the service adjustment for 3rd party.

The performance requirements related to the exposure of network energy-related characteristics such as energy consumption and CO2e emissions depending on the target use case are documented in clause 7.13.

#### 6.15a.5.2	Requirements

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party.

NOTE 1: Energy consumption information can include ratio of renewable energy and carbon emission information when available. The reporting period could be set, e.g., on monthly or yearly basis and can vary based on location.

NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

Subject to operator’s policy, agreement with 3rd party and consent by the customer, the 5G system shall be able to expose the network performance statistic information (e.g. the data rate, packet delay and packet loss) together with energy consumption information resulting from service provided to the customer, to the authorized third party, related to the same time interval e.g. hourly or daily.

Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit).

Subject to operator policy, the 5G system shall provide means for the trusted 3rd party, to configure which network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party, needs to be exposed along with the information on energy consumption for serving this 3rd party.

Based on operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time.

Editor’s Note: insert CPRs related to TR 22.883 cl “6.1.1 Exposure of network energy-related characteristics”. This may imply to reword some existing requirements.

### 6.15a.6	Network actions leveraging energy efficiency as a service criteria

#### 6.15a.6.1	Description

This clause addresses requirements to the 5G system that leverage energy-related information (e.g., energy consumption, energy efficiency), amongst others (e.g., network load), as criteria for network internal optimization actions targeting energy savings, within and across operators in a localized (i.e., geographically bound) and/or temporal (i.e., time bound) manner.

One of the strategies to save energy within mobile networks is to shut down some RAN nodes at times of low usage.

Eventually only one communication service could be used on a local basis among operators at times of low usage, as further energy saving gain to be exploited. Agreements could be put in place between operators so that in the low load periods (e.g., nighttime) only one of multiple mobile networks may be active in an area and will provide communication service to the subscribers of all networks, whereas the other networks can apply cell shutdown of their own infrastructure to obtain network energy savings.

Alternatively, based on risks of power outage nation-wide/region-wide, regulators could ask operators to “optimize” their coverage e.g., shutdown some nodes in overlapping coverage areas during energy peak hours and/or in specific geographical areas, whilst still guaranteeing minimum coverage/service (in particular to fullfill regulatory requirements for services such as emergency calls, PWS, MPS and MCS).

This can also apply between NPN operators and/or with PLMN operators.

#### 6.15a.6.2	Requirements

Subject to regulatory requirements and operators’ policies, the 5G system shall enable an operator to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators.

NOTE 1: The other operators are assumed to stop providing access to their own network infrastructure within the same geographical area to save energy during that time.

NOTE 2: Policies may include predefined times/locations, energy consumption/efficiency thresholds, etc.

NOTE 3: It is assumed that the 5G system can collect charging information associated with serving UEs of other operators.

SECOND CHANGE (NEW TEXT)

## 7.13	KPIs for energy-related characteristics exposure

The functional requirements related to exposing network energy-related characteristics can be found in clause 6.15a.5.

Exposing carbon equivalent emissions at subscriber level can rely on but does not imply any real-time collection, monitoring or exact subscriber-specific measurements or calculations. It can be provided by means of attribution based on a combination of statistics, computations and estimates as accurate as possible, e.g. based on subscriber’s data volume and operator’s carbon intensity to provide the communication service, which can be collected from external sources. Hence the accuracy of the exposed information depends on this input.

Deriving and providing such information too frequently to many users may consume network energy. Hence, the following KPIs have been defined as a compromise to achieve specific use cases whilst trying to minimize their energy impact.

Table 7.13-1 defines the performance requirements related to the exposure of network energy-related characteristics such as energy consumption and carbon emissions depending on the target use case and associated intended impact.

- To contribute to the comprehensive energy consumption and CO2e transparency in the end-to-end service chain (including both 3GPP and non-3GPP related providers), it is important that the frequency of such indications is meaningful to the user during a session of the communication service (e.g. not more frequently than once per minute when using a video streaming application).  Such information may impact the behavior of users during their current session.

- To empower mobile subscribers (individuals and companies) to contribute to overall energy and carbon footprint reduction by understanding the carbon equivalent emissions resulting from their use of the communication service, it is important to provide periodic information (e.g. per day over a month period), although such reports may only impact the behavior of users during future sessions.

Table 7.13-1: Performance requirements for energy-related characteristics exposure

|  |  | Characteristic parameter (KPI) |  |  |
| --- | --- | --- | --- | --- |
| Information | Use Cases | System Granularity | Time Granularity | Time Range | Obtainment (informative) |
| Carbon emissions (e.g. in kg of CO2e) | Subscriber’s carbon footprint information exposure to 3rd party. (NOTE 1) | UE | Day | Month | Calculation (e.g. average or statistical model) based on attribution of network energy consumption, carbon intensity and/or renewable energy ratio. |
|  | Energy Consumption and CO2e transparency in the end-to-end service chain (NOTE 2) | Service data flow | Minute | Session duration |  |
| Energy consumption (e.g. in kWh) | Energy Consumption and CO2e transparency in the end-to-end service chain (NOTE 2) | Service data flow | Minute | Session duration | Calculation of network energy consumption based on attribution e.g. measurements, service settings, external information and models. |
| NOTE 1: Such indication should be meaningful to the user to raise awareness on a monthly reporting basis or to incentivize the service consumer (out of scope of 3GPP). NOTE 2: Frequency of such indications should be meaningful to the user during the communication service (e.g. video streaming application). |  |  |  |  |  |

Editor’s Note: update KPI table as needed to align with consolidation

END OF CHANGES