3GPP TSG-WG SA1 Meeting #109	S1-250212

Athens, Greece, 17-21 February 2025

Source:	Huawei

pCR Title:	Pseudo-CR on update of Use Case 5.20

Draft Spec:	3GPP TR 22.887 v1.0.0

Agenda item:	7.3.1

Document for:	Approval

Contact:	Fangyuan Zhu< zhufangyuan@huawei.com >

Abstract: This document provides some updates for the use case 5.20.

1. Introduction

The aim of this contribution is to update the use case 5.20.

2. Reason for Change

According to the defintions of  TS 23.501 V19.2.0, the Mobile Base Station Relay(MBSR) consists of IAB-UE and gNB-DU. The IAB-UE function behaves as a UE.

TS 23.501 V19.2.0 3.1 Definitions Mobile Base Station Relay: A mobile base station acts as a relay between a UE and the 5G network. Such mobile base station relay can for example be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle). See description of TS 22.261 [2]. A mobile Base Station Relay is supported in 5GS with the IAB-architecture with mobility as specified in clause 5.35A and that described in TS 38.401 [42]. 5.35 Support for Integrated access and backhaul (IAB) 5.35.1 IAB architecture and functional entities ….. Figure 5.35.1-1 shows the IAB reference architecture with two backhaul hops, when connected to 5GC. Figure 5.35.1-1: IAB architecture for 5GS The IAB-UE function behaves as a UE and reuses UE procedures to connect to: - the gNB-DU on a parent IAB-node or IAB-donor for access and backhauling; - the gNB-CU on the IAB-donor via RRC for control of the access and backhaul link; - 5GC, e.g. AMF, via NAS; - OAM system via a PDU session or PDN connection (based on implementation). …. 5.35A Support for Mobile Base Station Relay (MBSR) 5.35A.1 General The MBSR uses the IAB architecture as defined in clause 5.35 and operates as an IAB node (with an IAB-UE and gNB-DU) with mobility when integrated with the serving PLMN. The architecture described in clause 5.35 applies unless specific handling is specified in clause 5.35A. Additionally, the following limitations apply to the MBSR: - the MBSR has a single hop to the IAB-donor node; - NR Uu is used for the radio link between a MBSR and served UEs and between MBSR and IAB-donor node. Regulatory requirements (e.g. emergency services, priority services) are supported when UEs access 5GS via a MBSR. LCS framework as defined in TS 23.273 [87] is used for providing the location service to the served UEs, with additional enhancements described in clause 5.35A.5.

According to the approved scope of FS_5GSAT_Ph4(see SP-240495), this study focuses on multi-orbits satellite access for multiple services and simultaneous UE connections with multi-orbits are out of scope.

# SP-240495 # 4 Objective The objectives of this study are the following: - to identify the gaps and potential new service requirements related to enhanced support for emergency communications and mission critical services using satellite access - regulatory requirements if applicable should be considered e.g., geographical area aspect - to study new use cases and identify new potential requirements for 5G system, related to: - multi-orbits satellite access for multiple services Note: Simultaneous UE connections with multi-orbits are out of scope - ability to notify the user that a mobile terminated communication (e.g. MMTel call) had failed when the UE was unreachable in satellite access. - to identify the gaps and potential service requirements to support IMS voice calls using GEO satellite access - regulatory requirements if applicable should be considered. - to identify the gaps and potential updates of KPIs related to satellite access

Obervations: The simultaneous MBSR(MBSR-UE) connections with multi-orbits are out of scope.

However, the current descripitons of use case 5.20 are focusing on the simultaneous connectivity to multi orbit satellite for MBSR. And the following Editor’s note is captured in this use case.

Editor’s note: whether the use of simultaneous connectivity to multi orbit satellite for MBSR is within the scope of the FS_5GSAT_Ph4 is FFS

Proposal#1:it is proposed to update Use Case 5.20, in order to properly fit with multi-orbits satellite access according to the approved scope of FS_5GSAT_Ph4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v1.0.0.

* * * First Change * * * *

5.20	Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR)

5.20.1	Description

Satellite access is a communication technique for provisioning of connectivity in rural and remote areas not covered by terrestrial access networks. Satellite access could be provided through satellites located in various orbits e.g. GEO, MEO, LEO. Each of these orbits has its own specificities related to coverage, data rates, capacity and latency. Similarly different services require different access network capabilities. In line with this, services requiring low latency and high data rates would optimally be served via LEO satellite access while delay tolerant services (e.g. txt, messaging) could be routed through a GEO satellite access. Combining multi-orbit satellite access links (LEO/MEO/GEO) could be of interest in cases when a single satellite access cannot meet the prescribed requirements (e.g., capacity, latency). Specific characteristic of each satellite access needs to be taken into account when combining it with another backhaul link type in order to achieve required QoS of each user within a group of users (aggregate QoS).

Figure 5.20.1-1 shows an example of multi-orbit satellite access where access  links can be established through GEO and NGSO satellite constellation. Usage of multi-orbit satellite accesses serves a mobile base station relay (MBSR) located on the airplane (or possibly on the ground on a remote location).

Figure 5.20.1: Illustration of multi-orbit satellite use-case exploiting GEO  and NGSO satellite access links serving mobile base station relay (MBSR) on board satellite. ISL indicate intersatellite links.

5.20.2	Pre-conditions

MLK is an airline company known for its high end on-board services. MLK takes good care of its passenger satisfaction and intends to upgrade its on-board entertainment services with on-line messaging, internet access, video streaming and gaming. MLK is looking for connectivity solutions that will enable the upgrade of its on-board entertainment. Cost efficiency of the connectivity solution is of great importance for MLK in order to remain competitive on the tight airline market.

5.20.3	Service Flows

A satellite operator SAOP having GEO and NGSO satellites in its satellite fleet offers satellite access connectivity solutions for the airplanes in MLK fleet anywhere around the globe. SAOP offers the most cost efficient solution for MLK as the different service types e.g., messaging, internet, video/gaming, will transfer their traffic over satellite access links optimal for the selected service e.g., GEO, MEO, LEO, respectively.

While on the ground the mobile base station relay (MBSR) mounted on an airplane is connected to the 5G core through a terrestrial mobile access network. The passengers on board the airplane acquire wide variety of services (latency sensitive, latency tolerant, messaging, video or gaming) through the terrestrial mobile access network and the MBSR.

After the take-off, the mobile base station relay mounted on the airplane loses the connectivity through terrestrial mobile access and switches over to satellite access through multiple satellite orbits i.e. LEO, MEO and/or GEO satellites. Initially, the MBSR is connected to the network through LEO satellite links and the passengers on board the airplane can enjoy on-line gaming service. Then due to mobility and out of coverage of the LEO, the MBSR is switched to GEO satellite links and passengers can enjoy messaging service. Satellite access through distinct orbits have different properties with respect to latency, data rate, spectrum, coverage and availability of the satellites in different orbits. 5G system is capable of differentiating traffic among various service types of the airplane passengers. For example, 5G system will route latency tolerant services’ traffic (e.g., messaging) through GEO satellite access, latency sensitive services’ traffic (e.g., mission critical communication, on-line gaming gaming) carried via a mobile base station relay through LEO satellite access and video traffic  carried via a mobile base station relay on demand through MEO satellite access.

5.20.4	Post-conditions

MLK improved offer of their services in a cost efficient manner. MLK is able to provide wide range of services (latency sensitive, latency tolerant, high data rate and high reliability) by exploiting multiple satellite access paths.

5.20.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] specifies the requirements for satellite backhauling i.e. that the 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g., on the latency and bandwidth of the specific backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

The above listed requirement are focused on usage of a single satellite orbit. The cases of possible usage of multi-orbit satellite connectivity are not explicitly considered.

5.20.6	Potential New Requirements needed to support the use case

[PR 5.20.6-001] The 5G system with satellite access supporting connectivity over multiple satellite orbits shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the multi-orbit satellite accesses (e.g., over GEO, MEO, LEO) to provision required aggregated QoS for traffic relayed via a mobile base station relay.

[PR 5.20.6-002] The 5G system with satellite access supporting connectivity over multiple satellite orbits shall be able to collect and distinguish charging information related to the multi-orbit satellite access  traffic carried via a mobile base station relay.

* * * End of Changes * * * *