3GPP TSG-SA WG1 Meeting #109	S1-250233

Athens, Greece, 17-21 February 2025

Title:	Energy control support on slice level

Agenda Item:	8.1.1 System and Operational Aspects

Source:	IIT Bombay

Contact:	Preksha Shah (30005100@iitb.ac.in)

Abstract: This document proposes a use case along with requirements for energy control support on slice level.

---------- Use Case template ----------

x.1	Use case on supporting energy control support on slice level

### x.1.1	Description

Mobile networks consume significant amounts of energy, contributing to greenhouse gas emissions and climate change. Prioritizing network energy efficiency is paramount. Recent ongoing work in 3GPP underlines energy awareness in mobile networks by monitoring energy consumption and supporting the collection of charging information on various granularities, one of which is per network slice. Such monitoring provides valuable information for optimizing network operations and minimizing expenses. Efficient utilization of this information has the potential to make significantly more energy and cost-sensitive decisions. This allows for an efficient strategic approach to network resource allocation and ultimately leads to substantial savings and sustainability.

This use case will describe a scenario in which upon request the mobile network operator (MNO) deploys private slice(s) for 3rd party to consume services. In this scenario, the 3rd party (service consumer) for example, an Industry, Institute, small-scale business, or start-up desires to curtail its expenditures associated with its private slice services. Hence, the 3rd party is concerned about the access-related energy use of its private slice. This use case demonstrates by providing the 3rd party the ability: to define an upper bound on the aggregate quantity of energy consumption for its private slice(s); to request means to limit the energy consumption of its private slice(s), the 6G supports sustainability goals.

### x.1.2	Pre-conditions

The 6G system of MNO (service provider) supports energy information exposure to 3rd parties for their private slices based on the agreement.

### x.1.3	Service Flows

An MNO deploys a private 6G network slice for an Industry ‘A’ to support its internal services like Industrial Internet of things (IIoT), predictive maintenance, file sharing, email, cloud computing, virtual private network (VPN), security services, etc. Industry ‘A’ is a small-scale industry and has budget limitations. Thus, it wants to reduce both the energy consumption and the charges associated with its private slice. Industry ‘A’ decides to define a private slice energy credit control limit that sets an upper bound on the aggregate quantity of energy consumption of its private slice. When this threshold is exceeded, the operator exposes this event to Industry ‘A’. Industry ‘A’ asks the MNO to use gating and preemption to manage service access and prioritization.

Figure 1: Enforcement of energy credit limit on per slices basis for an Industry

### x.1.4	Post-conditions

The 3rd party is able to perform private slice energy credit control and reduce its operational cost.

x.1.5	Existing features partly or fully covering the use case functionality

Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity.

NOTE 1: Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party.

NOTE 2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc.

NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

The 5G core network shall support collection of all charging information on either a network or a slice basis.

The 5G core network shall support collection of charging information based on the slice that the UE accesses.

Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a means to associate energy consumption information with charging information based on subscription policies for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services without QoS criteria.

NOTE 1: The result of the credit control is not specified by this requirement.

NOTE 2:	Credit control [49] compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed.

Subject to operator’s policy, the 5G system shall support a means to define subscription policies and means to enforce the policy that define a maximum energy consumption (i.e. quantity of energy for a specified period of time) for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit).

Based on operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party.

NOTE 1: Energy consumption information can include ratio of renewable energy and carbon emission information when available. The reporting period could be set, e.g., on monthly or yearly basis and can vary based on location.

NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

### x.1.6	Potential New Requirements needed to support the use case

[PR x.6-001] Subject to operator’s policy and agreement with 3rd party, the 6G system shall support policies that define a maximum slice energy credit limit.

NOTE 1: The definition of slice energy credit limit is FFS.

[PR x.6-002] Subject to operator’s policy and agreement with 3rd party, the 6G system shall support a mechanism to perform slice energy credit limit control.

NOTE 1: The result of the credit control is not specified by this requirement.