3GPP TSG-SA WG1 Meeting #109	S1-250236

Athens, Greece, 17-21 February 2025

Title:	Use case on handling massive emergency signalling

Agenda Item:	8.1.1 System and Operational Aspects

Source:	IIT Bombay

Contact:	Rashmi Yadav (30005709@iitb.ac.in)

Abstract: This document proposes a use case along with requirements for handling massive emergency signalling.

---------- Use Case template ----------

## x.1	Use case on handling massive emergency signalling

## x.1.1	Description

Massive emergency scenarios, such as large-scale industrial accidents, public safety events, or natural disasters, trigger an unprecedented surge in network traffic. An effective response to such incidents may rely on the usage of a diverse range of network devices and entities including IoT sensors, industrial automation systems, drones, staff personnel, and various communication units. During such incidents, a large number of devices attempt to establish or maintain network connections simultaneously, causing a sudden increase in connection attempts. Influx of simultaneous connection requests from a massive number of 6G users in an emergency situation results in a significant rise in UE signalling (related to registration, authentication, and session establishment). Due to these simultaneous requests, a signalling storm may occur in the network overwhelming the network's control plane. Since the network control plane is responsible for functionalities such as device registration, authentication, session establishment, resource allocation, and mobility management, a signalling storm or control plane congestion can cause delay in providing critical services to massive number of users such as establishment of data sessions.

To handle such scenarios, it may be beneficial to adopt a more scalable and flexible approach to UE signalling. In this context, a mechanism that treats (a subset of) UE signalling as data, to be handled outside the network control plane, may help. Such an approach may prevent control plane exhaustion and mitigate the risks associated with delayed emergency handling for massive connectivity scenarios. It should be noted that a subset of UE signalling, e.g., the registration and authentication, is not directly associated with data session setup. However, today these are also handled by the network control plane, which may adversely impact the network control plane’s ability to quickly setup the data sessions for massive number of users, especially in emergency scenarios.

Figure 1 illustrates the handling of UE signalling for massive emergency scenario. In the existing network, UE signalling terminates at the network control plane and hence, in massive emergency scenarios, the control plane may get overloaded (shown through the red lines), which may affect the user service delivery especially for ultra-time sensitive applications. However, termination of UE signalling independent of control plane, as shown on the right side of Figure 1, facilitates the ultra-low latency signalling for such massive emergency network services.

Figure 1. Handling of control (UE) signalling for massive emergency scenarios .

### x.1.2	Pre-conditions

### A large organization with operations spread across multiple locations relies heavily on the 6G system to support massive subscriptions, including IoT devices, environmental sensors, and communication tools for its workforce. The control room serves as the central hub, interacting with these devices through the 6G system to monitor operations and issue control commands. During normal conditions, routine signalling for data transmission and device synchronization is efficiently managed.

### However, a sudden emergency at one of the units of the organization escalates into a critical situation. Thousands of IoT devices and sensors simultaneously attempt to establish connections with the control room to report abnormal conditions. Organization staff from multiple locations also seek immediate communication for emergency instructions. This surge in UE signalling (registration, authentication, session establishment requests) overwhelms the control plane, creating congestion in the network control plane (within the 6G system). As a result, delays in data session setup prevent timely reporting of critical sensor data and the dissemination of corrective commands from the control room, posing serious risks to various organization operations and its safety.

### x.1.3	Service Flows

### Considering such massive connectivity delay issues, Organization A subscribes to the "Massive Emergency Services" in the 6G system, where signalling takes place similar to a data flow for all the UEs in its operational region. The pre-established signalling session allows a UE to exchange signalling with the 6G network as soon as it is needed. Periodic updates and device status messages are seamlessly handled through this signalling session, ensuring readiness for emergencies.

### During an emergency, UEs (as shown in Figure 2) immediately attempt to connect the control room to update the about the emergency. Since the organization has already subscribed to the Massive Emergency Service package, UEs always have signalling session “ON” which is terminating in the network at “Massive emergency signalling service function”. During emergency, UEs can immediately connect to the network for session establishment and sessions are established quickly with the help of ‘massive emergency signalling service function’. This pre-established signalling session avoids the congestion in the control plane and therefore ensures low-latency UE signalling even during massive connectivity scenarios.

Figure 2. Handling of massive emergency signalling in any organization which has subscription package of “Massive Emergency Services”.

### x.1.4	Post-conditions

Massive emergency scenario was controlled timely by the control room without any additional delay in session establishment (service providing) for its massive number of users in the 6G network.

x.1.5       Existing features partly or fully covering the use case functionality

TS 22.261, clause 6.4.2.1 on resource efficiency include the following requirement:

- The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g. 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	type of mobility support;

-	communication pattern (e.g. send-only, frequent or infrequent);

-	characteristics of payload (e.g. small or large size data payload);

-	characteristics of application (e.g. provisioning operation, normal data transfer);

-	UE location;

-	timing pattern of data transfer (e.g. real time or non-delay sensitive).

TS 22.261, clause “6.4.2.4 Efficient control plane” contains the following requirement

- The 5G system shall minimize the signalling that is required prior to user data transmission.

NOTE:	The amount of signalling overhead may vary based on the amount of data to be transmitted, even for the same UE.

TS 22.261, clause “6.8 Dynamic policy control” contains the following requirement

- The 5G system shall support optimised signalling for prioritised users and traffic where such signalling is prioritized over other signalling traffic.

### x.1.6	Potential New Requirements needed to support the use case

[PR x.6-001] The 6G system shall support UE signalling as data (signalling transfer as data transfer) for handling massive emergency signalling.

Note: UE signalling signifies the signalling messages exchanged between the UE and the network control plane currently.