3GPP TSG SA WG 1 Meeting #109 	S1-250241

Athens, Greece, 17-21 February 2025

Title:	Use Case on provision of Emergency services in 6G Networks

Agenda item:	8.1.1

Source:	SK Telecom

Contact:	DongJin Lee <dongjin@sk.com>

SeongJun Lee <seoul.lee@sk.com>

Abstract: This document proposes the provision of robust and reliable emergency services for non-IMS users in 6G networks. While IMS-based emergency services are well-defined and widely implemented, there is a critical need to ensure that users who do not utilize IMS can also access emergency services seamlessly. By integrating support for non-IMS emergency communications, 6G networks can enhance public safety, provide comprehensive coverage, and improve the overall reliability and responsiveness of emergency services. This use case outlines the necessary pre-conditions, service flows, post-conditions, existing features, and potential new requirements to achieve this goal, ultimately contributing to a more inclusive and resilient emergency communication system.

1. Introduction

As the landscape of wireless communication continues to evolve, the primary service usage of most users has expanded beyond traditional voice calls to include diverse applications such as data services, mVoIP, and social networking services (SNS). This shift necessitates a corresponding evolution in network capabilities to ensure that emergency services are accessible and reliable across all types of communication services. While IMS-based emergency services are well-defined and widely implemented, there is a growing need to ensure that users who do not utilize IMS can also access emergency services seamlessly.

This document proposes the provision of robust and reliable emergency services for non-IMS users in 6G networks. By integrating support for non-IMS emergency communications, 6G networks can enhance public safety, provide comprehensive coverage, and improve the overall reliability and responsiveness of emergency services. This use case outlines the necessary pre-conditions, service flows, post-conditions, existing features, and potential new requirements to achieve this goal, ultimately contributing to a more inclusive and resilient emergency communication system.

2. Reason for Change

The current 5G networks primarily focus on providing emergency services through IMS-based protocols. However, with the increasing diversity in communication methods and the growing number of users relying on non-IMS services, there is a critical need to extend emergency service capabilities to non-IMS users. The shift in user behavior from traditional voice calls to data services, mVoIP, and SNS necessitates a comprehensive approach to emergency service provision that includes all types of communication services.

Without the integration of emergency services for non-IMS users, a significant portion of the user base may be left without access to critical support during emergencies. This gap in service provision can lead to severe consequences, including delayed response times and inadequate assistance in life-threatening situations. Therefore, it is imperative to develop standardized protocols and procedures that ensure all users, regardless of their service type, can access emergency services reliably and efficiently.

By addressing this need, 6G networks can enhance public safety, provide inclusive service coverage, and improve the overall reliability and responsiveness of emergency communications. This change will ensure that the network can dynamically adapt to varying conditions, optimize resource allocation, and maintain high levels of service quality and reliability. Ultimately, the integration of emergency services for non-IMS users will contribute to the realization of a more resilient and autonomous network capable of meeting the diverse needs of modern communication users.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change (new) * * * *

5.x	Use Case on provision of Emergency services in 6G Networks

5.x.1	Description

This use case aims to enable 6G network services to provide robust and reliable emergency services for non-IMS users. While IMS-based emergency services are well-defined and widely implemented, there is a growing need to ensure that users who do not utilize IMS can also access emergency services seamlessly. This capability will enhance public safety by ensuring that all users, regardless of their service type, can reach emergency services in critical situations. By integrating emergency service support for non-IMS users(e.g., robot, machine, IoT device, cellular phone), 6G networks can provide comprehensive coverage and improve the overall reliability and responsiveness of emergency communications.

As the primary service usage of most wireless communication users has expanded from traditional voice calls to diverse applications such as data services, mVoIP, and social networking services (SNS), it is essential to define standardized call processing procedures that accommodate these varied communication methods. The evolution of user behavior necessitates a corresponding evolution in network capabilities to ensure that emergency services are accessible and reliable across all types of communication services. This includes not only traditional voice calls but also data-driven and application-based communications.

5.x.2	Pre-conditions

In region A, Operators B and C provide 6G network services.

The user's Device D is subscribed to Operator B.

5.x.3	Service Flows

1. Device D is currently receiving services from network B.

2. Due to a failure in network B, Device D can no longer receive normal services through network B.

3. Device D recognizes that network C is available in the region A.

4. Device D requests emergency services from network C.

5. Network C provides data or call services based on the type of emergency service request from Device D. Since network C cannot verify the integrity of Device D, data services are allowed on a limited basis.

6. After a certain period, Device D's connection may be terminated at the discretion of network C.

5.x.4	Post-conditions

The temporary access granted to non-IMS users in network C without roaming agreements with network B is monitored and managed to ensure network stability, with the possibility of revoking Device D's access based on network C’s conditions.

Network C should ensure that providing emergency services to Device D does not affect the services of existing subscribers.

Device D can use emergency services (e.g., Data, Voice, SNS, mVoIP) through network C, to which it is not subscribed.

5.x.5	Existing features partly or fully covering the use case functionality

While some emergency service functions provided in current 5G networks may partially cover this use case, they are primarily designed for IMS-based users and do not fully support the requirements of non-IMS users. These existing functions include basic emergency call routing and location-based services, but they lack the comprehensive support needed for non-IMS emergency communications.

While there are existing features and frameworks that provide some level of support for emergency services(IMS call), they are not sufficient to meet the comprehensive requirements of providing emergency services to non-IMS users in 6G networks. Significant enhancements and new capabilities are needed to achieve the desired level of reliability, responsiveness, and coverage.

11.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 6G network shall support temporary data services for emergency communications when a device connects to an unregistered PLMN. This capability is essential to ensure that users can access emergency services even when they are outside their home network or in areas where their usual network is unavailable.