3GPP TSG SA WG 1 Meeting #109 	S1-250260

Athens, Greece, 17 – 21 Feb 2025	(revision of S1-25xxxx)

Source:	Nokia

pCR Title:	pCR on TR 22.883 consolidation, conclusion and recommendations

Draft Spec:	3GPP TR 22.883-100

Agenda item:	7.2 (FS_EnergyServ_Ph2)

Document for:	Approval

Contact:	Laurent-Walter Goix (laurent-walter.goix@nokia.com)

Abstract: This document addresses the consolidation of potential requirements not yet consolidated, as well as conclusions and recommendations to be added to TR 22.883.

1. Introduction

The following changes are proposed with respect to consolidation categories:

- Renaming existing category “Exposure of information about network energy-related characteristics”

- Adding a new category dedicated to the “Energy-related characteristics as service criteria”

The categories are intended to potentially map with cl 6.15a “Energy Efficiency as a Service Criteria” of TS 22.261 whenever relevant. Some categories may require the creation of new sections under that clause.

Conclusions of the study work and recommendations for normative work are also proposed.

2. Reason for Change

TR 22.883 is expected to be sent for approval to SA at the end of SA1#109 and thus needs to include all agreed CPRs as well as the study conclusions and recommendations.

3. Conclusions

In the final version of this contribution, all agreed CPRs will be incorporated into this pCR and the conclusions be adapted to the agreements.

4. Proposal

It is proposed to agree the changes in 3GPP TR 22.883-100.

FIRST CHANGE

# 6	Consolidated Potential Requirements and KPIs

## 6.1	Consolidated potential requirements

### 6.1.1	Exposure of information about network energy-related characteristics

This clause consolidates potential requirements from clauses 5.3, 5.5, 5.6, 5.8 and 5.9 related to the exposure of information about network energy-related characteristics (including CO2e emissions) in general, and attributed to users at subscriber, UE or service data flow granularity.

Table 6.1.1-1 – Exposure of information about network energy-related characteristics Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1.1-1 | Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to expose to authorized 3rd parties the carbon equivalent emissions resulting from the use of the communication service, related to one or more specific UEs of home subscribers (e.g. fleet of vehicles, IoT devices, company phones etc), over a specific time period (e.g. month etc). NOTE 1: This requirement does not apply to the use of the communication service when roaming. NOTE 2: Exposing carbon equivalent emissions at subscriber level can rely on but does not imply any real-time collection, monitoring or exact subscriber-specific calculations. It can be provided by means of statistics, computations and estimates as accurate as possible, e.g. based on subscriber’s data volume and operator’s carbon intensity to provide the communication service. In particular, it is expected such information to be provided on demand, not more granularly than on a per-day basis. NOTE 3: The exposed carbon equivalent emissions can further include information to allow to compare subscribers of e.g. the same PLMN, NPN or slice over the same time period. Such information can take the form of e.g. a range (i.e. minimum and maximum) or average of carbon equivalent emissions per subscriber, or the form of a category, class or score of carbon emissions (e.g. A to F, 1 to 5). | PR.5.6.6-1 | NOTE 2 may apply to other CPRs. |
| CPR 6.1.1-2 | Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to expose to an authorized 3rd party the current and expected energy related characteristics of the network resources used in serving a UE. NOTE: This information can be used by applications to schedule non time-critical data transfer to or from the UE. | PR.5.8.6-1 | Reworded for clarity to relate to UE (with user consent) and to generalize to any authorized 3rd party |

### 6.1.2	Service adjustments based on energy-related characteristics

This clause consolidates potential requirements from clauses 5.1, 5.2, 5.4, 5.7 and 5.9 related to the adjustment of the communication service based on energy-related characteristics in general, including energy rationing situations or changes of energy supply mix, as well as different related policies.

Table 6.1.2-1 – Service adjustments based on energy-related characteristics Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1.2-1 | Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable the operator to configure policies to reduce experienced QoS considering energy constraints. NOTE: This does not imply exact measurement of energy information at the per-flow level. | PR 5.2.6-2 | Reworded for clarity, also to refer to operator’s policy and to refocus on experienced QoS. Added NOTE. |

Editor's Note: The potential requirements PR.5.1.6-1, PR.5.1.6-2, PR 5.2.6-1, PR.5.3.6-1, PR.5.4.6-1, PR.5.5.6-1, PR.5.7.6-1, PR.5.7.6-2 will be considered for consolidation once the ENs are resolved.

### 6.1.3	Charging-related aspects

This clause consolidates potential requirements from clause 5.7 related to charging when performance of services with QoS criteria is degraded due to network energy saving actions.

Table 6.1.3-1 – Charging-related aspects Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

Editor's Note: The potential requirement PR.5.7.6-3 will be considered for consolidation once the ENs are resolved.

### 6.1.4	Energy-related characteristics as service criteria

This clause consolidates potential requirements from clause 5.9 leveraging energy-related characteristics as service criteria, for example for the network to provide a set of target UEs.

Table 6.1.4-1 – Exposure of energy-related characteristics as service criteria Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.2	Consolidated potential KPIs

The 5G network shall be able to support the following KPIs with respect to energy-related characteristics.

| Use Cases | Characteristic parameter (KPI) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Information | System Granularity | Time Granularity | Time Range | Obtainment (informative) |
| Subscriber’s carbon footprint information exposure to 3rd party (NOTE 1) | Carbon equivalent emissions (e.g. in kg of CO2e) | UE | Day | Month | Calculation (e.g. average or statistical model) based on aggregation/attribution of network energy consumption, carbon intensity and/or renewable energy ratio. |
| Energy Consumption and CO2e transparency in the end-to-end service chain (NOTE 2) |  | Service data flow | Minute | Session duration |  |
| Energy Consumption and CO2e transparency in the end-to-end service chain (NOTE 2) | Energy consumption (e.g. in kWh) | Service data flow | Minute | Session duration | Calculation of network energy consumption based on attribution e.g. measurements, service settings, external information and models. |
| NOTE 1: Such indication should be meaningful to the user to raise awareness on a monthly reporting basis or to incentivize the service consumer (out of scope of 3GPP). NOTE 2: Frequency of such indications should be meaningful to the user during the communication service (e.g. video streaming application). |  |  |  |  |  |

# 7	Conclusion and recommendations

This document analyses a number of new use cases to support energy efficiency as service criteria, in particular targeting:

- The consideration of network CO2e emissions as part of energy-related characteristics to be exposed by the network, at UE and service flow granularities, and to be used as criteria (in relation to the energy supply mix) for potential adjustments of the communication service (e.g. user plane path);

- The consideration of limited energy availability (e.g. energy rationing situations in the network, UE battery saving) to potentially impact the communication service provided by the network;

- The exposure towards UEs of current and future network energy-related characteristics, and their potential impact on the communication service (service availability characteristics), as well as the provision of policies for UEs to act upon these characteristics;

- The user consent and preferences to allow the network to affect the user service experience, including for services with QoS criteria, when performing energy saving actions – with proper charging notification.

The resulting potential consolidated requirements have been captured in clause 6.1.

Clause 6.2 contains consolidated KPIs clarifying the expected levels of granularity and frequency of information about energy-related characteristics needed to address the studied use cases.

It is recommended to proceed with normative work in TS 22.261 based on the identified consolidated requirements and KPIs.

It is further recommended to address some considerations captured as notes in the consolidated requirements (e.g. on attribution, frequency and accuracy of energy-related characteristics, on targeting net energy savings) into some common description clause in TS 22.261 during normative work.

NOTE:	The present document will not be revised to align with normative specification.

END OF CHANGES