3GPP TSG-SA WG1 Meeting #109	S1-250296

Athens, Greece, 17-21 February 2025										(revision of S1-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0039 | rev | - | Current version: | 20.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleaning gap analysis comments in the TR 22.989, related to missing normative requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Deletion of text ‘CR is needed’ used in the gap analysis comments of some refined use cases and for the added new ones, to indicate that normative requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deletion and amendment of the comments were performed in the following use cases: a) multi-train voice communication b) call forwarding for Ad hoc Group calls c) take-over by the train driver of an on-going call outside the train cabin d) status of a functional identity e) authorisations and/or change of privileges based on FRMCS user’s identities/talker status during Ad hoc Group calls f) arbitration related use cases g) presence of FRMCS User identity/functional identity |  |  |  |  |  |  |  |  |
| Take |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The version of the TR will keep useless content that was only useful for intermediate steps of adjustments/additions of use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.5, 6.2.5.5, 6.2.6.5, 6.2.8.5, 6.22.5.5, 6.22.6.5, 9.3.10.5, 9.9.3a.5, 9.16.3.5, 9.18.2.5, 9.18.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 6.2.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.3-001] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), a Driver or a Ground FRMCS User shall be able to put the voice communication on hold. The voice communication between the remaining users shall not be affected by a Driver or Ground FRMCS User putting the voice communication on hold. | A | 22.280 | 5.4.1, 5.4.2, 5.1.5 R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is considered sufficient to mimic the desired behaviour. Put a multi-train voice communication on hold based on Ad hoc Group Calls is not covered. |
| [R-6.2.3-002] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), a Driver shall be able to leave the voice communication at any time. After a Driver has left the communication, the remaining users are informed. | A | 22.280 | TS 22.280: R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is considered sufficient to mimic the desired behaviour. All other use cases of leaving, without criterion evaluation, when using Ad hoc Group calls, need queueing mechanisms which are not defined yet for Ad hoc Group Calls. |
| [R-6.2.3-003] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s) the Driver is able to terminate the communication based on configuration (authorised FRMCS User). | A | 22.179 22.280 | TS 22.179: 6.2.3.4, 6.2.3.5, 6.2.4 TS 22.280: R-6.4.4-003, R-6.4.4-004, [R-6.15.5.2-003] based on Ad hoc Group Calls. |
| [R-6.2.3-005] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System terminates the voice communication if the last Driver has left (although (multiple) Ground FRMCS Users are still active in the communication). All involved users are informed. | A | 22.280 22.179 | 22.179 6.2.4 008 22.280 6.4.9 001 CR 22.280 0048 rev1: R-6.1-006, R-6.4.9-001 [R-6.15.5.2-003] & [R-6.15.5.2-004] based on Ad hoc Group Calls |
| [R-6.2.3-006] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), a Ground FRMCS User shall be able to leave the communication. After a Ground FRMCS User has left the communication, the remaining users are informed. | A | 22.280 | 6.4.4 002; 6.4.5 001; 5.1.5 003-008 R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is considered sufficient to mimic the desired behaviour. Same comment as for [R-6.2.3-002], also applicable to controllers. |
| [R-6.2.3-007] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), authorised Controller shall be able to terminate the communication. All involved users are informed. | A | 22.280 | [R-6.15.5.2-003]. Termination of Ad hoc Group Calls by an authorised user &[R-6.15.5.2-005]. The Ad hoc Group shall not persist, when the Ad hoc Group Communication has terminated. |

Next change

#### 6.2.5.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.5-001] | The FRMCS System shall enable an authorised FRMCS User to merge two multi-train voice communications, based on the same criterion (e.g., location of the initiator). | A | 22.280 | Not covered. for an authorised user to merge Ad hoc group communications, using a new criterion, based on the union of Ad hoc Group IDs of both merged communications. |
| [R-6.2.5-002] | The FRMCS system shall record, as metadata, the timestamp of merging of the two multi-train communications | A | 22.280 | Partially covered in [R-6.15.4-004]. to add the relevant metadata for merged communications. |
| [R-6.2.5-003] | The FRMCS shall notify the participants of the two initial multi-train communications for the new merged voice communication. | A | 22.280 | Not covered.. Merging notification is missing for Ad hoc Group Calls |
| [R-6.2.5-004] | The FRMCS system shall enable any participant of a merged multi-train voice communication to keep the authorisations of the initial multi-train voice communications | A | 22.280 | Not covered.. Authorisations for merged communications based on Ad hoc Group calls are kept as prior to merging. |
| [R-6.2.5-005] | The FRMCS system shall enable a participant who is authorised to terminate at least one of the initial multi-train communications, to also terminate the merged multi-train voice communication. | A | 22.280 | Not covered.. Termination of merged multi-train communications for Ad hoc Group calls. |
| [R-6.2.5-006] | The FRMCS system shall enable any participant of a merged multi-train voice communication, to keep the floor control policy for multi-user talker control of the initial multi-train voice communications | A | 22.179 | Not covered.. Transfer of floor control policies to the merged multi-train communication for Ad hoc Group Calls. |
| [R-6.2.5-007] | The FRMCS system shall set the number of simultaneous talkers of the merged multi-train communication to the sum of the number of simultaneous talkers of the original communications, with multi-user talker control. | A | 22.179 | Not covered.. Addition of simultaneous talkers for Multi-user talker control for merged multi-train communication with Ad hoc Group Calls. |
| [R-6.2.5-008] | The FRMCS system shall enable the establishment of a merged multi-train voice communication with a set-up time, specified as NORMAL | A/T | N/A | See §12.10 |
| [R-6.2.5-009] | The FRMCS system shall enable the participants of a merged multi-train voice communication to address only the participants of their initial group, prior to merging, whenever needed. | A | 22.280 | Not covered. for addressing each separate group, without involving all the participants of the merged group. |

Next change

#### 6.2.6.5 	Potential requirements and gap analysis

Potential requirements derived from the gap analysis of §6.2.5.5 are also applicable for this use case. An additional one is:

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.6-001] | The FRMCS System shall enable an authorised FRMCS User to merge two multi-train voice communications, based on different criteria (e.g., initiator’s location, selected addressed area). | A | 22.280 | Not covered. for an authorised user to merge Ad hoc group communications, using different criteria (e.g., initiator’s location for one communication and selected addressed area for the other communication) |

Next change

#### 6.2.8.5 	Potential requirements and gap analysis

Potential requirements derived from the gap analysis of §6.2.6.5 are also applicable for this use case. An additional one is:

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.8-001] | The FRMCS System shall enable an authorised FRMCS User to merge two multi-train voice communications, based on Ad hoc Group Calls with same or different criteria, keeping unchanged the ‘original’ communications which are cancelled after the ‘merged’ communication is successfully established. | A | 22.280 | Not covered. for an authorised user to merge Ad hoc group communications, using same or different criteria (e.g. location criterion for both ‘original’ calls or location for one ‘original’ call and functional alias(es) for the ‘other ‘original’ call) |

Next change

#### 6.22.5.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.5-001] | The FRMCS System shall enable authorised FRMCS User to configure forwarding of incoming voice communications to another FRMCS user of the same FRMCS domain, based on different criteria | A/T | 22.280 22.179 | a) Call Forwarding is missing for Ad hoc Group calls, based on criteria. b) Authorisation of Call Forwarding for Ad hoc Group calls to another FRMCS User of the same FRMCS domain based on FRMCS user identity, functional identity, or subparts of functional identities is missing. NOTE: Resolution of functional alias activated by multiple users will be further elaborated. |
| [R-6.22.5-002] | The FRMCS System shall support the forwarding of incoming voice communications based on one of the following criteria: - All incoming communications are forwarded; - When FRMCS user is not reachable (e.g. out of coverage); - Busy (i.e., the FRMCS user is connected in another call); - When the FRMCS user is not answering the incoming call within the set (time) interval; - When the FRMCS user rejects the presented communication | A/T | 22.179 22.280 | a) All the forwarding conditions mentioned, are missing for Ad hoc Group calls based on criteria. Automatic and manual commencement modes have to be considered. b) Busy for Ad hoc Group call can be handled at application level, so out of scope of 3GPP. (e.g., based on arbitration rules) c) Forwarding of presented and rejected call by the user is missing for Ad hoc Group calls with manual commencement mode. |
| [R-6.22.5-003] | The FRMCS System shall be able to support configurable time or number of rings when the FRMCS User is not answering | A/T | 22.179 22.280 | Call Forwarding of Ad hoc Group call with manual commencement mode based on configurable time or number of rings for not answering, is missing. |
| [R-6.22.5-004] | The FRMCS System shall support forwarding of incoming voice communications for user-to-user and multi-user. | A/T | 22.280 22.179 | Call forwarding is missing for Ad hoc Group call. Same comment as for [R-6.22.5-001]. |
| [R-6.22.5-005] | The FRMCS System shall be aware of the busy condition of the targeted FRMCS user to forward the incoming voice communication to another FRMCS User. | A/T | 22.179 | Busy for Ad hoc Group call can be handled at application level, as per comment of [R-6.22.5-002]. |
| [R-6.22.5-006] | The FRMCS system shall provide information to the initial destination of the call that the incoming call has been successfully forwarded sending back the new FRMCS User’s identity or functional identity. | A/T | 22.280 22.179 | Notification to the initial destination about the call status forwarding and new FRMCS User’s Identity or functional identity is missing in Ad hoc Group calls. |

Next change

#### 6.22.6.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.6-001] | The FRMCS system shall allow a FRMCS user to login into multiple FRMCS devices, separately, by using different FRMCS User Identities (e.g., FRMCS User Identity_A, such as cabin device and FRMCS User Identity_B, such as handheld). | A | 22.280 | [R-5.10-001] & [R-5.10-002]. Partially covered. Clarification is missing about the authorised user who is using different MC Service IDs and Functional Alias(es) in each device. |
| [R-6.22.6-002] | The FRMCS system shall allow the FRMCS user to take-over a functional alias, when logged into multiple FRMCS devices, using separate FRMCS User Identities for each device. | A/T | 22.280 | [R-5.9a-009] Partially covered. Only authorised user is allowed to take-over a functional alias that cannot be shared. This capability needs to be extended to multiple devices, as well. |

Next change

#### 9.3.10.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.3.10-001] | The FRMCS System shall provide the status of the functional identities (e.g., functional identity not activated, ready for operation, transfer pending) to a FRMCS User, before attempting to activate it. | A/T | TS 22.280 | Not covered. |

Next change

#### 9.9.3a.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.9.3a-001] | When an FRMCS User performs MCX authentication and authorisation, the FRMCS System shall be able to trigger a change of communication privileges for that particular FRMCS User. | A | Partly covered (refer to communication privileges below) | Identification of the targeted user of the communication privilege status change: partly covered by MCX authentication and authorisation mechanism (MC ID only, no FA). Change of user profiles on MCX authentication and authorisation: covered MCX User Service Authorization is the function that validates whether or not a MCX user has the authority to access certain MCX services. |
| [R-9.9.3a -002] | When an FRMCS User activates or deactivates Functional Alias(es), the FRMCS System shall be able to trigger a change of communication privileges for that particular FRMCS User. | A | Partly covered (refer to communication privileges below) | Identification of the targeted user of the communication privilege status change: covered by FA mechanism (MC ID + list of FA(s)). Change of user profiles on FA activation or creation of new profiles for FA: not covered. |
| [R-9.9.3a -003] | When the floor is granted or released for a particular FRMCS User (including multi-talker floor control), the FRMCS System shall be able to trigger a change of communication privileges for all FRMCS Users affiliated to that group. | A | Partly covered | Identification of the targeted users of the communication privilege status change: Partly covered. Only MCPTT ID(s) and list of FA(s) of the requester(s) (floor control request & multi-talker) are known by the system. Change of user profiles when floor granted/released: not covered. |
| [R-9.9.3a -004] | The FRMCS System shall support Communication privilege as (1) authorisation to put a group voice communication on hold. | A | Covered | Put a group voice communication on hold: covered by the affiliation mechanism which is considered sufficient to mimic the desired behaviour. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status:not covered. |
| [R-9.9.3a -005] | The FRMCS System shall support Communication privilege as (2) authorisation to leave a group voice communication. | A | Covered | Leave a group voice communication: covered by the affiliation mechanism which is considered sufficient to mimic the desired behaviour. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status: not covered. |
| [R-9.9.3a-006] | The FRMCS System shall support Communication privilege as (3) authorisation to terminate a group voice communication. | A | Not covered | Terminate a group voice communication: strictly performed by the system. Authorisation mechanism: not covered. |
| [R-9.9.3a -007] | The FRMCS System shall support Communication privilege as (4) authorisation to merge Railway Emergency Alerts. | A | [Need analysis] | [Need analysis – finalize potential requirements for Merging of Railway Emergency Alerts first] |
| [R-9.9.3a -008] | The FRMCS System shall support Communication privilege as (5) authorisation to kick out another FRMCS User from a group voice communication. | A | Covered by 22.280 | Kick out another user: covered by remote change of affiliation (mandatory mode) 22.280 6.4.6.1. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status: not covered. |
| [R-9.9.3a -009] | The FRMCS System shall support Communication privilege as (6) authorisation to invite another FRMCS User to a group voice communication. | A | Covered by 22.280 | Invite a user to join: covered by remote change of affiliation (negotiated mode) 22.280 6.4.6.2. Authorisation mechanism: authorization of changing affiliation based on registration status by registration / deregistration to / from a functional identity or by talker status: not covered. |
| [R-9.9.3a-010] | The FRMCS System shall support Communication privilege as (7) authorisation to initiate private voice, video or data communication. | A | Covered by 22.280 | Initiate private voice, video or data communication: covered by MC private call request mechanism. Authorisation mechanism: covered by authorisation check based on user profile. |
| [R-9.9.3a -011] | The FRMCS System shall support Communication privilege as (8) authorisation to initiate group voice, video or data communication. | A | Covered by 22.280 | Initiate group voice, video or data communication: covered by group call request mechanism. Authorisation mechanism: covered by authorisation check based on user profile. |

Next change

#### 9.16.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.16.3-001] | When an incoming private communication is received by the FRMCS User, the FRMCS System shall be able to automatically accept the communication if arbitration rules are configured to do so (based on FRMCS User identities, registered functional identities or FRMCS Application types). | A | 22.280 | Partly covered by private call setup in automatic commencement mode (request for automatic commencement mode – automatic answering – is trigerred by the calling party, not by the System). In addition, configuration for automatic commencement mode is based on MC User ID only. Arbitration rules based on FRMCS Application types (i.e., MCPTT, MCData or MCVideo) is supported but not on Functional Alias(es). |
| [R-9.16.3-002] | When an incoming group communication is received by the FRMCS User, the FRMCS System shall be able to automatically accept the communication if arbitration rules are configured to do so (based on FRMCS User identities, registered functional identities or FRMCS Application types). | A | 22.280 | Covered by affiliation and functional alias mechanisms. |
| [R-9.16.3-003] | When the FRMCS User is using multiple FRMCS Equipment simultaneously, the FRMCS System shall be able to automatically accept an incoming private communication to the default FRMCS Equipment. | A | 22.280 | Covered by 5.10 Support for multiple devices (i.e., multiple MCX UEs) and [R-6.7.2-005] default device designated by the receiving user. Editor’s note on multiple devices support: default UE covered by stages 2&3? (didn’t find it) |
| [R-9.16.3-004] | The default FRMCS Equipment is configured by either the FRMCS User or by the FRMCS System Administrator. | A | 22.280 | 5.10 Support for multiple devices (i.e., multiple MCX UEs) and [R-6.7.2-005] & [R-6.7.2-006]: |
| [R-9.16.3-005] | When the FRMCS User is using multiple FRMCS Equipment simultaneously, the FRMCS System shall be able to automatically accept an incoming private communication to other FRMCS Equipment if communication to the default FRMCS Equipment fails to be established. | A | 22.280 | [R-6.7.2-006] |

Next change

#### 9.18.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.18.2-001] | The FRMCS System shall automatically set the status of the FRMCS Users based on the status of their MC Service ID, when not registered to a functional identity yet. | A/T | TS 22.280 | Availability of the MC Users based on the status of MC Service ID, automatically set by the system, is not defined. |
| [R-9.18.2-002] | The FRMCS System shall set the status of a MC Service ID to: - On-line, when successfully logged into the FRMCS system and not involved in any communication - Occupied when involved in communication - Offline, when not logged into the FRMCS system - Do not disturb (manual option) | A/T | TS 22.280 | Status of the MC service ID, based on conditions, is not defined. |
| [R-9.18.2-003] | The FRMCS system shall allow an authorized user to be aware of the status of the MC Service IDs of FRMCS users in a certain area. | A/T | TS 22.280 | Availability of the status of FRMCS users to authorised user, based on MC Service Id, is not defined. |

Next change

#### 9.18.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.18.3-001] | The FRMCS System shall allow displaying on the HMI, the status of all functional alias(es) the FRMCS user has activated. | A/T | TS 22.280 | Availability of the MC Users based on the status of functional alias(es), automatically set by the system, is not defined. |
| [R-9.18.3-002] | The FRMCS System shall allow an authorized user to display the status of the functional alias(es) of all the FRMCS users in a certain area. Status of activated functional alias(es) shall be: - Ready for operation, when the functional alias is activated. - Unavailable, when temporarily inactive - Offline, when functional alias is not activated | A/T | TS 22.280 | Status of activated functional alias(es) and update of the status when not used during a communication is not defined. |
| [R-9.18.3-003] | The FRMCS system shall allow an authorized user to be aware of the status of the FRMCS users in a certain area based on the status of their functional alias(es) | A/T | TS 22.280 | Availability of the status of FRMCS users to authorised user, based on functional alias(es), is not defined. |

End of changes