3GPP TSG-SA WG1 Meeting #109 	S1-250312

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	Siemens

New use case title:	New use case on Cooperating Mobile Robots

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.7

Document for:	Approval

Contact:	Michael Bahr (bahr et siemens dod com)

Abstract: This use case proposes to address Industrial Communication between collaborative robots over 6G networks. It is proposed to be added to the “Further Use Cases on Industry and Verticals” clause in the 6G TR 22.870.

---------- Use Case template ----------

## 11.x	Use Case on Cooperating Mobile Robots

### 11.x.1	Description

At the centre of this use case are autonomous robots with the ability to move, sense their environment, and perform a productive task collaboratively. These robots can communicate with one another, with other machines, and with nearby humans to perform individual tasks that contribute to a common cooperative objective. The purpose of communication is safety and cooperation, to enable a group of robots to perform tasks beyond their individual capabilities and to enable individual robots to perceive their environment beyond their local capabilities.

This use case focuses primarily on local ad hoc connectivity embedded in private networks. Scenarios may include industrial manufacturing campus, construction site, and smart living.

In this context, the network of the future is envisioned to enable local cooperation among robots, to support autonomous task-solving by the robots, to enhance the safety of human-machine interaction, and to ensure that only authorized machines and humans can participate in task solving.

Figure 11.x.1-1: Cooperating Mobile Robots Use Case, including two example scenarios

Scenarios within this use case family revolve around the need to complete a task that surpasses the capabilities of a single robot. The solution lies in robot cooperation, where multiple robots form a cluster to collaboratively accomplish the task. These robots utilize local ad hoc connectivity to establish a subnetwork for the following purposes:

- Coordinating and controlling their actions for successful task execution.

- Exchanging raw or processed sensor data.

- Sharing AI/ML models, weights, and federated learning.

- Ensuring safety for humans nearby in the event of an emergency.

A subnetwork is task-specific, spatially confined, and time-bounded. Its creation is initiated by a device, in this case, a robot. The subnetwork utilizes a portion of the communication resources from the private network within which it is embedded. Once the task is completed, the subnetwork is disbanded, and the communication resources are subsequently freed.

Cooperating Mobile Robots will be present in many environments. Several considered scenarios are mainly indoor – some closer to consumers with smaller areas (smart workshop, 11.x.3.6), others closer to industrial manufacturing with potentially larger areas (cooperative carrying 11.x.3.1, lot-size-1 production 11.x.3.2, automated industrial tasks 11.x.3.3). Some are outdoor within a local well-confined geographical area such as a factory yard (collaborative carrying 11.x.3.1), a field (autonomous farming 11.x.3.5), or a construction site (autonomous construction site 11.x.3.4). In these environments, line-of-sight links may be scarce for macro cells. Thus, femto- or pico-cells should be preferred for beam-based coverage. Propagation models used for network planning should include the appropriate multipath effects.

The majority of users are machines, with a comparatively smaller number of human users.

The following problems and challenges need to be addressed in this use case. The 6G system can be an important enabler for these:

- Understanding and addressing the communication requirements of machines in the future. This includes existing industrial automation systems, as well as the emerging type of autonomous embodied agents that encompasses industrial robots, service robots, and cobots, i.e., collaborative, cooperating, or cognizant robots. These robots interact with the physical world through sensors and actuators, utilize advanced machine learning for decision-making, and employ wireless connectivity to collaboratively perform complex tasks and adapt to changing environments.

- Using limited resources efficiently (materials, energy, and human effort). For a sustainable development, it is essential to further automate well-defined, structured, and repetitive tasks in the physical world. This will require advances in connectivity for smart tools and machines that will allow mobile robots to collaborate, thereby optimizing productivity, resource efficiency, and costs.

- Adapting to dynamic requirements of the market. In the future, the ability to address real-time fluctuations in demand and personalized customer specifications will be essential in industries such as manufacturing, transportation, construction, and logistics. A key advantage will be the ability to use machines in different configurations. Within a collaborative network, the potential of a single machine to leverage the capabilities of other machines amplifies the versatility of the fleet. Combined with enhanced automation, this allows for more agile responses to real-time market requirements.

- End-user access to custom manufacturing. Enhancing the capabilities of end-users, including tradespeople and individuals without technical expertise, to manufacture products such as mechanical components, electronic devices, and architectural structures, is a step towards self-sufficiency. As cooperating mobile robots make manufacturing processes more accessible and decentralized, goods can be produced as and when needed at the specific locations where they are needed. New possibilities for customization will emerge. This shift will transform business relationships and will lead to improved transparency and accountability in the manufacturing sector.

- Safe and trustworthy interactions with tools that can make decisions. As robots gain decision-making and collaboration capabilities, one major concern is the assurance of human safety and the establishment of trust when these autonomous machines operate in human-inhabited spaces.

Potential sustainability impacts of the use case are clustered by main topic / KVI. For each KVI, a selection of key values is provided together with some background information.

Energy: i.e., focus on the energy usage for the correct operation of robots, both on communication links, as well as in the data processing, data storage, and data collection aspects

-	[  to be decreased] Total energy usage (kWh) in communication dimensions for the cobots

-	[  to be decreased] Total energy usage (kWh) for data transfer (optimising packets, and data volume)

Selected background information:

-	Potential benefits: Resource efficiency: Functionalities may be provided by machines with less materials, energy, and waste generated

-	Potential areas of attention (risks to be mitigated): Energy is consumed and materials are used to manufacture, deploy, and operate cobots and associated services.

Materials/Waste:

-	[  to be increased] Life expectancy of robots: making better and longer-lasting hardware and/or software, easily upgradable, modular. Avoid having to replace the entire cobot because of failures in the 6G-related aspects

-	[  to be increased] # of virtualised functionalities: Preventing or avoiding material usage for dedicated hardware, when these features / or capabilities could be virtualised.

Selected background information:

-	Potential areas of attention (risks to be mitigated): The disposal of machines and devices results in increased electronic waste.

Safety:

-	[  to be decreased] # of injuries at work/level of severity of work-related injuries/perception of risk: Aimed at ensuring the system will be reliable and thus, no accidents will be produced because of failures in the communication links, or misinterpretation of data in the AI/ML algorithms/Sensing/Positioning/etc.

Selected background information:

-	Potential benefits: Safer work environment leading to less injuries.

Trustworthiness/Privacy/Security:

-	[  to be increased] Level of acceptance of cobots by humans: Ensuring that the appropriate privacy and security requirements are in place, that no personal data is at risk, that cobots will be resilient and responsive in case of hacking, etc. Overall, ensuring they are trustworthy and accepted.

-	[  to be decreased] # of data leaks/breaches/cyber attacks, with personal information compromised: Measuring the impact of the above KVI.

Selected background information:

-	Potential benefits: Increased autonomy by robots supporting people with disabilities.

-	Potential areas of attention (risks to be mitigated): People’s privacy may be breached by unauthorized use of robots’ and cobots’ sensors.

Productivity/Efficiency: Ensure that 6G is reliable and resilient, and so, no downtimes in a given production scenario are due to failures in the communication-related aspects.

-	[  to be decreased] Downtimes.

-	[  to be increased] Production KPIs (distinctive per factory).

Selected background information:

-	Potential benefits: Increased local and global productivity, cost efficiency and enhanced competitiveness from the use of the collaborative robots.

### 11.x.2	Pre-conditions

Private networks: It is assumed that the environment in which the network is deployed is dedicated to a specific purpose and to private use, e.g., manufacturing, construction, farming, living, and similar. Trade-offs among coverage, data rate, latency, reliability etc. can be tailored according to specific application needs inside the private, local networks.

Local networks: The collaborating mobile robots use case deploys local wireless networks in well-confined geographical areas such as factories, industrial sites, enterprises, construction sites, or fields. The local networks are primarily private networks, in several scenarios with functionality and data storage on-premise.

Subnetwork: support of (very) low latency and high reliability by peer-to-peer and mesh networking of co-located collaborative robots (especially in cooperative carrying scenario, 11.x.3.1). (direct device connections)

Connection to local network: Subnetworks usually have a (very) low latency / high reliability connection of one or multiple subnetwork nodes to the local wireless network in order to reach local edge nodes, local AI/ML compute server, and other centralized local functionalities.

Infrastructure option: support of (very) low latency and high reliability by 6G communication between collaborating manufacturing robots, between collaborating robots and functionalities located in the local 6G network such as local edge nodes, AI/ML compute servers, and other local centralized functionalities. (direct network connections)

Wide area connectivity: especially outdoor scenarios (autonomous farming 11.x.3.5, autonomous construction site 11.x.3.4) benefit by wireless 6G communication for wide area connectivity.

Availability of electrical power: Electrical power is generally available. Nevertheless, power efficiency is nonetheless required for battery-driven devices like mobile robots and as a general requirement supporting sustainability.

Management and orchestration (M&O). Resources are properly allocated for providing the cooperating mobile robot service with the required level of QoS. In particular, the resource allocation is executed at the radio access level, either on macro cell or femto- or pico-cells and at the operator network level. Depending on the network architecture, the resources available on different segments of the network (e.g. transport, edge, core, subnetwork) are properly allocated.

### 11.x.3	Service Flows

#### 11.x.3.1	Scenario: Cooperative Carrying With Mobile Robots

Cooperative carrying with mobile robots is a concept where multiple autonomous mobile robots (AMRs) work together to transport an object that exceeds the carrying capacity of a single robot (see also Figure 11.x.1-1).

- Industrial Communication: Collaborative robots exchange sensor data and industrial control information with each other for cooperative carrying. Time synchronization and deterministic highly reliable and low latency communicaton between cooperating robots are needed.

- Digital Twinning: Sensor data and sensing information is sent to edge nodes, digital twins, and potentially to AI applications.

- Artificial Intelligence: The autonomous mobile robots have AI capabilities for task planning and autonomous operation. AI model transfer in case of decentralized learning or from e.g. a local AI/ML compute server.

The collaborative mobile robots are in close proximity. They are mobile and can move in unison as a group. They have sensing capabilities as well as AI capabilities for task planning and autonomous operation. The collaborative mobile robots for cooperative carrying can be found indoors on the factory floor and outdoors in the yard of a factory, a construction site, a campus, and similar places.

The special focus of this scenario is on collaboration between several AMRs, deterministic industrial communication between AMRs in a moving subnetwork (time synchronization, highly reliable and low latency communication).

#### 11.x.3.2	Scenario: Lot-size-1 Production

Lot-size-1 production is a manufacturing approach in which each product is manufactured individually and uniquely, rather than in batches. This method is facilitated by the collaboration of robots with different capabilities within a production cell, as well as the cooperation between logistics AMRs and production cells. This scenario requires industrial communication for the exchange of industrial control information (time synchronization, deterministic URLLC communicaton). The 6G system needs to support data communication for digital twinning and artificial intelligence applications.

The special focus of this scenario is on collaboration between robots and between robots and logistics AMRs as well as dynamic task and process planning (digital twinning, AI applications).

#### 11.x.3.3	Scenario: Automated Industrial Tasks

In warehouses or logistic sites, cobots and logistics AMRs perform automated industrial tasks (e.g., quality check at production line, movement of goods between different workstations, from storage to shipping, or from quality check to repair), autonomous system diagnostics and functionality (re-)allocation of the participating robots (e.g., for charging or repairment) without disrupting the warehouse process. The quality check at production line may use machine vision technology. Depending on the sensing type and the video resolution, high data throughput may be required. Artificial intelligence applications and Digital Twinning will be used in this scenario.

The special focus of this scenario is on automated industrial tasks of collaborating robots such as machine vision services and autonomous incident mitigation, for instance, by functionality re-allocation.

#### 11.x.3.4	Scenario: Autonomous Farming

Autonomous farming refers to the use of self-driving, smart machines that work collaboratively in agricultural operations such as weed picking or harvesting.

The collaborative self-driving smart agricultural machines are in close proximity (subnetwork) and can move in unison as a group. They operate outdoors in urban areas (fields, meadows). Local machine-to-machine communication (e.g. direct device communication) for exchange of control information, sensing information, and positioning information (relative and absolute). Wide-area human-to-machine communication for remote monitoring, remote control, and human intervention.They have location, sensing, and AI capabilities for autonomous, collaborative operation and safety.

The special focus of this scenario is on location capabilities in a moving subnetwork.

#### 11.x.3.5	Scenario: Autonomous Construction Site

On an autonomous construction site, different types of robots work collaboratively in unison to construct a building (see also Figure 11.x.1-1). The transportation of materials, such as bricks, must seamlessly integrate with advanced construction machinery like 3D printers. Other robots are responsible for post-processing tasks, such as drilling holes autonomously. This scenario requires:

- Industrial communication for the exchange of industrial control information (time synchronization, deterministic URLLC communicaton), location information, and sensing information for cooperative construction work. This includes human-to-machine communication for remote monitoring, remote control of construction machines, and human intervention.

- Sensing and localization capabilities for positioning, location services (absolute and relative), detection of human presence, quality monitoring, collision avoidance, etc. Various sources are possible such as 6G devices with ICAS or HD video cameras (requires high uplink data throughput).

- Immersive Extended Reality with high-bandwidth and low latency communication for local and outdoor walkthroughs of future construction stages at the construction site but also in remote offices. Virtual guidance for complex construction aspects for previews and better understanding, remote participation of stakeholders.

- Support for digital twins of the buildung that provide detailed information about all project stages.

- Support for artificial intelligence applications on autonomous construction machinery.

The collaborative construction machinery operates in a local private network on a construction site outdoors but becoming more and more indoors as the construction advances.

The special focus of this scenario is on the dynamics of the collaboration and the large variety of 6G services such as industrial communication, positioning, AI/ML support, digital twinning, and immersive extended reality.

#### 11.x.3.6	Scenario: Smart Workshop

In a smart workshop, cooperating robots seamlessly connect material flow between various manufacturing stations (logistics AMRs). A high level of integration and autonomy of the machines enables tradespeople and hobbyists to produce items that were previously only possible in a factory setting. This scenario requires industrial communication for the exchange of industrial control information (time synchronization, deterministic highly reliable and low latency communicaton) between cooperating robots and between HMI devices and manufacturing stations (real-time monitoring and control). Furthermore, the 6G system needs to support sensing and localization (positioning, detection of human presence, collision avoidance, etc.), data communication for digital twinning and artificial intelligence appliations.

### x.1.4	Post-conditions

The cooperating mobile robots finished the assigned task(s) collaboratively. If a subnetwork was setup for a specific task, for instance, as for collaborative carrying with mobile robots (see 11.x.3.1), the subnetwork will be terminated. In other, more static scenarios, the cooperating mobile robots will continue to the next task to be done collaboratively.

### x.1.5	Existing features partly or fully covering the use case functionality

Local ad hoc connectivity: A collaborative task is characterized by its localized nature. Direct communication between connected devices allows this to be exploited.

Support of Private Networks: The network environment is usually dedicated to a specific purpose and private use in geographically limited areas. Trade-offs among coverage, data rate, latency, reliability etc. can be tailored according to specific application needs inside the private, local networks and to on-premise functionalities and data storage.

Energy efficiency: Energy efficiency is a general requirement on wireless communication. However, energy saving in the 6G network must not endanger successful, efficient, and high-quality industrial manufacturing.

### 11.x.6	Potential New Requirements needed to support the use case

The exchange of locally relevant information between robots will be crucial for cooperation. Key building blocks are the evolution of embedded device-to-device communication towards subnetworks supporting highly reliable and low latency industrial communication, together with mechanisms for local reservation of communication resources. Mobile robots may need the capability to request and establish these reservations when local connectivity is required.

5G ultra-reliable low-latency communication does not meet the latency, reliability, and scalability requirements in several demanding industrial and vertical scenarios, especially when looking at scalability with larger numbers of URLLC connections and high reliability in operational settings. Especially 5G direct device communication does not support URLLC and other industrial functionality such as time synchronization.

Interruptions in industrial manufacturing operations typically entail huge financial and material loss. Safety-critical applications are intended to protect human lives. As such, applications in this domain have strict service-level reliability and E2E latency requirements. The service area may be highly localized to subnetworks with a limited number of UEs. Increased autonomy in machines may accommodate better scalability, lower reliability and higher latency.

Subnetworks build from local ad hoc connectivity with stringent latency and reliability targets: A collaborative task is characterized by its localized nature. Direct communication between connected devices allows this to be exploited, leading to the formation of subnetworks. These subnetworks, envisioned to be task-specific, temporary, and localized, are embedded within broader campus networks. This structure aids in meeting stringent latency and reliability targets within each subnetwork.

[PR-11.x.6-01]	The 6G system shall be able to support direct device connection between a group of UEs (subnetwork) with periodic deterministic communication with high reliability and low latency (see Table 11.x.6-1 use case (1) for respective service performance requirements).

[PR-11.x.6-02]	The 6G system shall be able to support time synchronization (working clock domain) between the UEs within the group of UEs with direct device connection.

[PR-11.x.6-03]	The 6G system shall be able to support the instantiation and termination of subnetworks between a group of UEs.

[PR-11.x.6-04]	The 6G system shall be able to support the instantiation and termination of subnetworks using direct device connection between members of the subnetwork.

[PR-11.x.6-05]	The 6G system shall be able to provide mechanisms for local reservation of communication resource in a subnetwork (group of UEs with direct device communication).

Autonomous mobile robots may join and leave the subnetwork dynamically.

[PR-11.x.6-06]	The 6G system shall be able to support the dynamic joining and leaving of UEs to/from a subnetwork (group of UEs).

The localized and ad hoc nature of subnetworks in a 6G environment may induce frequent handovers of potential direct network connections as the subnetwork or members of the subnetwork move. These subnetworks, while logically embedded in a campus network, may exhibit nomadic behaviour within it, and the roaming of subnetworks between different campus networks may also occur.

[PR-11.x.6-07]	The 6G system shall be able to support mobility of the group of UEs using direct device connections (subnetwork) with periodic deterministic communication with high reliability and low latency (see Table 11.x.6-1 for respective service performance requirements). Some or all UEs may use direct network connection for additional communication services.

[PR-11.x.6-08]	Subject to network operator policies and agreement between the network operators, the 6G system shall be able to support service continuity of the group of UEs using direct device connections with service performance requirements given in Table 11.x.6-1 between different 6G networks.

[PR-11.x.6-09]	The 6G system shall be able to provide positioning with positioning accuracies as given in Table 11.x.6-2.

Integrated sensing capabilities within the 6G network and devices (ISAC) can potentially enhance a robot's perception of its environment. Robots and cobots depend on capturing the environmental context. Network-integrated sensing may complement or replace dedicated onboard sensors. Efficient transport of data/information from connected external sensors is likely needed.

[PR-11.x.6-10]	The 6G system shall be able to provide sensing-related capabilities and sensing information to the UEs.

The introduction of AI/ML traffic types and AI/ML execution in edge nodes can further enhance robot coordination. Cooperating robots significantly benefit from advances in machine learning. Network-side support for efficient exchange of training data, weights, and models, along with federated learning, will likely become relevant. Complementing the onboard computation capabilities of robots, offloading computation and advanced ML to nearby edge nodes or providing it by the network as an over-the-top service may add another layer of efficiency. Moreover, future systems are expected to exploit context information derived from sensing, the application, or a shared knowledge base, to improve both application and communication performance.

[PR-11.x.6-11]	The 6G system shall be able to support AI/ML-related communication between the UEs attached to collaborative mobile robots / autonomous embodied agents within flexible manufacturing.

The integration of sensing and positioning capabilities in the radio interface may aid with the collection of data that is needed for advanced machine learning applications. For maximum accuracy, it is likely that data from device-based sensors will be utilized in fusion with information from the network. Trust management is crucial for safe machine-machine and human-machine interactions, and will require reliable, deterministic, and low-latency E2E message transport mechanisms along with secure sensing results and manipulation-protected positioning.

[PR-11.x.6-12]	The 6G system shall be able to support communication between UEs and the 6G network with performance requirements according to Table 11.x.6-1 use case (2).

[PR-11.x.6-13]	The 6G system shall be able to support parallel communication of diverse communication services, such as video, periodic deterministic industrial communication, digital twin data, AI/ML traffic, which potentially might require synchronized transmission.

Table 11.x.6-1: Performance Requirements for Cooperating Mobile Robots use case

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency (ms) | Application data rate (Mb/s) | Reliability (%) | Message size (byte) | Service area (m²) | Transfer interval (ms) | Connection density [devices/m2] | UE speed (km/h) |  |
| (1) Cooperating Mobile Robots – Cooperative Carrying (11.x.3.1) | ≥ 0.8 | ≤2.5 (UL, DL) | 99.999 to 99.99999 (note 4) | 250 to 500 | 20 to 100 | ≥ 1.66 | ≤ 0.5 (note 1) | < 20 | Industrial Communication/ control traffic |
| (2) Cooperating Mobile Robots | 1 to 10 | <10 <250 (note 2) | 99.999 to 99.99999 (note 4) | (NOTE 3) | 20 up to 1,000,000 (note 5) | ≤ 10 | ≤ 0.5 (note 1) | < 20 | Robot to Network (Parent Network, Campus Network) |
| NOTE 1: Local concentration of mobile robots, for instance, cooperative carrying assuming 8 robots on a space of 4 m x 4 m carrying an object. NOTE 2: Significantly higher data rate depends on type of exchanged data, for instance, immersive XR, AI/ML traffice, digital twin data, etc. It is expected uplink video will be most demanding. NOTE 3: Any common message size to be expected. Message size depends on type of exchanged data, for instance, sensor data, video streams, immersive XR, AI/ML traffic, digital twin data, etc. NOTE 4: The control of real-time industrial processes requires very high service reliability. Can be lower for non-real-time digital twin or in other contexts (e.g. training, engineering, planning). NOTE 5: Service area depends on scenario and size of location/collaboration area (subnetwork of collaborative carrying mobile robots (11.x.3.1), factory, construction site (11.x.3.5), field (11.x.3.4)) |  |  |  |  |  |  |  |  |  |

Table 11.x.6-2: Performance Requirements for Cooperating Mobile Robots use case

| Use Cases | Positioning accuracy (m) | Velocity (km/h) | Remarks |
| --- | --- | --- | --- |
| Cooperating Mobile Robots | <0.1 <1 | < 20 | Collaborative tasks such as handing over material and objects or robot navigation in close distance to other objects require more accurate positioning. Tasks like robot localization can work with less accurate positioning. |
| Human presence detection | 0.4 | Pedestrian | 40 cm resolution within 1 m of the machine |