3GPP TSG-SA WG1 Meeting #109 	S1-250314

Athens, Greece, 17th-21th  Feb. 2025	(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0833 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements on multi-orbit support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Novamint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph4 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP SA1 has studied several use cases related to multi orbit in Rel-20 FS_5GSAT_Ph4. The corresponding requirements have been captured in TR 22.887. It is proposed to introduce the corresponding requirements and KPIs into TS 22.261. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces requirements and KPIs related to multi orbits as defined in TR22.887 consolidation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi orbit satellite access is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.46.2, 6.46.3, 6.46.x (new), 7.4.2, 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.46	Satellite access

### 6.46.1	Overview

The following requirements apply for a 5G system with satellite access.

NOTE:	For the KPIs for a 5G system with satellite access, see clause 7.4.

### 6.46.2	General

A 5G system with satellite access shall support different configurations where the radio access network is either a satellite NG-RAN or a non-3GPP satellite access network, or both.

A UE supporting satellite access shall be able to provide or assist in providing its location to the 5G network.

A 5G system with satellite access shall be able to determine a UE's location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE.

NOTE:	This is also applicable for UE using only satellite access. The determination of a UE’s location can be based on 3GPP and/or non-3GPP positioning technologies subject to operator’s policies.

A 5G system with satellite access shall be able to support low power MIoT type of communications.

Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE independently of the UE’s GNSS capability.

Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite.

Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location.

### 6.46.3	Service continuity

For a 5G system with satellite access, the following requirements apply:

- A 5G system with satellite access shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by different operators having an agreement.

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) for a UE engaged in an active communication, when the UE changes from a direct network connection via 5G terrestrial access to an indirect network connection via a relay UE (using satellite access) and vice-versa.

NOTE:	It is assumed that the 5G terrestrial access network and the satellite access network belong to the same operator.

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites).

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the communication path between UEs extends to additional satellites (through ISLs).

-	Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support service continuity with no interruption when the communication path of a UE (e.g., Relay or CPE) is moving between satellites of the same network operating in any orbit type with given characteristics.

-	Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when the communication path of a UE (e.g., Relay or CPE) is moving between terrestrial access and satellite access networks or between satellites networks potentially supporting different orbit types with different characteristics.

### 6.46.4	Roaming aspects

For a 5G system with satellite access, the following requirements apply:

-	A 5G system with satellite access shall enable roaming of UE supporting both satellite access and terrestrial access between 5G satellite networks and 5G terrestrial networks.

-	UEs supporting satellite access shall support optimized network selection and reselection to PLMNs with satellite access, based on home operator policy.

### 6.46.5	Resource efficiency

For a 5G system with satellite access, the following requirements apply:

-	A 5G system with satellite access shall support the use of satellite links between the radio access network and core network, by enhancing the 3GPP system to handle the latencies introduced by satellite backhaul.

-	A 5G system with satellite access shall be able to support meshed connectivity between satellites interconnected with ISLs.

### 6.46.6	Efficient user plane

For a 5G system with satellite access, the following requirements apply:

- A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfils the agreed QoS.

- A 5G system with satellite access shall be capable of supporting simultaneous use of 5G satellite access network and 5G terrestrial access networks.

-	A 5G system with satellite access shall be able to support both UEs supporting only satellite access and UEs supporting simultaneous connectivity to 5G satellite access network and 5G terrestrial access network.

-	Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved.

### 6.46.7	Satellite and Relay UEs

For a 5G system with satellite access, the following requirements apply:

-	A 5G system with satellite access shall be able to support relay UEs with satellite access.

NOTE:	The connection between a relay UE and a remote UE is the same regardless of whether the relay UE is using satellite access or not.

-	A 5G system with satellite access shall support mobility management of relay UEs and the remote UEs connected to the relay UE between a 5G satellite access network and a 5G terrestrial network, and between 5G satellite access networks.

-	A 5G system with satellite access shall support joint roaming between different 5G networks of a relay UE and the remote UEs connected to that relay UE.

### 6.46.8	Store and Forward Satellite Operation

#### 6.46.8.1	Description

NGSO (MEO/LEO) based satellite access is raising higher demands on the amount of ground stations, and the availability and stability of the connectivity to ground station for UEs to obtain end-to-end network services anytime.

S&F (Store and Forward) Satellite operation in some level provides a way to enable autonomously network service to UEs without the satellite always being connected to the ground station, which can extend the service availability for the areas without the connectivity to ground station via feeder link or ISL (e.g. at sea, very remote areas lack of ground-station infrastructures), improve the ground segment affordability with fewer ground stations and allow more robust UE services with the satellite under intermittently/temporarily unavailable feeder link.

This is particularly relevant for delay-tolerant communications via NGSO space segment.

The requirements below refer to S&F (Store and Forward) Satellite operation.

NOTE:	For more information on Store and Forward Satellite operation see Annex J.

#### 6.46.8.2	Requirements

Subject to operator’s policies, a 5G system with satellite access shall be able to support S&F Satellite operation for authorized UEs e.g. store data on the satellite when the feeder link is unavailable; and forward the data once the feeder link between the satellite and the ground segment becomes available.

A 5G system with satellite access shall be able to inform a UE whether S&F Satellite operation is applied.

Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data retention period.

Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data storage quota.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting S&F Satellite operation shall be able to support a mechanism to configure and provision specific store and forward QoS and policies for a UE (e.g. forwarding priority, acknowledgment policy).

A 5G system with satellite access supporting S&F Satellite operation shall be able to provide related information (e.g. estimated delivery time to the authorised 3rd party) to an authorized UE.

A 5G system with satellite access shall be able to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g. estimated delivery time to the authorised UE).

Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

NOTE:	It is assumed that the satellite constellation knows which satellite has a feeder link available. However, this is outside the scope of 3GPP.

Subject to operator’s policies, a 5G system with satellite access supporting the S&F Satellite operation shall be able to support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available.

A 5G system with satellite access supporting S&F Satellite operation shall support mechanisms for a UE to register with the network when the network is in S&F Satellite operation.

A 5G system with satellite access supporting S&F Satellite operation shall support mechanisms to authorize subscribers for receiving services when the network is in S&F Satellite operation.

### 6.46.9	UE-Satellite-UE communication

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of a UE-Satellite-UE communication.

Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast).

### 6.46.10	Positioning aspects for satellite access

For a 5G system with satellite access, the following requirements apply:

-	Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access.

-	A 5G system with satellite access shall be able to provide positioning service to a UE using only satellite access and the information on positioning services (e.g. supported positioning performance).

NOTE: 	UE can be with or without GNSS capabilities

-	A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE and network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS).

* * * Second Change * * * *

### 6.46.10	Multi-Orbit

For a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities), the following requirements apply:

- A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand.

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide the UE with the necessary information to acquire and access the suitable satellite among those available.

NOTE 1: The information provided to the UE can include satellites information operated by roaming partners.

NOTE 2: Acquisition of satellites may be required prior to the initial access procedure, including, e.g. the determination of the antenna pointing direction. In this use case, network selection procedures follow the existing procedures.

- Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support switching between satellite orbit types with different characteristics depending on the latency requirements or QoS.

- A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics.

* * * Third Change * * * *

### 7.4.2	Requirements

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

The 5G system with satellite access shall support communication service availabilities of at least 99,99%.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/s | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | [1] Gbit/s per plane | [300] Mbit/s per plane | TBD | TBD | TBD | 40% | Up to [1500] km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Video surveillance (note 4a) | [0,5] Mbit/s | [3] Mbit/s | TBD | TBD | TBD | N/A | Up to 120km/h or stationary (note 4b) | Vehicle mounted or fixed installation |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% Assumption of 3:1 Downlink / Uplink ratio, anticipating future usages The Downlink & Uplink throughput can be achieved using one or multiple links Note 4a: Refer to video surveillance data transmitted (in UL) from a UE on the ground (e.g. picture or video from a camera) using satellite NG-RAN to connect to 5GC, and video surveillance-related configuration or control data sent (in DL) to the UE/device. 0.5 Mbit/s for DL experienced data rate is based on MAVLINK protocol that is widely used for UAV control. 3 Mbit/s for UL experienced data rate is based on the assumed sum from 2.5 Mbit/s for video streaming and 0.5 Mbit/s for data transmission. Note 4b: Up to 120km/h applies to vehicle mounted while stationary applies to fixed installation. Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario. |  |  |  |  |  |  |  |  |

* * * Fourth Change * * * *

## 9.4	Satellite Access

In a 5G system with satellite access, charging data records associated with satellite access(es) shall include the location of the associated UE(s) with satellite access.

NOTE: The precision of the location of the UE can be based on the capabilities of the UE or of the network.

A 5G system with satellite access supporting S&F Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites).

A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, for UE-Satellite-UE communication.

A 5G system with satellite access supporting multiple satellite orbits types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to user data traffic or mobile base station relay traffic over the different satellite access types.

* * * End of Changes * * * *