3GPP TSG-SA WG1 Meeting #53	S1- 110084

# Nashville, USA, 14th – 18th February 2011

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.368 | CR | 0080 | rev |  | Current version: | 11.0.1 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MTC - IMS Service Layer Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Telefonica |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SIMTC |  |  |  |  |  |  | Date: |  | 07/02/2011 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) |  |
| Reason for change: |  | MTC Devices may need to make use of the service layer capabilities provided by the IP multimedia core network subsystem and Application Servers in the frame of enriched services |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add requirements for the network to handle the registration of MTC Devices and applications running on MTC Devices onto the IP multimedia core network subsystem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MTC Devices will not be able to make use of the service layer capabilities provided by the IP multimedia core network subsystem and Application Servers, ruling out the option to build some enriched services that need it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------- First Change ----------------

# 7	Service requirements

## 7.1	Common service requirements

### 7.1.1	General

The following are MTC common service requirements:

-	The network shall enable the network operator to identify per subscription which individual MTC Features are provided to a particular MTC Subscriber.

-	The network shall provide a mechanism for the MTC Subscriber to activate or deactivate MTC Features.

NOTE:	The activation/deactivation functionality can be provided via a web interface that is outside the scope of 3GPP specifications.

-	The network shall provide a mechanism for the network operator to control the addition or removal of individual MTC Features to a subscription (e.g. based on matching or mismatching of MTC Features).

-	The network shall provide a mechanism for the network operator to restrict activation of MTC Features (e.g. based on matching or mismatching of MTC Features).

-	The network operator shall be able to restrict the use of a USIM to specific MEs/MTC Devices (e.g. based on matching or mismatching of MTC Features and/or other types of MTC Devices).

-	The network shall provide a mechanism to reduce peaks in the data and signalling traffic resulting from very large numbers of MTC Devices (almost) simultaneously attempting data and/or signalling interactions.

- 	The system shall provide mechanisms to efficiently maintain connectivity for a large number of MTC Devices.

-	The network shall provide mechanisms to handle MTC Devices and applications on MTC Devices registering on the IP multimedia core network subsystem and accessing its capabilities including interaction with IMS application servers/enablers.

-	The IP multimedia core network subsystem shall enable MTC Devices or applications running on MTC Devices to register with a temporary assigned ID instead of with a uniquely and permanently assigned IMS identity.

-	The system shall provide mechanisms to lower power consumption of MTC Devices.

-	MTC Devices may be kept offline or online when not communicating, depending on operator policies and MTC Application requirements.

-	MTC Devices may keep their data connection or not keep their data connection when not communicating, depending on operator policies and MTC Application requirements.

---------------- End of Change ----------------