3GPP TSG-SA1 (Services) — SA1#53	 		     		S1-110057

# 14 – 18 February 2011

# Nashville, TN USA

Source:	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, InterDigital

Title:		Proposed Use Case for the TR 22.895

Document for:	Discussion and approval

Agenda Item:	9.6

Work Item / Release:	SSO_Int/Rel-11

# 1. Introduction

This pCR proposes a new use case for the  TR 22.895, Study on Service aspects of integration of Single Sign-On (SSO) frameworks with 3GPP operator-controlled resources and mechanisms.

# 2. pCR

******* Begin Changes **********

## 4.X Accessing a Third Party Web Service using OpenID

A user, who has a subscription with a mobile network operator, accesses a Web service which supports OpenID, for example an online photo service. The user’s network operator is also an OpenID identity provider. Upon making a request, which contains the user’s OpenID identifier to the online photo service, the user gains an authenticated access to the photo service for viewing access-protected photographs without being involved in the authentication procedure.

### 4.X.1 Pre-conditions

- The user’s network operator is also an OpenID identity provider

- The OpenID identifier has been established by the network operator

- The online photo service is a Web service that supports OpenID

- The online photo service trusts the user’s network operator for the user  authentication

### 4.X.2 Entities involved in the use case

The following entities are involved in the authentication procedure:

- User with the user equipment (UE), which is capable of running a Web client and communicating with the appropriate operator credentials

- Application Web server — an entity providing an online photo service. It plays the role of a Relying Party

- OpenID identity providing service (OP), which is controlled by the operator

- Operator’s HSS

### 4.X.3 Additional requirements for the entities participating in the authentication

- Requirements for UE

UE must be capable of:

- Authenticating with the use of operator controlled credentials .

- Conditions for the application Web server (Relying Party)

The application server must be able to support OpenID specification [aa]

- Requirements for the OP

OP must be able to:

- Perform authentication based on operator controlled credentials.

- Serve as an OpenID identity provider

********* End Changes ************

It is agreed to add the following text to the Deployment Scenarios section of the TR:

The following deployment scenarios apply to all Use Cases in Section 4.

The operator is able to provide the authenticated access in operator environments where GBA is supported or not.