3GPP TSG-SA WG1 Meeting #70	S1-151590

Los Cabos, Mexico, 13-17 April 2015	(revision of S1-151349)

Title:	V2I Traffic Flow Optimisation Use Case

Agenda Item:

Source:	Nokia Networks

Contact:	Juergen Merkel (juergen.merkel(at)nokia.com)

Abstract:

This use case describes vehicles V2I / V2N communication to optimise traffic flow when approaching traffic lights. This use case addresses the situation when approaching the vehicle has to stop even though there are no other cars around at an intersection. Depending on the other traffic around this function will provide green light to a car when approaching traffic lights and an indication of speed at which the green light will be met without having to stop or miss the green light phase.

---------- Use Case template ----------

## 5.X	V2I / V2N Traffic Flow Optimisation Use Case

### 5.X.1	Description

This use case describes vehicles V2I/N (Vehicle to Network) communication to a centralised ITS server referred here to as “entity” to optimise traffic flow when approaching traffic lights. This use case addresses the situation when approaching the vehicle has to stop even though there are no other cars around at an intersection. Depending on the traffic situationthis function will provide green light to a car when approaching traffic lights and an indication of speed at which the green light will be met without having to stop or miss the green light phase.

To enable this information about vehicles approaching traffic lights has to be made available well in advance i.e. in most cases beyond ProSe range. When coming in ProSe range communication might be switched from network to direct communication, if deemed useful.

Compared to V2V communication within ProSe range the delay and latency requirements are more relaxed due to the longer distances and the non-safety related nature of this use case.

Editors Note: Need for the definition of new mode V2N needs to be clarified.

### 5.X.2	Pre-Conditions

Vehicles A, B, C on road 1 are approaching an intersection with road 2, Vehicle D is approaching the intersection on road 2. All vehicles are equipped with UEs supporting V2I/N. The intersection is equipped with traffic lights.

Depending on the V2X deployment model, Vehicle A, B, C and D can be subscribed to different PLMNs and can be international roamers. This shall not adversely impact the function of the V2X service e.g. by increased delays.

### 5.X.3	Service Flows

V2I/N enabled Vehicles A, B, C and D are transmitting their position, speed and direction of travel to an entity controlling the traffic light(s) and giving back information, such as speed recommendations, to the vehicles. The function of this entity is out of scope for 3GPP.

The 3GPP network will provide additional information to the entity enable the entity to authenticate the vehicle more easily.

The 3GPP network will provide a rough indication of position for the entity to verify the location information received from the vehicle directly.

The entity recommends a proposed speed to the vehicles and influences the phases of the traffic lights. The drivers could be notified of this recommended speed by an announcement or other means such as a head up display.

### 5.X.4	Post-Conditions

Let’s assume without traffic optimisation Vehicle D would arrive at the intersection shortly before vehicles A, B and C, causing the traffic light to stop vehicles A, B, and C. With traffic flow optimisation i.e. taking into account the speed and direction of the cars and the maximum allowed speed there are several other options such as:

- all vehicles will receive indications of a proposed speeds at which all of them will arrive at the traffic lights and meet a green light there or

- the traffic light will stop vehicle D and give way to vehicles A, B and C to minimise energy consumption by causing a lesser number of vehicles to stop and accelerate again.

If the drivers adhere to the recommendations, the traffic flow will be impacted least.

### 5.X.5	Potential Requirements

[PR.5.X.5-001] The E-UTRAN shall be capable of transferring V2I/N service messages to/from UEs supporting V2I/N with variable message payload of 50-300 Bytes.

[PR.5.X.5-002] The E-UTRAN shall be capable of transferring V2I/N service messages to/from UEs supporting V2I/Nwith maximum frequency of 1 messages per second and a minimum frequency of 1 message per 10 seconds.

[PR.5.X.5-003] The E-UTRAN shall be capable of transferring V2I/N service messages to/from highly mobile UEs supporting V2I/N applications with an end-to-end delay no larger than 1000 ms.

[PR.5.X.5-004] The E-UTRAN shall support anonymity and integrity protection of communication.

[PR.5.X.5-005] The 3GPP network shall provide means to support the entity to authorise the UEs supporting V2I/N.

[PR.5.X.5-006] Based on 3GPP network means the 3GPP network shall provide the location of the UEs supporting V2I/Nto the entity.

[PR.5.X.5-007] All UEs supporting V2I/N independent of their association with different HPLMN or roaming condition shall experience the same service quality from the 3GPP network, for example on delays, latency and ease of use of the service.