3GPP TSG-SA WG1 Meeting #71 ad hoc on SMARTER (5G)	S1-153057

Vancouver, Canada, 19 – 21, October, 2015

Title:		New SMARTER Use Case on support for flexible broadcast content caching in 3GPP system

Agenda Item:	3.5	New use cases

Source:		Dish Network

Contact: 	Ojas Choksi (ojas.choksi@dish.com)

Abstract: This submission proposes a new use case for support of caching of broadcast content in 3GPP system and efficient delivery of the cached broadcast content to the users

5.x Broadcast Content Caching and Delivery

5.x.1 Description

Video based services and applications have been instrumental for the massive growth in mobile broadband traffic.  The forecast for mobile data traffic [1] seems to suggest that the video services will continue to remain a key driver for future growth in mobile data traffic; by 2019, as much as 70% of the total mobile data traffic is expected to be video based. Simultaneously, over the last decade, advances in the semiconductor technologies have driven down the unit cost and volume of storage devices, thereby allowing for a flexible and cost effective deployment of content caching entities at the edge of the network.  Deploying broadcast content caching at the edge 1) provides a better user experience (lower latencies and channel switching times) for the end-user, 2) allows the operators to dimension their network and backhaul more cost-effectively and 3) in some scenarios, efficiently utilize its limited radio resources.

5.x.1.1 Pre-conditions

Wireless operator A has deployed a 3GPP system.

Three types of broadcast content caching entities, X, Y and Z have been deployed within the network.

Entity X is co-located with the integrated terminal/set-top box.  Entity Y is co-located with the radio site M.  Entity Z is located at a local aggregation point.

5.x.1.2 Service Flow

Wireless operator A decides to deliver popular video content using the multicast / broadcast function (MBMS) of the 3GPP system.

The content caching entities X, Y and Z receive and store the broadcast/multicast content.

User 1 is at home and accesses the video content on a personal device via a device to device interface from content caching entity X located in the 3GPP enabled set-top box.

User 2 is at the local coffee shop and served by site M.  User 2 accesses the video content via content caching entity Y.

User 3 is walking in an urban area and is being served by the Pico-cell N.  User 3 accesses the video content via content caching entity Z located at the local aggregation point.

5.x.1.3 Post-Conditions

Users 1, 2 and 3 are able to watch the video content of interest, previously broadcasted, from a content caching entity through efficient utilization of network and radio resources.

5.x.2 Potential Service Requirements

- The 3GPP system shall be able to support a content caching entity located at multiple locations within the network.

- The 3GPP system shall support a content caching entity integrated within a terminal.

- The 3GPP system shall support a content caching entity at the location of the radio site.

- The 3GPP system shall support a content caching entity at a local aggregation point of its choice.

- The operator shall have the flexibility to deploy the caching entity at a combination of local aggregation point, several sites within the radio network and in devices with sufficient storage capacities.

- The content caching entities shall be able to join the broadcast/multicast group and cache the broadcast/multicast content being delivered to/via radio access network.

- The user shall be able to access the broadcast content via the content caching entity.

- The 3GPP system shall be able to route the content request from the user to the optimal content caching entity irrespective of its location within the 3GPP system.

- The 3GPP system shall support delivery of the cached content using the QoS subscription of the user.

- The 3GPP system shall support charging for content delivered to the user by the content caching entity.

5.x.2 Potential Operational Requirements

References

[1] Cisco Visual Networking Index: Global Mobile Data Traffic Forecast Update 2014-2019 White Paper, Table 6 (http://www.cisco.com/c/en/us/solutions/collateral/service-provider/visual-networking-index-vni/white_paper_c11-520862.html)