3GPP TSG-SA WG1 Meeting #71	S1-153196

Vancouver, Canada, 19-21 October 2015	(revision of S1-153070, S1-153150)

Title: Fronthaul/Backhaul Network Sharing

Agenda Item:	3.5 New use cases

Source:	ITRI

Contact:	Chia-Lin Lai <Chia-LinLai@itri.org.tw>

Abstract: <provide a short description of the content>

This paper proposes a fronthaul/backhaul network sharing use case and the related requirements.

---------- Use Case template ----------

5.xx Fronthaul/Backhaul Network Sharing

5.xx.1	Description

Network sharing can also imply sharing of fronthaul/backhaul network resources by using emerging 5G method (e.g., network slicing) or existing method (e.g., NFV). Fronthaul/backhaul networks can be deployed by multiple operators. This will allow network operators to benefit by reducing deployment cost and provide better Quality of Service (QoS), increase in revenue e.g. by optimally sharing network resources with other operators when considering traffic conditions, peak hours, etc.

NOTE: Fronthaul network is referred to e.g., the network between Remote Radio Head (RRH) and Baseband Unit (BBU) whereas backhaul network is referred to the network e.g., between BBU and the core network.

5.xx.2	Potential service requirements

The 3GPP system shall be able to provide fronthaul/backhaul network sharing information and capabilities to other network operators.

5.xx.3	Potential operational requirements

The 3GPP system shall allow network operators to be able to share fronthaul/backhaul network resources.