3GPP TSG-SA WG1 Meeting #76Bis	S1-170018

Spokane, Washington, USA, 16-20 Jan 2017

Title:	Align with recent NGMN values on Latency requirements

Agenda Item:	5.1

Source:	Huawei Technologies, Hisilicon

Contact:		Andrey Krendzel (andrey.krendzel@huawei.com)

Laurence Meriau (laurence.meriau@huawei.com)

Abstract: This document proposes to align with recently agreed NGMN latency requirements of use cases for vertical industries to SMARTER TS 22.261.

# Introduction

Currently SMARTER TS table 7.2.2-1 latency refers only to a one-way latency on the radio layer.

The recent NGMN white paper on Vertical Industries and Implications for 5G (September 2016) has done further progress in this direction and specifies now end-to-end latency requirements for use cases from vertical industries, in particular, the automotive ecosystem, transport and logistics, health and wellness, smart cities and utilities, media and entertainment, and industrial IoT industries.

It is worthwhile to add them in SMARTER TS 22.261.

Proposal

It is proposed to add in SMARTER TS 22.261 the end-to-end latency KPIs retrieved from NGMN documents related to use cases from the vertical industries to the section 7.2 low latency and high reliability. An extract of NGMN original text and table rows related to latency for vertical industries is given in the Annex in the end of this document.  For the automotive use cases KPIs for high density platooning and remote vehicle operation are taken from the 5G V2X TR 22.886 to be aligned with this study.

* * * Start of 1st change * * *

## 7.2 Low latency and high reliability

Editor's Note: Text in this section is still to be elaborated

### 7.2.1	Overview

Several service scenarios demand very low latency and very high reliability. The overall service latency depends on the delay on the radio interface, transmission within the 3GPP system, transmission to a server which may be outside the 3GPP system, and data processing. Some of these factors depend directly on the 3GPP system itself, whereas for others the impact can be reduced by suitable interconnections from the 3GPP system to services or servers outside of the 3GPP system, for example, to allow local hosting of the services. The services and their performance requirements can be found in table 7.2.2-1. Latency in the table refers to the one-way latency on the radio layer. The low latency values required to support the potential opportunities in the use cases on vertical industries are summarised in table 7.2.X-1. Latency in this table refers to the end-to-end latency at the application layer.

### 7.2.2	Services and KPIs

Services requiring low latency and high reliability can be categorized in the following groups:

-	Industrial control - Conventional industrial control is characterised by high requirements on the communications system regarding latency, reliability, and availability. Systems supporting industrial control are usually deployed in geographically limited areas but may also be deployed in wider areas (e.g., city- or country-wide networks), access to them may be limited to authorised users, and they may be isolated from networks or network resources used by other cellular customers.

-	Industrial automation - Industrial automation is characterised by high requirements on the communications system regarding reliability and availability. Systems supporting industrial automation are usually deployed in geographically limited areas, access to them may be limited to authorised users, and they may be isolated from networks or network resources used by other cellular customers.

-	Tactile interaction - Tactile interaction is characterised by a human being interacting with the environment or people, or controlling a device, and relying on tactile feedback.

-	Audio-visual interaction - Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a device, and relying on audio-visual feedback.

-	Remote control - Remote control is characterised by a device being operated remotely, either by a human or a computer.

Table 7.2.2-1 Performance requirements for low-latency and high-reliability services.

| Service | Latency | Reliability | Experienced data rate | Payload | Traffic density | Connection density | Coverage NOTE 1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Industrial control | 1ms | 99,999 % | 1 Mbps | Small | 1 Gbps/km2 | 1 000/km2 | Very high | NOTE 2 |
| Industrial automation | 10ms | 99,999 % | 1 Mbps | Small | 100 Mbps/km2 | 100/km2 | High |  |
| Tactile interaction | 0,5ms | [99,999 %] | [Low] | [Small] | [Low] | [Low] | [Very high] | NOTE 2 |
| Audio-visual interaction | [10ms] | [99,9 %] | [250 Mbps] | [Big] | [Low] | Low] | [High] | NOTE 3 |
| Remote control | [5ms] | [99,999 %] | [From low to 10 Mbps] | [Small to big] | [Low] | [Low] | [Very high] |  |
| NOTE 1: The coverage requirement applies to the service area, e.g., inside a factory. NOTE 2: Traffic prioritization and hosting services close to the end-user may be helpful in reaching the lowest latency values. NOTE 3: Audio-visual interaction requires very low-delay audio and video coding, and high video frame rates (e.g., 120 fps). NOTE 4: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |

Editor's Note: All values in this table are FFS.

### 7.2.X	Latency KPIs for vertical industries

Table 7.2.X-1 Latency needs to support use cases from vertical industries

| Services/Use cases | Automotive use cases | Transport, logistics, IoT use cases | Health and wellness, smart cities use cases | Media and entertainment |
| --- | --- | --- | --- | --- |
| Description | Expand detectable range beyond on board sensor capability by sharing views or detected objects among traffic participants, coordinate trajectories among vehicles, sharing coarse driving intention, real-time remote operation of vehicles | Real-time sensing, reporting, feedback, control, remote, asset tracking, monitoring; context-aware services, recommendations at shopping mall, airport | Live video feed (4K, 8K, 3D for remote healthcare (consultation, monitoring) and assisted surgery, real-time commands to control medical devices for treatment (e.g., medication, surgery); remote monitoring, surveillance and guidance for citizens and law enforcement officers. | Media production services based on aggregation of various media feeds at servers; real-time peer-to-peer or server-client sharing of data (object information) for collaborative gaming, live streaming at live events |
| Latency | For short term environment modelling (sensor sharing): <20 ms end-to-end For cooperation (coordinated control): - <10 ms end-to-end for cooperative manoeuvres, high density platooning - <100 ms end-to-end for coarse driving intention For remote vehicle operation: 5-20 ms end-to-end | For massive connectivity for time-critical sensing and feedback <30 ms end–to-end. For remote drone operation and cooperative farm machinery: 10-30 ms end-to-end Real-time control for discrete automation: ≤1 ms end-to-end | For real-time video/ telepresence/ augmented reality for remote healthcare and assisted surgery, for monitoring and guidance (smart cities): 100 ms end-to-end Real-time command and control for remote medication and surgery: - end-to-end For smart grid: - <5 ms end-to-end for transmission/ grid backbone, - <50 ms end-to-end for distribution/ grid backhaul, Time-critical sensing and feedback for smart cities: - 30 ms end-to-end | For live streaming in crowded areas, services for media production, augmented reality for collaborative gaming etc: 20 ms end–to-end |

* * * End of 1st change * * *

# Annex: NGMN original text and table rows related to latency for vertical industries

Executive Summary

The NGMN 5G White Paper, published in February 2015, outlines, among other things, a wide range of use cases, and discusses the requirements or the required considerations and studies in enabling them. In order to fully realize a sustainable ecosystem, both technical and economic realities need to be carefully assessed. This includes a prominent focus on “Verticals” requirements of 5G, in growing interaction with, engagement of, and input from the verticals industries. This paper highlights the technical requirements for a number of selected use cases across the vertical industry and the external factors that need to be considered to make these use cases economically viable. /…/

Table 1: Capabilities required for relevant automotive use cases

| Use case attribute | V2N for mid/ long-term environment modelling (dynamic high-definition digital map update) | V2X for short term environment modelling (sensor sharing) | V2X for cooperation (coordinated control) | V2N for remote vehicle operation |
| --- | --- | --- | --- | --- |
| Description | - Uploading of sensed data to servers for dynamic digital map update - Downloading of the latest digital map information | Expand detectable range beyond on board sensor capability by sharing views (e.g., raw sensor data) or detected objects (e.g., abstract object information) among traffic participants by V2X | - Coordinate trajectories among vehicles by negotiating over V2X (distributed control via V2V or centralized control via V2N/ V2I), e.g., for collision avoidance, overtaking, platooning, merging - For example, at intersections, it may be the traffic signal that collects information, makes coordinated decision, and commands control - Another example is to share detailed planned trajectory via V2X for collaborative driving - Another example is to share coarse driving intention (e.g., changing lanes or moving/ stopping/ parking in T sec at [x,y,z]) for changing lanes, merging at highway and roundabout, crossing at 4-way stop and have consensus among all involved vehicles via V2X | Real-time remote operation of vehicles - Disasters - Unknown/unexpected terrains (e.g., mining) - Handling hazardous material (e.g., nuclear plants) |
| Potential technology (/ies) | 5G | 5G | 5G | 5G |
| Latency | Not critical (100 ms end-to-end seems to be tolerable) - Downloading may be for maps to be used 5 min later, and hence not time-critical | <20 ms end-to-end | - <3 ms end-to-end for platooning - <10 ms end-to-end for cooperative manoeuvres. - <100 ms end-to-end for coarse driving intention - These requirements apply regardless of whether the messages are exchanged directly between vehicles (PC5 – sidelink) or via the infrastructure (Uu – uplink, core network and downlink). The exact latency budget for interfaces at different protocol layers will depend on the technical design. | 10-30 ms end-to-end |

Table 2: Capabilities required for relevant transport and logistics use cases

| Use case attribute | Massive connectivity for non-time-critical sensing | Massive connectivity for time-critical sensing and feedback | V2N for remote drone operation |
| --- | --- | --- | --- |
| Description | Non-time-critical sensing and reporting. For example - Predictive maintenance, asset tracking, monitoring | Time-critical sensing, reporting and feedback/ control. For example - Context-aware services Recommendation at shopping mall, airport | Real-time remote operation of drones |
| Potential technology (/ies) | Wi-Fi, LPWA, 4G, 5G? | 4G, 5G | 5G |
| Latency | Not critical | <30 ms end–to-end. | 10-30 ms end-to-end |

Table 3: Capabilities required for relevant health & wellness use cases

| Use case attribute | Real-time video/ telepresence/ augmented reality for remote healthcare and assisted surgery | Massive sensor connectivity for health and wellness monitoring | Real-time command and control for remote medication and surgery |
| --- | --- | --- | --- |
| Description | Live video feed (4K, 8K, 3D) in both uplink and downlink for remote healthcare (consultation, diagnosis, treatment, monitoring) and assisted surgery | Connectivity for sensors, wearable devices and other medical devices for health monitoring and info-mediation | Real-time commands to control medical devices for treatment (.e.g., medication, surgery) |
| Potential technology (/ies) | 5G | 4G, LPWA, 5G? | 5G? |
| Latency | 100 ms end-to-end | In the order of seconds to minutes | 10-100 ms end-to-end |

Table 4: Capabilities required for relevant smart cities and utilities use cases

| Use case attribute | Real-time video for monitoring and guidance | Massive connectivity for non-time-critical sensing | Massive connectivity for time-critical sensing and feedback |
| --- | --- | --- | --- |
| Description | Live video feed (HD, 4K, 8K, 3D) in both uplink (remote monitoring, surveillance) and downlink (guidance for citizens and law enforcement officers) | Non-time-critical sensing and reporting. For example, weather, pollution | Time-critical sensing, reporting and feedback/ control. For example Detection of natural disasters, smart grid control and context aware lighting |
| Potential technology (/ies) | 5G | Wi-Fi, LPWA, 4G, 5G? | 4G, 5G |
| Latency | 100 ms end-to-end | In the order of seconds to minutes | - 30 ms end-to-end - Smart grid: - <5 ms end-to-end for transmission/ grid backbone, - <50 ms end-to-end for distribution/ grid backhaul, - <1 s end-to-end for access |

Table 5: Capabilities required for relevant agriculture use cases

| Use case attribute | Massive connectivity for sensors and actuators | V2X for cooperative farm machinery |
| --- | --- | --- |
| Description | Monitoring of farm conditions (e.g., soil, water level, livestock) and actuation of machinery (e.g., sprinklers, feeding) | Coordinate trajectories among farm machinery for coordinated operation/manoeuvres, etc. |
| Potential technology (/ies) | Wi-Fi, LPWA, 4G, 5G? | Wi-Fi-based, 4G, 5G? |
| Latency | In the order of seconds to minutes | 10-30 ms end-to-end |

Table 6: Capabilities required for relevant media and entertainment use cases

| Use case attribute | Guaranteed QoS broadband services for media production | Low-latency services for collaborative gaming | Broadband services for live streaming in crowded areas | Mobile wireless backhaul for in-vehicle entertainment |
| --- | --- | --- | --- | --- |
| Description | Guaranteed QoS (data rate, latency, reliability, etc.) services to aggregate various media feeds at servers | Real-time peer-to-peer or server-client sharing of data (object information, live video feeds, etc.) for collaborative gaming | Live streaming of various angle views and replays at live events (sports game, concert, etc.) for spectators | Mobile wireless backhaul for on-board access points/ small cells |
| Potential technology (/ies) | 5G | 5G | 5G | 5G |
| Latency | 20 ms | 20 ms | 20 ms | 20 ms |

Table 7: Capabilities required for relevant industry IoT use cases

| Use case attribute | Massive connectivity for non-time-critical sensing | Massive-connectivity for time-critical sensing | Real-time control | Real-time video/ telepresence/ augmented reality |
| --- | --- | --- | --- | --- |
| Description | Non-time-critical sensing and reporting. For example - Predictive maintenance, asset tracking, monitoring | Time-critical sensing and reporting for process automation | Real-time control for discrete automation | - Live video feed (4K, 8K, 3D) in both uplink and downlink for remote assistance. - Augmented reality for task guidance, etc. |
| Potential technology (/ies) | Wi-Fi, LPWA, GSM, 3G, 4G, 5G? | LPWA, GSM, 3G, 4G, 5G | 5G | 5G |
| Latency | Not critical | 10 ms – order of seconds | ≤1 ms end-to-end | 100 ms end-to-end |