3GPP TSG-SA WG1 Meeting #77	S1-171048

Jeju Island, South Korea, 13-17 Feburuary 2017	(revision of S1-17xxxx)

Title:	Proposed Section 12.x "Use case: Moving Authority transmission”

Agenda Item:	8.2

Source:	ETRI

Contact:	Hyun Seo Oh < hsoh5 (at) etri.re.kr >,

Han Berg Cho < hbcho@etri.re.kr >

Abstract: This paper provides the use cases family on FRMCS System principles related use cases.

12	FRMCS System principles related use cases

12.x	Use case: Moving Authority transmission

12.x.1	Description

This use case describes Moving Authority message transmission for train safe operation.

12.x.2	Pre-conditions

It is assumed that there is a connection for information transmission among a train-borne UE, a radio block centre and a train centre

The trackside has multiple radio block centre

The radio block centre is connected to 3GPP network and train control centre

The train driver uses the train-borne UE

The train driver has message display equipment

12.x.3	Service flows

The train-borne UE reports the situation and request to permit train moving to radio block centre

The radio block centre sends Moving Authority message including the permitted distance to move

And reports to train control centre

The train-borne UE receives Moving Authority message and starts moving

When the train crosses over new radio block centre, The train-borne UE requests new Moving Authority

The new radio block centre send new Moving Authority message

And reports train control centre

12.x.4	Post-conditions

The train driver receives Moving Authority message for safe operation

The train driver receives new Moving Authority message when the radio block centre changes

The train control centre monitors train moving status

12.x.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-12.x-001] | The FRMCS system shall transmit Moving Authority message from the radio block centre to trainborne UE |  |  | [The requirement is not covered by 3GPP network architecture. It is related to MC Data system] |
| [R-12.x-002] | The FRMCS system shall transmit the new Moving Authority message from the radio block centre to trainborne UE when the radio block centre changes |  |  | [Gap analysis: The requirement is not covered by 3GPP network architecture. It is related to MC Data system] |
| [R-12.x-003] | The FRMCS system shall transmit the new Moving Authority message from the radio block centre to trainborne UE with [99.999%] message reception rate |  |  | [Gap analysis: The requirement is related to MC Data service and needs study] |

Reference

[1] 	TTAI.KO-06.438: “LTE based Railway Communication System Architecture (Conventional and High Speed Railway”, December, 2016.