3GPP TSG-SA WG1 Meeting #77	S1-171165

Jeju, Korea, 13-17 February 2017	(revision of S1-171165)

Title:	Authentication for Private Slices

Agenda Item:	7.1

Source:				Qualcomm Incorporated

Contact:			Jack Nasielski   jackn@qti.qualcomm.com

Abstract: This document provides text for the SMARTER TS Security clause. The requirements are drawn from the SMARTER building block TRs.

## --- 1st change ---

## 6.1	Network slicing

### 6.1.1	Description

Network slicing allows the operator to provide customised networks. For example, there can be different requirements on functionality (e.g., priority, charging, policy control, security, and mobility), differences in performance requirements (e.g., latency, mobility, availability, reliability and data rates), or they can serve only specific users (e.g., MPS users, Public Safety users, corporate customers, roamers, or hosting an MVNO).

A network slice can provide the functionality of a complete network, including radio access network functions and core network functions (e.g., potentially from different vendors). One network can support one or several network slices.

### 6.1.2	Requirements

The 5G system shall allow the operator to create, modify, and delete a network slice.

The 5G system shall allow the operator to define and update the set of services and capabilities supported in a network slice.

The 5G system shall allow the operator to configure the information which associates a device to a network slice.

The 5G system shall allow the operator to configure the information which associates a service to a network slice.

The 5G system shall allow the operator to assign a device to a network slice, to move a device from one network slice to another, and to remove a device from a network slice based on subscription, device capabilities, operator's policies and services provided by the network slice.

The 5G system shall support a mechanism for the VPLMN to assign a device to a network slice with the needed services and authorised by the HPLMN, or to a default network slice.

The 5G system shall enable a device to be simultaneously assigned to and access services from more than one network slice of one operator.

Traffic and services in one network slice shall have no impact on traffic and services in other network slices in the same network.

Creation, modification, and deletion of a network slice shall have no or minimal impact on traffic and services not in the same network slice but in the same network.

The 5G system shall support the adaptation of capacity, i.e. elasticity of capacity of a network slice.

The 5G system shall ensure that elasticity of capacity of one network slice has no impact on traffic and services provided by other network slices in the same network.

The 5G system shall support means by which the operator can add and remove network functions to the network such that they can be used in a network slice.

The 5G system shall support means by which the operator can differentiate functionality and performance provided in different slices.

The 5G system shall support providing connectivity to home and roaming users in the same network slice.

Inbound and outbound roaming is not required for devices that use a private network slice.

## --- End 1st change ---

## --- 2nd change ---

## 8.3	Authentication

The 5G network shall support a resource efficient mechanism for authenticating groups of devices.

The 5G system shall support an efficient means to authenticate a user to a device (e.g., biometrics).

The 5G system shall be able to support authentication over a non-3GPP access technology using 3GPP credentials.

The 5G system shall support operator controlled alternative authentication methods (i.e., alternative to AKA) with different types of credentials for network access for IoT devices to allow new isolated deployment scenarios (e.g., for industrial automation).

Based on operator policy, the 5G system shall support access to isolated private network slices dedicated for use by trusted 3rd parties using identities, credentials, and authentication methods provided and managed by the 3rd party and supported by 3GPP.

## --- End 2nd change ---