3GPP TSG-SA WG1 Meeting #78	S1-172305

# Porto, Purtugal, 8-12 May 2017														(revision of S1-172042)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.186 | CR | 0009 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NOTE 2 UL/DL values in 5.3 Advanced Driving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2X |  |  |  |  |  |  | Date: |  | 2017-04-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 5.3, Advanced Driving are presented: the DL and UL data rate for intersection safety information exchanged between a RSU and a UE are UL:50Mbps” and DL: 0.5Mbps. These values are considered for 200 UEs. The associated NOTE2 values is for 1 UE and proposes “ UL: 2.5Kbps and DL: 250Kbps. The values in the Note2 are the incorrect, as they should be: - For UL (unicast): 50Mbps/200UEs = 250kbps per UE - For DL: :0.5Mbps per UE as: - If multicast is used: each UE would receive the total of the 0.5 Mbps. - Unicast is not considered: if unicast would be used, DL would be 0.5 Mbps/200 UEs= 2.5kbps per UE, i.e. each message received per second by a UE would be around 310Byte (=2.5kbps/8bit), this is much smaller than the 22.5KB (=450Byte * 50 message/second) expected in this scenario, so unicast looks impossible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NOTE2 is modified with DL value as 0.5Mbps and UL value as 250kbps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The wrong values in NOTE2 will not match the related values in the table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of 1st Change  ***************************************

5.3	Requirements to support Advanced Driving

Table 5.3-1 Performance requirements for advanced driving

| Communication scenario description |  | Req # | Payload (Bytes) | Tx rate (Message/Sec) | Max end-to-end latency (ms) | Reliability (%) (NOTE3) | Data rate (Mbps) | Min required Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Degree |  |  |  |  |  |  |  |
| Cooperative collision avoidance between UEs supporting V2X applications. |  | [R.5.3-001] | 2000 |  | 10 | 99.99 | 10 |  |
| Information sharing for automated driving between UEs supporting V2X application. | Lower degree of automation | [R.5.3-002] | 6500 (NOTE 1) | 10 | 100 |  |  | [700] |
|  | Higher degree of automation | [R.5.3-003] |  |  | 100 |  | 53 (NOTE 1) | [360] |
| Information sharing for automated driving between the UE supporting V2X application and the RSU | Lower degree of automation | [R.5.3-004] | 6000 (NOTE 1) | 10 | 100 |  |  | [700] |
|  | Higher degree of automation | [R.5.3-005] |  |  | 100 |  | 50 (NOTE 1) | [360] |
| Emergency trajectory alignment between UEs supporting V2X application. |  | [R.5.3-006] | 2000 |  | 3 | 99.999 | 30 | 500 |
| Intersection safety information between a RSU and UEs supporting V2X application. |  | [R.5.3-007] | 450 | 50 |  |  | DL: 0.5 UL: 50 (NOTE 2) |  |
| Cooperative lane change between UEs supporting V2X applications. | Lower degree of automation | [R.5.3-008] | 300-400 |  | 25 | 90 |  |  |
|  | Higher degree of automation | [R.5.3-009] | 12000 |  | 10 | 99.99 |  |  |
| Video sharing between a UE supporting V2X application and a V2X application server. |  | [R.5.3-010] |  |  |  |  | UL: 10 |  |
| NOTE 1: This includes both cooperative manoeuvers and cooperative perception data that could be exchanged using two separate messages within the same period of time (e.g., required latency 100ms). NOTE 2: This value is referring to a maximum number of 200 UEs. The data rate per UE is UL: 250Kbps for unicast and DL0.5Mbps for broadcast. NOTE 3: Sufficient reliability should be provided even for cells having no values in this table |  |  |  |  |  |  |  |  |

Editor’s note:	The KPI values of minimum required communication range in requirement [R.5.3-002] [R.5.3-003] [R.5.3-004] [R.5.3-005] need update based on further study.

***************************** End of 1st Change  ***************************************