3GPP TSG-SA WG1 Meeting #84 	S1-183174

Spokane, USA, 14 - 18 November 2018	(revision of S1-18xxxx)

Title:	Intercom system for large live events

Agenda Item:	8.3

Source:	Sennheiser, Fraunhofer HHI, Fraunhofer IIS, BBC, EBU

Contact: 		Konstantin.Septinus(at)sennheiser.com;Rafael.Paiva(at)sennheiser.com

Abstract: This contribution proposes a use case for multiple audio-visual content and service production to be supported by 3GPP systems in TR22.827. The contribution introduces a intercom system operated via a 3GPP network. The contribution introduces the concept of talkgroups and individual audio channels while also maintaining a group call at the same time. The intercom system can support a huge amount of participants over a long period of operation time without any interruption.

---------- Use Case template ----------

## x.1	Intercom system for large live events

### x.1.1	Short Description

For example, at a music festival an intercom system is used for two way voice communications between individuals in groups such as: security, camera operators, stage personal, etc. for organizing, monitoring, instructions and inquiry.

Handsets (UE) are organized in logical groups (talkgroups). Every crew-member at the music festival (or any other event where intercom is used) has a handset which is always listening on appropriate talkgroups. The members of a talkgroup are persons that are located in different positions of an event area.

Each handset can be part of many talkgroups. The intercom system can accomodate up to 1000 active handsets. Talkgroup management mechanism is required to support the members joining or leaving a group.

In any intercom system, there is always a central control device, the master unit. The master unit is used to organize all connected handsets, assign them to talkgroups, reconfigure handsets in real time and to communicate verbal instructions during production and/or setup. To talk directly to any group and/or any individual handset the controller uses the master unit. Selecting a talkgroup and talking into a microphone enables every member of the talkgroup to hear the given instructions or questions. While an intercom system uses typically a one-to-many transmission method, every handset can be instructed to send audio back to the master unit. The talkgroup can optionally hear this talkback message. Communication can be switched from one-to-many to a one-to-one communication.

Audio quality must be good enough to distinguish human voice with loud background noise. The intercom system is limited in time and space, meaning during the event the service is needed at the venue, but afterwards it can be disabled again and does not need to be available on site when there is no ongoing production.

Today’s intercom systems use dedicated networks. This use case describes how to achieve the benefits of intercom systems using a 3GPP network so that customer service, contribution, and public security networks might usefully be merged into one technology, potentially 5G. The challenge is to provide reliable connectivity between many devices that listen and speak to talkgroups, even in very crowded locations.

### x.1.2	Actors:

Controller operates the master unit: Alan, who has many colleagues

Security staff member: Brian

Camera operator: Charles

Stage personal: David, Ed

### x.1.3	Pre-conditions

1. A 5G non-public network (NPN) covering all areas of the event location offers an intercom system service to its subscribers

2. The 5G NPN operates logically independently from other co-located radio networks at the event location (e.g. PLMN)

3. The master unit works as an application server and is connected to the edge of the 5G NPN.

5. Handsets (5G UEs) automatically discover the intercom system service offered by the 5G NPN and join the talkgroups they wish to be members of.

### x.1.4	Service Flows

Intercom NPN

The intercom system operates in a number of modes (demonstrated in the following use cases)

Case 1: Broadcast mode - Master unit controller (Alan) sends message to everyone

- Alan selects broadcast mode

- Alan presses the talk button on the microphone at the master unit

- Alan talks into the microphone

- Audio message is received simultaneously by Brian, Charles and David (and up to 1000 other users)

Case 2: Individual mode - Direct message from master unit controller (Alan) to Charles

- Alan selects individual mode and selects Charles as a recipient

- Alan presses the talk button on the microphone at the master unit

- Alan talks into the microphone

- Audio message is received by Charles only

- Charles can optionally reply using push-to-talk (PTT)

- Optional PTT message from Charles only received by Alan (not other master unit controllers)

Case 3: Uplink

- Brian sends an audio message to Alan (or other master unit controllers) with PTT or voice activity detectionation

- Alan selects Brian as a recipient

- Alan press the button on the microphone at the master unit

- Message from Alan received by Brian

- Repeat 1 to 4 for a conversation

Case 4: Downlink message to talkgroups

- Alan selects a talkgroup (a group of users e.g. stage personnel)

- Alan press the button on the microphone at the master unit

- Audio message is received simultaneously by David and his stage personnel colleagues

- Ed (a member of stage personnel) did not understand the message and request a repeat from Alan using PTT on his handset

- Audio message from Ed goes to all members of the stage personnel talkgroup

Case 5: Uplink message to talkgroups

- Ed sends audio message to a talkgroup (e.g. security talkgroup) with PTT or voice activity detection

- Audio message is received simultaneously by David and his stage personnel colleagues including Ed

- David and Ed may be in close proximity, but could not talk to each other directly due to loud noise. Once they can see each other’s lips, the ongoing Intercom communication must by lip-synchronous

- Ed can also listen to his own voice through the intercom channel, the communication delay should be small enough not to disturb him while talking

### x.1.5	Post-conditions

- The intercom service offered by the 5G NPN is deactivated after the event (e.g. music festival) is over

### x.1.6	Potential Impacts or Interactions with Existing Services/Features

To be defined.

Editor Note’s: to be completed in cooperation with 3GPP

### x.1.7	[Potential] Requirements

[PR x.1.6-001] The 3GPP system supports the establishment of a low latency group communication service between the intercom master unit (application server) and the handsets (UEs) within a service area of up to 100.000 m2.

[PR x.1.6-002] The 3GPP low latency group communication service shall be able to accomodate up to 30 simultaneous talkgroups, while each talkgroup shall be able to accommodate up to 1000 handsets moving at walking speed.

[PR x.1.6-003] The 3GPP low latency group communication service may be able to handle up to 60 simultaneous active uplink streams, which result from the uplink channels from all handsets.

[PR x.1.6-004] The 3GPP low latency group communication service makes use of multicast delivery in a designated geographical area (service area of the event up to 100.000m2)

[PR x.1.6-005] The 3GPP low latency group communication service shall support each UE within a group to send/receive an audio quality stream composed of packets with periodicity of 2.5 ms, up to 768 kbps (16 bits, 48 kHz audio).

[PR x.1.6-007] Audio quality streams shall be transmitted with a packet error rate lower than 1%

[PR x.1.6-008] The 99th percentile of the end-to-end delay of packets should be smaller than 2.5 ms

[PR x.1.6-009] The 3GPP low latency group communication service requires a communication group management mechanism to support members (UEs) joining/leaving a group for communication.