3GPP TSG-SA WG1 Meeting #84	S1-183380

Spokane, USA, 12 – 16 November 2018	Revision of S1-183102

Title:	Overview of Multihop Communications in 3GPP 5GS

Agenda Item:	8.11

Source:	OMESH Networks

Contact:	Liang Song (songl@omeshnet.cn)

Document for:  Approval

Abstract: This document provides an overview of Multi-hop communications for the SID on FS_REFEC.

3GPP has been working on the 5G system with all communication KPIs much higher than 4G LTE, including 10-20Gbps peak data rate, 1Gbps user experience rate, 3-5 times higher in spectrum efficiency, up to 500km/h mobility, 1ms air interface latency, 1 million connections per square km in network density, 100 times in energy efficiency, and 10Tbps per square km in flow density. However, it was not expected that a single system will satisfy all the requirements co-currently. Three primary application scenarios including eMBB, mMTC, and URLLC were identified, e.g., in SMARTER, which presents a triangle of application scenarios emphasizing on different aspects of the requirements. SA1 has also been working on new verticals that requires merged features. For example, eV2X and CAV studied verticals that require combined features of eMBB and URLLC. It is understood that the future smart spaces based on Internet of Everything (IoE) require all types of UEs being interconnected, and more combined features of all communication KPIs will be necessary.

Requirements of these new verticals drive the introduction of multi-hop communications in 3GPP 5G.  The motivations can be analyzed from three angles.

First, multi-hop communications improve network capacity especially when massive UE-to-UE and UE-to-Edge communications are largely required. In network information theory, it had been identified over a decade that single-hop cellular network has a capacity of O(B), where B is the spectrum bandwidth. The same capacity can be increased to O(B∙N), with a multihop network of evenly distributed traffics in terms of source-destination pairs, where N is the number of nodes including UEs. The network capacity can be further increased to O(B∙N) in a multihop network with clustered traffics or with mobility. Multi-hop naturally decreases the interference range of radio communications, so that radio resources such as spectrum can be spatially re-used. Take 10 hops as an example, average number competing neighbors can be reduced by over 100 times; end-to-end throughput can therefore be improved by up to 100 times; end-to-end latency can be reduced by 10 times by taking 10 hops; average link budget can be reduced by 30dB; end-to-end energy consumption can therefore be reduced by 100 times. And actually the theoretical performance metrics can improve with the number of wireless hops.

Figure X.1 Multihop improved Capacity

Second, multi-hop communications improve the coverage and energy efficiency for UEs. Take the figure below as an illustration. Concrete walls in typical urban areas can introduce link attenuation of over 20-30 dB, which can be much larger than antenna gain that outdoor base stations can provide. It will be either impossible or energy draining to have the UEs D and C to communicate to the gNB directly. Since multihop improves the UE QoS including both rate and latency in such scenarios, it also reduces infrastructure costs by providing less UE energy consumption and sparser deployment in operators’ network infrastructure.

Figure X.2 Multihop improved Coverage

Third, multi-hop communications can potentially enable a few vertical applications in the interests of 3GPP, which are outlined in the table below.

| Verticals | Specialities | Feature Description |
| --- | --- | --- |
| Remote or emergency Networks | Lack of network and power cable infrastructure to provide wireless coverage | In some special industrial or public-safety types of environment, such as smart meters, logistics, underground mining, remote unmanned areas, underground pipes, exhibition places, emergency sites including for example under forest fires, earthquake and hurricane etc., there is a significant lack of reliable cable infrastructure in both network and power. Multi-hop wireless can be a potential way to improve coverage and energy efficiency, so that 5G services can be provided reliably over a required period of servicing time. |
| Collaborative Robots | Dynamic and potentially very high-density network, with strict latency and reliability requirements | In emerging areas such as smart factory, smart farming, enhanced V2X, robots are deployed to collaboratively complete complex tasks with no or limited human operations. The densely populated robots would need to exchange and process information in a highly dynamic networking environments, e.g., with volatile traffics, dynamic UEs with high mobility, and very strict requirements on reliability and latency. Multi-hop wireless can potentially provide reliable communications for automatic robots in close or large proximity, under dynamic networking environments. |
| Smart Spaces (e.g., inHome, smart city and community, etc.) | Merging KPIs of eMBB, mMTC, and URLLC due to IoE interconnections, with potentially very high density and high capacity requirements | Future IoE based smart cities and communities can present potential killer applications of 5GS to municipalities and consumers. Traditionally, IoT devices are often considered small and of very limited computing and communication capabilities. In order to provide enhanced user experience over current eMBB (mobile Internet) and mMTC (IoT) solutions, it is often identified that next generation IoT UEs will have decent processing power, and need to interconnect with eMBB or URLLC/V2X UEs in close or large proximity. These new requirements would potentially merge KPI features of eMBB, mMTC, and URLLC into the future IoE based smart spaces. Multi-hop wireless can potentially provide much higher capacity to such new requirements, especially when massive UE-to-UE communications are considered. |

Table X.1 Potential Verticals by Multihop

On RAN perspectives on R16, network based multihop relay is being studied in IAB, and UE based multihop relay may potentially be introduced in NR-sidelink. However, key system KPIs of multihop use cases and traffic scenarios hasn’t been properly documented to provide guidance to the RAN studies. With all the potential multihop use cases and traffic scenarios, scalability shall be provided where all communication performance metrics such as data rate, latency, and energy consumption shall not be degrading with the number of wireless hops due to dynamic wireless networking issues such as link blockage and congestions.

The detailed KPIs of multihop relay on 5GS shall be provided in TR22.886.

Proposal: Principals and contexts of the above text shall be provided in the overview section of TR22.886.