3GPP TSG SA WG1 Meeting #84	S1- 183403

Spokane, WA, USA, 12 - 16 November 2018	(revision of S1-3361, S1-183134)

Title:	cyberCAV – 22.104 –network performance: Communication service availability Agenda Item:

Source:	Huawei, Siemens,B-com

Contact:	Amanda Xiang, amanda.xiang@huawei.com,

Daniel Medina, daniel.medina@huawei.com,

Bahr, Michael bahr@siemens.com

Mathieu LAGRANGE Mathieu.LAGRANGE@b-com.com

Abstract: This contribution provides network performance requirement text as well as detail explanation annex regarding the relationship between communication service availability and network performance KPI, such as reliability.

# Rationale

TR 22.804 defines communication service availability and its performance requirements, which are defined and measured at the logical interface between application and communication system. Communication service availability requirements need to be translated into 3GPP network reliability performance requirements such as packet error ratio (PER).

# Proposal

# Instruction to the editor, the proposed text will be added to the end of clause 5.1 overview.

------------------------- START OF PROPOSED CHANGES ----------------------------

### ------------------------- Start of Change 1 ----------------------------

-

------------------------- End of Change 1 ----------------------------

------------------------- Start of Change 2----------------------------

TT

PER=Number of PDUs not successfully delivered by target PDU layer to egress interface during time period TNumber of PDUs received by source PDU layer via ingress interface during time period T

p

p=EPER=limT→∞Number of PDUs not successfully delivered by target PDU layer to egress interface during time period TNumber of PDUs received by source PDU layer via ingress interface during time period T

### ------------------------- End of Change 2 ----------------------------

### ------------------------- Start of Change 3 ----------------------------

5.1 overview

Communication Service Availability is considered as important service performance requirement for cyber physical application, especially for the applications with deterministic traffic. Although the communication service availability can’t be directly measured by the network, communication service availability requirement is the combination of latency, survival time and reliability requirement for 5G system, because the system is considered unavailable to the cyber physical application when an expected message is not received (e.g. transfer time is bigger than the maximum transfer latency) by the application after application’s survival time expires.

An example of the relationship between network reliability, survive time configuration and communication service availability of logical link is illustrated in the following table 5.1-1.

Table 5.1-1example of relationship between reliability and communication service availability when survival time is one transfer cycle.

| Communication service availability | Reliability ( as defined in TS 22.261) 1-p |
| --- | --- |
| ### 99.999% | ### 99.9% |
| ### 99.999999% | ### 99.99% |
| ### 99.9999999% | ### 99,999% |
| ### 99,9999999999% | ### 99.9999% |
| ### 99,999999999999% | ### 99.99999% |

### ------------------------- End of Change 3 ----------------------------

### ------------------------- Start of Change  4 ----------------------------

STTTF1TTR1TTF2TTR2

m=TsurvivalTcycle+1TcyclexxTsurvival=2 Tcyclem=3

pAlink

Alink=1-pm=1-pTsurvivalTcycle+1

m

pUlink=1-Alink=10-6mTsurvivalm

pUlink=10-6Alink=99.9999%

| Tsurvival | m | p |
| --- | --- | --- |
| 0 | 1 | 10-6 |
| Tcycle | 2 | 10-3 |
| 2 Tcycle | 3 | 10-2 |

m=2

m=2

| p | 1-p | Ulink=pm=pTsurvivalTcycle+1 m=2 | Alink=1-pm=1-pTsurvivalTcycle+1 m=2 |
| --- | --- | --- | --- |
| 10-3 | 99.9% | 10-6 | 99.9999% |
| 10-4 | 99.99% | 10-8 | 99.999999% |
| 10-5 | 99.999% | 10-10 | 99.99999999% |
| 10-6 | 99.9999% | 10-12 | 99.9999999999% |
| 10-7 | 99.99999% | 10-14 | 99.999999999999% |

MTBFlink

Alink

Alink=MTBFlinkMTBFlink+MTTRlink

MTBFlinkMTTRlink

m=TsurvivalTcycle+1Tcycle

NMTBFlinkψn=P(N>n)n

MTBFlink=n≥0ψn

pψn

ψn=(1-p) ψn-1for m=1ψn=1-p(ψn-1+p ψn-2)for m=2ψn=1-p(ψn-1+p ψn-2+p2 ψn-3)for m=3⋮⋮

m

ψn=(1-p)k=1mpk-1 ψn-k

n≥m

n≥mψn&=(1-p)k=1mpk-1 n≥mψn-kMTBFlink-n=0m-1ψn&=(1-p)k=1mpk-1 MTBFlink-n=0m-k-1ψn

n=0m-1ψn=m

MTBFlink-m&=(1-p)k=1mpk-1 MTBFlink-(m-k)&=k=1mpk-1 MTBFlink-m+k-k=1mpk MTBFlink-m+k&=k=0m-1pk MTBFlink-m+k+1-k=1mpk MTBFlink-m+k&=MTBFlink-m+1+k=1m-1pk-pmMTBFlink

MTBFlink&=1pmk=0m-1pk=1-pmpm(1-p)≈1pm

RMTTRlinkφr=P(R>r)r

MTTRlink&=r≥0φr&=r≥0pr&=11-p

φr=prr≥0

MTBFlinkMTTRlink

Alink=MTBFlinkMTBFlink+MTTRlink=1-pmpm(1-p)1-pmpm(1-p)+11-p=1-pm

pAlink

p≤m1-Alink

p≤p

-------------------- END OF PROPOSED CHANGES ----------------------------