3GPP TSG-SA WG1 Meeting #84	S1-183511

# Spokane, Washington, USA, 12 - 16 November 2018(revision of S1-183024)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.179 | CR | 0067 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Group Call Commencement Modes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY2 |  |  |  |  |  |  | Date: |  | 2018-11-13 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some Railway group call setup scenarios require the presence of specific users. For example, the setup cannot proceed unless at least one group member has activated a specific Functional Alias. In order to enable the implementation of these scenarios in MCPTT, additional commencement mode requirements are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New commencement mode requirements are added to section 6.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to implement some Railway features in Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

### 6.2.1	Commencement modes for MCPTT Group calls

[R-6.2.1-001] The MCPTT Service shall be capable of allowing an MCPTT Group call setup request to proceed without prior acknowledgement by any MCPTT User of that MCPTT Group.

[R-6.2.1-001a] The MCPTT Service shall be capable of allowing an MCPTT Group Call setup request to proceed only if a minimum number of MCPTT Group Members are currently affiliated.

[R-6.2.1-001b] The MCPTT Service shall be capable of allowing an MCPTT Group Call setup request to proceed only if specific MCPTT Group Member(s) are currently affiliated.

[R-6.2.1-002] An MCPTT User currently affiliated to an MCPTT Group shall acknowledge receipt of an MCPTT Group call setup request, if requested to do so by the MCPTT Service.

[R-6.2.1-003] The MCPTT User's acknowledgement may require direct interaction of the MCPTT UE with the human user, or may be automatically executed by the MCPTT UE, in accordance with policy established by an MCPTT Administrator.

[R-6.2.1-004] The MCPTT Service shall be capable of requiring that a minimum number of Affiliated MCPTT Group Members acknowledges receipt of the MCPTT Group call setup request before the audio transmission proceeds.

[R-6.2.1-005] The MCPTT Service shall be capable of requiring that specific MCPTT Users acknowledge receipt of the MCPTT Group call setup request before the audio transmission proceeds, regardless of the affiliation state of those users.

NOTE 1:	In this case the MCPTT Service affiliates the specific MCPTT Users who are not currently affiliated to the target MCPTT Group and then returns them to their previous affiliation state when the transmission ends.

[R-6.2.1-006] The MCPTT Service shall be capable of requiring that all MCPTT Users that are both affiliated to the MCPTT Group and in a given geographical area acknowledge receipt of an MCPTT Group call setup request before the audio transmission proceeds.

[R-6.2.1-007] The MCPTT Service shall provide a mechanism for an MCPTT Administrator to determine the subset of Affiliated MCPTT Group Members that shall acknowledge receipt of the MCPTT Group call setup request before the audio transmission proceeds.

NOTE 2:	In the following requirements, the term, "MCPTT Group Call setup request requires acknowledgement" is used when one or more of the acknowledgement conditions defined above (i.e., [R-6.2.1-004], [R-6.2.1-005], [R-6.2.1-006], and/or [R-6.2.1-007]) applies.

[R-6.2.1-008] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, and the required MCPTT Group Members do not acknowledge the call setup within a configured time (the "acknowledged call setup timeout"), the MCPTT Service may proceed with the call and then may notify the initiating MCPTT User that the acknowledgements did not include all required members.

[R-6.2.1-009] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, and the required MCPTT Group Members do not acknowledge the call setup within a configured time, the MCPTT Service may abandon the call and then may notify the initiating MCPTT User that the acknowledgements did not include all required members.

[R-6.2.1-010] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, the initiating MCPTT User shall at any time have the option of allowing the call to proceed regardless of the state of the acknowledgements (i.e., to "convert" the call to an unacknowledged call).

[R-6.2.1-010a] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, and the required MCPTT Group Members did not acknowledge the call setup within a configured time, the MCPPT Service shall be able to provide the list of MCPTT Group Members who did not acknowledge the call to the initiating MCPTT User.

[R-6.2.1-011] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, the acknowledged call setup timeout shall be established by an MCPTT Administrator.

[R-6.2.1-012] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, the behaviour in response to the expiration of the acknowledged call setup timeout shall be established by an MCPTT Administrator.

[R-6.2.1-013] If an MCPTT Group Call setup request requires acknowledgement from Affiliated MCPTT Group Members, the MCPTT Service shall support an indefinite (i.e., infinite) call setup timeout.

[R-6.2.1-014] If the MCPTT Service has knowledge that some affiliated members of a group can not be Participants in an unacknowledged MCPTT Group Call, the MCPTT Service shall provide an indication to the requester that the call is proceeding without all affiliated members, and shall provide the list of the missing members based on policy established by the MCPTT Administrator.

[R-6.2.1-015] If MCPTT User(s) are excluded from an MCPTT call as there is insufficient capacity to support their participation the MCPTT Service shall ensure that the MCPTT User(s) receive a notification that they have been excluded from the call for reasons of lack of capacity.

* * * End of changes * * *